3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404574

# Changsha, China, April 15 – April 19, 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for CSI compression

Agenda Item:	9.11.4

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability and interoperability issues for CSI compression.

- Discussion

      - 2.1 Time domain compression

In last meeting, RAN4 suggest companies to provide detail parameters for reference decoder. However, we found that AI based CSI model is still updating. Release 18 study item studied the sub-use case of spatial-frequency domain CSI compression using two-sided model. In Rel-19, The work item description [2] has identified a study objective associated with performance improvement:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

In RAN1 #116 meeting, different categories are agreed to be studied for different scenario. the AI model scheme for two-side CSI compression will extend from spatial/frequency compression to spatial/temporal/frequency compression and CSI compression plus prediction.

Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study: Case; Target CSI slot(s); Whether the UE uses past CSI information; Whether the network uses past CSI information. 0; Present slot; No; No. 1; Present slot; Yes; No. 2; Present slot; Yes; Yes. 3; Future slot(s); Yes; No. 4; Future slot(s); Yes; Yes. 5; Present slot; No; Yes.

For example,

- For case 0, spatial/frequency compression is assumed

- For case 1,2,5, spatial/temporal/frequency compression is assumed

- For case 3 and 4, spatial/temporal/frequency compression plus prediction is assumed

When time domain compression or prediction is considered, the AI model may be different from case 0, e.g. LSTM or other model may be further involved. Therefore, RAN4 needs to first discuss the scenario or scheme assumption for reference decoder first, e.g. whether time domain compression or prediction is considered or not. If RAN4 spend lots of time to align these model parameters based on Rel-18, it’s a question whether RAN4 needs to align these parameters based on Rel-19 later.

Proposal 1: RAN4 to discuss the assumption for reference decoder first, e.g. whether time domain compression or prediction is considered or not, which will have impact on the reference model type and structure.

      - 2.2 How to choose reference model

For option 3, the target is to fully standardize the inference encoder (including architecture and parameters).

Since there are many models and parameters from different UE and NW vendors, it’s challenge to align all the parameters. RAN4 needs to discuss how to define a rule to choose a model structure and decide the detail model parameter. There are many aspects needs to be considered:

- Performance

- Complexity

- Interoperability and compatibility

- other

here, we would like to emphasize the importance of interoperability and compatibility. The CSI reference decoder needs to be compatible with different CSI encoders from UE vendor to guarantee the performance. Otherwise, the reference decoder will only work for dedicated encoder structure, which will have great limitation on UE implementation.

Proposal 2: RAN4 needs to discuss how to define a rule to choose a reference decoder model if model architecture and parameters from companies are diverse.

      - 2.3 AI model structure

In the next, we will provide our AI structure and parameters for different assumptions:

- Spatial/frequency compression

- Spatial/time/frequency compression

Spatial/frequency compression

For spatial/frequency compression, a Transformer-based AI model is used.

Fig.1 The basic structure of Transformer model

Table 1: basic parameter for decoder

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Model architecture parametersa | Model type | Transformer |
|  | Model depth | 6 |
|  | Quantization method for the encoder output | Scalar |
|  | Encoder-decoder interface | 60, 120, 280bits, 240 bits for generalization |
|  | Fixed point representation | int16 |
|  | Format of input to encoder/output of decoder | 60, 120, 280bits |
| Model Training related parameters | Training procedure |  |
|  | Loss function | SGCS |
|  | Training datasets | Channel model: - Uma and/or Umi number of Tx/Rx ports - gNB: 32 ports,(8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - UE:2RX,(1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Dataset construction : 210,000 200 drop*21 cell*50 UE*1 samples per UE |
|  | Hyperparameters | Learning rate = 0.001, batch size = 128 optimization algorithm = Adam, |
|  | Cross-validation details | Dataset for training: 199,500 Dataset for Testing: 10000 Dataset for validation: 10500 |
| Generalization (may be applicable to all four options) | Performance requirements on test dataset(s) |  |
| Scalability (may be applicable to all four options) | Supported antenna port configurations |  |
|  | Supported feedback payloads | maximum 240bits, different feedback payloads are obtained by cutting off the tail of the maximum 240 bit |

The generalization evaluation of different size of CSI feedback payloads is conducted by parallel training of one encoder part and three decoder parts. The basic AI model for feedback payloads generalization is as Fig.1 shows. Setting the maximum feedback bit number as 240 bit, different feedback payloads are obtained by cutting off the tail of the maximum 240 bit. The output of encoder is always equal to the maximum bit number, while the input of the decoder is the truncated bit of the output of encoder.

Fig.2, The basic structure of AI model for feedback payloads generalization

The FLOPs and number of AI/ML parameters for the CSI generation part and the CSI reconstruction part are provided separately in Table 2. As a rank-common and layer-common AI model is used, the FLOPs is captured as 258*K M, wherein K is the maximum rank number and 258M is the FLPOs for one layer calculation.

Table 2: Complexity and memory storage of AI model

|  | CSI generation part | CSI reconstruction part |
| --- | --- | --- |
| FLOPs | 258*K*106 | 258*K*106 |
| Number of AI/ML model parameters | 10.79*106 | 10.79*106 |

Spatial/time/frequency compression

As Figure 3 shown, the AI model is LSTM and Transformer based. Different with AI model for spatial/frequency compression, the input of AI model for spatial/temporal/frequency compression includes both the CSI/eigenvector of current slot and accumulated information from historic slot. And the output of encoder is the compressed CSI/eigenvector after quantization, while an intermediate output of LSTM part of encoder is the accumulated information, which would be used as input for the next slot. Similar as the encoder part, the decoder needs to recovery the CSI/eigenvector based on both compressed CSI/eigenvector and the historic accumulated information calculated in the decoder side.

Fig.3: Structure of AI model for spatial/temporal/frequency compression

The FLOPs and the number of parameters are used for complexity and storage comparison between spatial/frequency compression and spatial/temporal/frequency compression. Table 3 shows the complexity and parameter numbers of different AI models. It is observed that the complexity of SFT compression is higher than SF compression, while the number of parameters of SFT compression is less than SF compression. Table 4 shows the parameters for AI model training.

Table 3: Complexity and memory storage of AI model

|  |  | CSI generation part | CSI reconstruction part |
| --- | --- | --- | --- |
| SF | FLOPs | 279 M | 279 M |
|  | Number of AI/ML model parameters | 10.79 M | 10.79 M |
| SFT | FLOPs | 754 M | 754 M |
|  | Number of AI/ML model parameters | 6.89 M | 6.89 M |

Table 4：Parameter for AI model training

| AI training parameter | Value |
| --- | --- |
| Quantization | Scalar quantization |
| Loss function | SGCS |
| Learning rate | 0.0001 |
| Optimizer | Adam |
| Epoch | 200 |
| Batch size | 128 |

Since RAN1 is still improving CSI compression performance, the decoder structure may be further updated. RAN4 needs to align the assumption for reference decoder.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: RAN4 to discuss the assumption for reference decoder first, e.g. whether time domain compression or prediction is considered or not, which will have impact on the reference model type and structure.

Proposal 2: RAN4 needs to discuss how to define a rule to choose a reference decoder model if model architecture and parameters from companies are diverse.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface