3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404579

# Changsha, China, April 15 – April 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on scheduling restriction for TCI state switch in mDCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-3-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For mDCI, two MAC CE command may arrive at different time or the processing procedure may be different, it’s possible that when TCI state switch for TRP1 has been finished, measurement for T/F, pathloss and L1-RSRP of TRP2 is undergoing. Therefore, scheduling restriction needs to be considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 8.22.6 for scheduling availability for DL TCI state switch for mDCI. Add 8.24.6 for scheduling availability for UL TCI state switch for mDCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MAC CE based dual TCI state switch requirement in mDCI will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.22.6, 8.24.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

8.22	Active downlink TCI state switching delay for unified TCI for multi-DCI mTRP

Unmodified sections omitted

8.22.6	Scheduling availability of UE during DL TCI state switch

In FR1,

If UE does not support [two TAs or RTD>CP capability],

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to T/F measurement or pathloss measurement based on SSB or CSI-RS.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the SSB or CSI-RS symbols used for T/F measurement or pathloss measurement.

If UE supports [two TAs and RTD>CP capability],

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to T/F measurement or pathloss measurement based on SSB or CSI-RS.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols and 1 symbol before and after symbols corresponding to SSB or CSI-RS symbols used for T/F measurement or pathloss measurement.

In FR2,

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the SSB or CSI-RS symbols used for T/F measurement or pathloss measurement.

<End of Change #1>

<Start of Change #2>

8.24	Active uplink TCI state switching delay for unified TCI for multi-DCI mTRP

Unmodified sections omitted

8.24.6	Scheduling availability of UE during UL TCI state switch

In FR1,

If UE does not support [two TAs or RTD>CP capability],

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to T/F measurement or pathloss measurement based on SSB or CSI-RS.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the SSB or CSI-RS symbols used for T/F measurement or pathloss measurement.

If UE supports [two TAs and RTD>CP capability],

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to T/F measurement or pathloss measurement based on SSB or CSI-RS.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols and 1 symbol before and after symbols corresponding to SSB or CSI-RS symbols used for T/F measurement or pathloss measurement.

In FR2,

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the SSB or CSI-RS symbols used for T/F measurement or pathloss measurement.

<End of Change #2>