3GPP TSG-RAN WG4 Meeting #110bis	R4-2404620

Changsha, China, 15th – 19th April, 2024

Agenda item:	9.11.1

Source: 		NTT DOCOMO, INC.

Title: 	Post-deployment validation discussions for NR AI/ML

Document for:	Discussion

- Introduction

Issue 1-2: Post deployment handling

Agreement:

- To ensure the AI performance after device deployment, discuss the following options further

  - Option 1: Conduct the conformance testing for AI model/functionality before deployment

    - FFS on the feasibility

  - Option 2: Design the test to verify the performance monitoring

    - Depend on the other WG progress

    - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable

  - Other options are not precluded

During the last meeting, post deployment handling was discussed [1], and the agreement currently has two options as follows:

This paper proposes how the post-deployment validation should be treated and conducted from RAN4 perspective based on previous agreement.

- Discussion

According to the previous agreement, currently there are two options for post deployment handling. Option 1 is conducting the conformance testing for AI model/functionality before deployment, Option 2 is designing the test to verify the performance monitoring. The necessity of post-deployment validation is to confirm the model/functionality validity during operation. The model change/drift appears due to the training results based on real network operation and/or condition difference from performance test. Therefore, if the conformance testing for AI model/functionality is used for post-deployment validation, it may need the large number of tests to satisfy the test coverage and it is not realistic. Thus Option 1 is not feasible.

Proposal 1: Conducting the conformance testing for AI model/functionality before deployment is not feasible because it may need the large number of tests to satisfy the test coverage and it is not realistic.

Considering the necessity of post-deployment validation, it is natural that performance monitoring includes it during the procedure. However, the performance monitoring and post-deployment validation has to be differentiated from RAN4 perspective. The purpose of performance monitoring should be confirmation whether the AI model/functionality currently in use can still be used or not. On the other hand, the purpose of post-deployment validation should be determination whether the AI model/functionality can be re-activated or not.

Proposal 2: The purpose of performance monitoring should be confirmation whether the AI model/functionality currently in use can still be used or not. On the other hand, the purpose of post-deployment validation should be determination whether the AI model/functionality can be re-activated or not.

Thus, post-deployment validation should be applied for AI model/functionality which is already deactivated through the performance monitoring. Currently RAN1 assumes that activation for monitoring purpose is needed to determine whether deactivated AI model/functionality due to the performance degradation can be re-activated or not. RAN4 also should consider the requirements of activation for monitoring purpose as post-deployment validation e.g., the number of trials, metric for re-activation which can be different from requirements before deployment.

Proposal 3: RAN4 should consider the requirements of activation for monitoring purpose as post-deployment validation e.g., the number of trials, metric for re-activation which can be different from requirements before deployment.

- Conclusion

In this contribution, we proposed our views on Post-deployment validation for NR AI/ML.

Proposal 1: Conducting the conformance testing for AI model/functionality before deployment is not feasible because it may need the large number of tests to satisfy the test coverage and it is not realistic.

Proposal 2: The purpose of performance monitoring should be confirmation whether the AI model/functionality currently in use can still be used or not. On the other hand, the purpose of post-deployment validation should be determination whether the AI model/functionality can be re-activated or not.

Proposal 3: RAN4 should consider the requirements of activation for monitoring purpose as post-deployment validation e.g., the number of trials, metric for re-activation which can be different from requirements before deployment.

References

[1] 3GPP, R4-2403712, “WF on AI/ML”, Qualcomm Incorporated