3GPP TSG-RAN WG4 Meeting # 110bis		R4-2404638

Changsha, China, 15th – 19th April, 2024

Agenda item:		6.16.7.3

Source:		Samsung

Title:	Discussion on RRM performance part for NTN mobility enhancements

Document for:		Discussion

- Introduction

In last RAN4#110 meeting, RAN4 completed the RRM core part of Rel-18 NTN enhancement. In previous meetings, the performance part of Rel-18 NTN enhancement hasn’t been discussed a lot. In this contribution, we discuss the performance requirements for NTN mobility enhancements according to the agreed core requirements.

- Discussion

Test cases list:

For the test cases design, it needs to define some test cases to verify the new core requirements.

For NTN bands below 10 GHz, according to the current core requirements for mobility enhancement, the core requirements are defined as:

- NTN-to-TN/TN-to-NTN cell reselection

- NTN-to-NTN time-based/location-based measurement initiation for cell reselection in earth-moving cell

- RACH-less HO

- NTN to NTN satellite switching without PCI change.

- NTN to NTN time and location-based trigger CHO enhancements.

According to the agreements of no test case for TN-to-NTN cell reselection, it is not introduced.

For earth-moving cell, for Rel-17, test cases are introduced for earth-fixed cell.

For time-based measurement initiation for cell reselection in earth-moving cell, the core requirements are reused as Rel-17 NTN FR1. No need to introduce test case.

For location-based measurement initiation for cell reselection in earth-moving cell, add one test case for it, use A.14.1.4 as baseline.

For RACH-less HO, define test case to verify new HO delay.

For NTN-NTN satellite switching with re-synchronization, it separated to hard switch and soft switch.

Proposal 1: For NTN mobility enhancement below 10GHz,

- Define one test case for NTN location-based measurement initiation for cell reselection for earth-moving cell

- Define test case for RACH-less HO

- Define test case for NTN to NTN satellite switching with re-synchronization

  - Hard switch

  - Soft switch

- Define test case for NTN to NTN time and location-based trigger CHO enhancements

- Conclusion

In this contribution, we provide our proposals for test cases of NTN mobility enhancements:

Proposal 1: For NTN mobility enhancement below 10GHz,

- Define one test case for NTN location-based measurement initiation for cell reselection for earth-moving cell

- Define test case for RACH-less HO

- Define test case for NTN to NTN satellite switching with re-synchronization

  - Hard switch

  - Soft switch

- Define test case for NTN to NTN time and location-based trigger CHO enhancements

- Reference

[1] R4-2403492, WF on RRM requirements for NR_NTN_enh, Qualcomm

[2] R4-2403572, Big CR to TS 38.133 on RRM requirements for NR NTN enhancement, Qualcomm