3GPP TSG-RAN WG4 Meeting #110bis	R4-2404644
Changsha, China, 15th – 19th April, 2024

Agenda item:	6.9.1.2

Source:	vivo

Title:	Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results-phase 3

Document for:	Approval

1	Introduction

Based on the agreed working procedure [1] and pass fail limit [2], phase 2 AC lab alignment was concluded in RAN4#110 meeting [3].

This contribution presents final conclusion of Rel-18 AC lab alignment. And updated RC harmonization measurements results and analysis is also shared.

2	Discussion

The reference value is derived based on linear average of all the labs results. The detailed analysis file is in the attached Excel worksheet of this contribution.

BHH: Beside Head and Hand, i.e., talk mode

BH: Beside Hand only, i.e., browsing mode

Figure 1 share the full-package of measured results of band n78 (LAD1 and LAD2) and band n28 (LAD3 and LAD4), from each test lab. Figure 2 analyse the deviation between each test lab and reference value.

Figure 1: Full package of NR FR1 TRP and TRS AC lab alignment measurement results

Figure 2: NR FR1 TRP and TRS AC lab alignment analysis, deviation between each test lab and reference value

Observation 1: For TRP, no outlier value is identified. The reference value is derived by the agreed linear average (with dBm) of all the available results, the maximum deviation between test labs and reference value is 0.76dB for n78 TRP and 1.22dB for n28 TRP.

Observation 2: For TRS, no outlier value is identified. The reference value is derived by the agreed linear average (with dBm) of all the available results, the maximum deviation between test labs and reference value is 1.24dB for n78 TRS and 1.55dB for n28 TRS.

Observation 3: The maximum deviation between test labs and reference value for n78 and n28 are within 0.75*MU, all the 8 labs with available results are well aligned at band n78 and n28.

Observation 4: Compared with phase 2 results submitted until RAN4#110 meeting, adding Lab 1 AC n28 results and updated Lab 3 AC n28 results based on retesting. The updated results are considered in this analysis.

With above analysis, the following proposal can be made as the final Rel-18 FR1 TRP TRS AC lab alignment outcome:

Proposal 1: 3GPP Rel-18 FR1 TRP TRS AC lab alignment activity for BHH can be successfully concluded within the 8 labs with anechoic chamber system are well aligned at FR1 (n78 to represent high band, and n28 to represent low band).

We also analyse the RC measured results (lab 9 add measurement results for band n78), with the following summary：

Figure 3: NR FR1 TRP and TRS RC lab alignment measurement results from each test lab (Lab9 n28 results is not available yet)

Figure 4: NR FR1 TRP and TRS RC lab alignment analysis, deviation between each test lab and RC reference value (Lab9 n28 results is not available yet)

Observation 6: For n78 and n28 TRP, the reference value is derived by the agreed linear average (with dBm) of all the 5 labs results, the maximum deviation between test labs and RC reference value is 1.47dB for BHH TRP and 1.18dB for BH TRP.

Observation 7: For n78 and n28 TRS, the reference value is derived by the agreed linear average (with dBm) of all the 5 labs results, the maximum deviation between test labs and reference value 1.70dB for BHH TRS and 1.58dB for BH TRS.

Observation 8: The max RC lab alignment deviation is within 0.75*MU, all available results are well aligned.

Proposal 3: 3GPP Rel-18 FR1 TRP TRS RC lab alignment activity can be successfully concluded (phase 3):

- 6 labs with Reverb chamber system are well aligned at n78

- 5 labs with Reverb chamber system are well aligned at n28

Proposal 4: RC vs AC harmonization can be further discussed until test labs finalize the LADs measurement.

3 Conclusion

Based on the above analysis in this paper, the following final outcome of Rel-18 3GPP FR1 TRP TRS AC lab alignment activity is proposed:

Observation 1: For TRP, no outlier value is identified. The reference value is derived by the agreed linear average (with dBm) of all the available results, the maximum deviation between test labs and reference value is 0.76dB for n78 TRP and 1.22dB for n28 TRP.

Observation 2: For TRS, no outlier value is identified. The reference value is derived by the agreed linear average (with dBm) of all the available results, the maximum deviation between test labs and reference value is 1.24dB for n78 TRS and 1.55dB for n28 TRS.

Observation 3: The maximum deviation between test labs and reference value for n78 and n28 are within 0.75*MU, all the 8 labs with available results are well aligned at band n78 and n28.

Observation 4: Compared with phase 2 results submitted until RAN4#110 meeting, adding Lab 1 AC n28 results and updated Lab 3 AC n28 results based on retesting. The updated results are considered in this analysis.

Proposal 1: 3GPP Rel-18 FR1 TRP TRS AC lab alignment activity for BHH can be successfully concluded within the 8 labs with anechoic chamber system are well aligned at FR1 (n78 to represent high band, and n28 to represent low band).

Based on the above analysis in this paper, the following outcome of Rel-18 3GPP FR1 TRP TRS RC lab alignment and RC harmonization activity is proposed:

Observation 6: For n78 and n28 TRP, the reference value is derived by the agreed linear average (with dBm) of all the 5 labs results, the maximum deviation between test labs and RC reference value is 1.47dB for BHH TRP and 1.18dB for BH TRP.

Observation 7: For n78 and n28 TRS, the reference value is derived by the agreed linear average (with dBm) of all the 5 labs results, the maximum deviation between test labs and reference value 1.70dB for BHH TRS and 1.58dB for BH TRS.

Observation 8: The max RC lab alignment deviation is within 0.75*MU, all available results are well aligned.

Proposal 3: 3GPP Rel-18 FR1 TRP TRS RC lab alignment activity can be successfully concluded (phase 3):

- 6 labs with Reverb chamber system are well aligned at n78

- 5 labs with Reverb chamber system are well aligned at n28

Proposal 4: RC vs AC harmonization can be further discussed until test labs finalize the LADs measurement.

References

- R4-2316995, Working procedure for Rel-18 lab alignment and harmonization campaign, vivo, RAN4#108bis

- R4-2321099, WF on NR_FR1_TRP_TRS_enh, vivo, 3GPP TSG-RAN WG4 Meeting #109

- R4-2402875, WF for [110][331] NR_FR1_TRP_TRS_enh, vivo, RAN4#110.

- R4-2318103	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity-Huawei	Huawei, HiSilicon

- R4-2318970	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	vivo

- R4-2319641	3GPP Rel-18 TRP TRS AC lab alignment activity from SRTC	SRTC

- R4-2319920	3GPP Rel-18 TRP TRS LAD measurement for AC lab alignment_n78	OPPO

- R4-2319288	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	SGS Wireless

- R4-2320177	CAICT measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity	CAICT

- R4-2318104	Measurement results for 3GPP Rel-18 TRP TRS RC harmonization-Huawei	Huawei, HiSilicon

- R4-2318971	Measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity	vivo

- R4-2319635	3GPP Rel-18 TRP TRS RC harmonization from SRTC	SRTC

- R4-2319921	3GPP Rel-18 TRP TRS LAD measurement for RC harmonization_n78	OPPO

- R4-2320178	CAICT measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity	CAICT

- R4-2400217	3GPP Rel-18 TRP TRS LAD measurements for AC lab alignment - Sporton	Sporton International Inc

- R4-2400551	3GPP Rel-18 TRP TRS LAD measurements for AC lab alignment - Sporton n28	Sporton International Inc

- R4-2400717	ELEMENT - Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment LAD1 and LAD2	Element Materials Technology

- R4-2400718	ELEMENT - Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment LAD3 and LAD4	Element Materials Technology

- R4-2400871	3GPP Rel-18 TRP TRS AC lab alignment activity from SRTC	SRTC

- R4-2401540	Measurement results of 3GPP Rel-18 TRP TRS AC lab alignment activity-full set	vivo

- R4-2401803	3GPP Rel-18 TRP TRS LAD measurement for AC lab alignment_n28	OPPO

- R4-2401834	CAICT measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity-v2	CAICT

- rev of R4-2400980	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	SGS Wireless

- R4-2401802	Update on LAD1 measurement data for lab alignment	OPPO

- R4-2400870	3GPP Rel-18 TRP TRS RC lab alignment activity from SRTC	SRTC

- R4-2401541	Measurement results of 3GPP Rel-18 TRP TRS RC harmonization activity-full set	vivo

- R4-2401801	3GPP Rel-18 TRP TRS LAD measurement for RC harmonization_n28	OPPO

- R4-2401835	CAICT measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity-v2	CAICT

- R4-2402986	Huawei-Measurement results for 3GPP Rel-18 TRP TRS RC harmonization	Huawei, HiSilicon

- Rev of R4-2404201, Huawei Measurement results for TRP TRS n28 AC talk mode- update, Huawei, HiSilicon, RAN4#110bis

- R4-2404600, 3GPP Rel-18 TRP TRS AC and RC lab alignment result update from SRTC, SRTC, RAN4#110bis

- R4-2404596, Measurement results for 3GPP Rel-18 TRP TRS RC harmonization_n78, Samsung, RAN4#110bis