3GPP TSG-RAN WG4 Meeting #110bis	R4-2404647
Changsha, China, 15th – 19th April, 2024

Agenda item:	6.9.1.1

Source:	vivo, CAICT, CMCC, China Telecom, China Unicom, Huawei, HiSilcon, Xiaomi, OPPO, MediaTek, Google

Title:	Performance metric for Coherent UL MIMO

Document for:	Approval

1	Introduction

In RAN4#109 meeting, it was agreed that the RAN4 will focus on performance metric discussion for coherent UL-MIMO with a goal to select a single metric as baseline in Rel-18 [1]. In RAN4#110 meeting, the corresponding comparison criteria to assist in down-selection were discussed and decided [2]. For sacking of progress, two more compromised options were added for consideration.

Issue 1-1-1: Performance metric for Coherent UE support multiple TPMI index 2~5 Options for next meeting: - Option 1 (averaging TRPTPMIx), TRPavg_TPMI: Option 1a averaging 4 TPMIs TRPs, Option 1b averaging 2 TPMIs TRPs - Option 2 (Max EIRPTPMIx), TRPmax_EIRP_TPMI - Option 3 (averaging Weighted Radiated Powers) with 4 TPMIs, FFS naming - Averaging of 4 partial TRPs - Option 4 (weighted averaging TRPsTPMIx), TRPweighted_avg_TPMI Agreements: RAN4 will consider above options and make decisions on a reference/baseline metric next meeting based on majority view. The following comparison criteria should be considered for making decisions next meeting. #; Criteria. 1; Testing time (calculated based on R4-2311672), considering multiple AC stabilization times. 2; Performance metric consistency. 3; Regulatory impacts. 4; Statistical properties. 5; Alignment with Other SDOs. 6; OEM antenna design. 7; Operator network deployment. 8; Representative of the operation in the field （Realism）.

This contribution provides proposals on concluding UE radiated performance metric for coherent UL-MIMO.

2	Discussion

The non-coherent UL-MIMO TRP test method has been defined in TR 38.870 v18.0.0, section 7.4.3.3, fixed TPMI index 2 is configured, the performance metric is TRP. The TxD TRP test method has been defined in TR 38.870 v18.0.0, section 7.4.3.2, with 2Tx configured, the performance metric is TRP. The common test procedure for coherent UL MIMO has been defined in section 7.4.3.3, with 2 or 4 TPMI index measurements based on UE declaration.

In last RAN4 meeting [3], the key debating on Option 1 and Option 2 is about “whether this option better presents network behaviour”, companies have different understandings, but the aligned outcome is that the group confirms Option 2 is idealized case of network. So new Option 3 and Option 4 come up, try to present the impacts of different weighting of TPMI index 2/3/4/5.

For now, there is no commercially available coherent UE in the market, it is not possible to collect the weighting information of TPMIs from real network. Therefore, to better understand the TPMI index indication probability in the network, we perform a system-level simulation to show different TPMI index indication percentage, the simulation details is listed in the Annex part of this contribution, the outcome shows that nearly the same percentage/probability of TPMI index 2/3/4/5. The statistical analysis demonstrates nearly equal weighting of the TPMIs, and then average metric of Option 1 is a good approach to better quantify the UE UL-MIMO performance. Meanwhile, under equal weighting, Option 4 is identical to Option 1.

We further compare the above Option 1 (mainly Option 1b with 2 TPMI case), Option 2 (two cases), and Option 4 with equal weighting, as summarized in Table 3:

Table 1: Performance metric selection based on comparison criteria

| Comparison criteria | Option 1b with 2 TPMI index TRPavg_TPMI | Option 2-1 (idealism) TRPmax_EIRP_TPMI | Option 2-2 (Worst case) TRPmin_EIRP_TPMI | Option 4 with equal weighting TRPequal_weighted_avg_TPMI |
| --- | --- | --- | --- | --- |
| Testing time (calculated based on R4-2311672), considering multiple AC stabilization times | Lowest (2 TPMI) ~50 to 70% of Options with 4 TPMI (Note: 50% assume separate full-TRP measurement for each TPMI index) | Highest (4 TPMI) |  | Highest (4 TPMI) Same as Option 1b, if two TPMI index case |
| Performance metric consistency | Averaging, widely used in OTA industry Aligned with other 2Tx test cases (non-coherent UL MIMO and TxD TRP) defined in TR 38.870 Aligned with 1Tx test cases, the advantages of single-layer UL MIMO performance can be estimated. | TRP using max EIRP at each point, never been used for OTA not comparable with 1Tx and other 2Tx test cases |  | Same as Option 1 |
| Regulatory impacts | Aligned with FR1 regulatory radiated power measurement, e.g., CTA (NAL), EU CE | Not aligned with any regulatory radiated power measurement |  | Same as Option 1 |
| Statistical properties | The averaged performance is close to the statistical performance of UE at a typical case in the field Well present the “Nearly equal TPMI index probability” | Can not present the correct TPMI index probability |  | Same as Option 1 |
|  |  | Ideal gNB algorithm for TPMI index indication ONLY targeting Max power at each point | Worst case of gNB algorithm for TPMI index indication targeting Min power at each point |  |
| Alignment with Other SDOs | Averaging, widely used in other OTA SDOs, e.g., CCSA, CTIA, ETSI | New metric, not adopted in any SDO |  | Same as Option 1 |
| OEM antenna design | Antenna design to meet typical performance | Antenna design assumes a perfect gNB indication | Antenna design assumes a worst gNB indication | Same as Option 1 |
| Operator network deployment | Follow the similar approach on taking traditional UE TRP OTA performance into consideration, when consider network deployment Traditional TRP OTA is 1Tx | Performance of ideal case, can not be used for real network deployment | Performance of worst case, can not provide guidance ono real network deployment | Same as Option 1 |
| Representative of the operation in the field (Realism) | Present typically/statistically UE operation in the field | Idealism, not presenting real filed | Worst case, not presenting real filed | Same as Option 1 |
|  | Note: in the field, the TPMI index indication is SRS-based, and also is gNB implementation, different gNB has different algorithm/mechanism. |  |  |  |

Regarding Option 3, we believe considering only partial TRP under each TPMI will hide the bad antenna performance at some angles, meanwhile the human body absorption impact (head or hand) may also not be correctly reflected.

In short, we think both Option 2-1 or 2-2 are corner cases of the gNB assumption, and Option 4 with equal weighting is identical to Option 1. With the overall considerations and comparison, we believe Option 1 should be baseline performance metric for Coherent UL MIMO, and Option 1b (2 TPMI index) is sufficient.

Proposal 1: Select option 1 (averaging TRPs) as baseline performance metric for coherent UL MIMO. Option 1b (2 TPMI index) is sufficient.

3 Conclusion

In this paper, we provide our proposals on concluding performance metric for coherent UL MIMO.

Proposal 1: Select option 1 (averaging TRPs) as baseline performance metric for coherent UL MIMO. Option 1b (2 TPMI index) is sufficient.

4	References

- R4-2321099, WF on NR_FR1_TRP_TRS_enh, vivo, 3GPP TSG-RAN WG4 Meeting #109

- R4-2402875, WF for [110][331] NR_FR1_TRP_TRS_enh, vivo, 3GPP TSG-RAN WG4 Meeting #110, March 2024

- R4-2401543, Performance metric for Coherent UL MIMO, vivo, CAICT, Huawei, HiSilcon, Xiaomi, OPPO, MediaTek, Google, RAN4#110

- R4-2305701, Further discussion on FR1 1 Layer UL-MIMO 2Tx SISO OTA TRP, MediaTek Inc, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting #106bis-e

- R4-2313775, On Phase Impacts on Single-Layer UL MIMO TRP Measurements and 2Tx Test Mode, Keysight Technologies, 3GPP TSG-RAN4 Meeting #108

- R4-2316319, On TxD and Single-Layer UL MIMO TRP Measurements, Keysight Technologies

- R4-2311672, Multi-TPMI TRP time estimation, Rohde & Schwarz, Keysight Technologies

- CTIA OTA Test Plan

- Discussion outcome for CCSA 1484.9 Measurement method for radiated RF power and receiver performance of wireless device Part 9:5G NR wireless device (Sub-6GHz)

- R4-2313628, 2Tx SISO OTA TRP Deviation and ECC, MediaTek Inc, 3GPP TSG-RAN WG4 Meeting #108, August 2023

- R4-2320175, Discussion on 2TX test methodology, CAICT, RAN4#109

- R4-2319915, Discussion on single-layer UL MIMO test methodology, OPPO

- R4-2320246, Discussion on TRP test method for Single-Layer UL-MIMO, Google

- R4-2320379, Discussion on FR1 2Tx TRP test method, Qualcomm

- R4-2318834, On Single-Layer UL MIMO for non-coherent/coherent UEs, Keysight Technologies

- R4-2400222, Discussion on Single-layer UL-MIMO TRP test method, Qualcomm, RAN4#110

- R4-2400706, On Single-Layer UL MIMO for non-coherent/coherent UEs, Keysight, RAN4#110

5	Annex

System level simulation details:

Table 2: System level simulation assumptions

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Urban Micro |
| Frequency Range | 0.7 GHz, 3 GHz, 6 GHz |
| Number of UEs per cell | 10 |
| Antenna setup and port layouts at gNB | (2,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2TX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Tx power | 23 dBm |
| Cell receiver noise figure | 5dB |
| Modulation | Up to 64QAM |
| Numerology | 14 OFDM symbol slot |
|  | SCS 15KHz, 50 PRB |
| Frame structure | All uplink |
| MIMO scheme | SU-MIMO |
| Transmission rank | maxRank = 1 |
| SRS configuration | Periodicity: 10 ms |
|  | Scheduling delay: 4 ms |
| Traffic model | Full buffer |
| UE distribution | Case 1: 80% indoor, 20% outdoor Case 2: 20% indoor, 80% outdoor |
| Receiver | MMSE-IRC |
| Codebook configurations | Rel-15 UL codebook: full-partial-non |
| Scheduling | Proportional fair |
| Precoder granularity | Full bandwidth |

The outcome is as following:

Table 3: Percentage of TPMIs with random power offset (±4dB) and phase offset (±40°) between two ports of coherent UL MIMO (Case 1 UE distribution: 80% indoor, 20% outdoor)

|  | Full coherent TPMI index probability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TPMI index | 0 | 1 | 2 | 3 | 4 | 5 |
| Codebook |  |  |  |  |  |  |
| 0.7GHz | 0.64% | 0.49% | 24.67% | 25.66% | 24.74% | 23.81% |
| 3GHz | 0.52% | 0.26% | 24.73% | 25.08% | 25.27% | 24.15% |
| 6GHz | 0.33% | 0.27% | 25.05% | 25.11% | 24.51% | 24.72% |

Table 4: Percentage of TPMIs with random power offset (±4dB) and phase offset (±40°) between two ports of coherent UL MIMO (Case 2 UE distribution: 20% indoor, 80% outdoor)

|  | Full coherent TPMI index probability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TPMI index | 0 | 1 | 2 | 3 | 4 | 5 |
| Codebook |  |  |  |  |  |  |
| 0.7GHz | 0.51% | 0.79% | 25.68% | 25.80% | 24.06% | 24.16% |
| 3GHz | 0.57% | 0.66% | 25.45% | 24.94% | 23.73% | 24.65% |
| 6GHz | 0.66% | 0.64% | 25.86% | 24.54% | 24.23% | 24.07% |