# 3GPP TSG-RAN WG4 Meeting # 110bis                                                           R4-2404745

Changsha, China, 15th – 19th April, 2024

Source:	China Unicom

Title:	Discussions on TDD intra-band CA HPUE

Agenda item:			9.1.2

Document for:	Approval

1. Introduction

In RAN-103 meeting, new Rel-19 WID[1] on UE RF enhancements for NR FR1/FR2 and EN-DC was approved, where companies found necessary to further increase the power class for TDD Intra-band CA scenarios into PC1.5. The objective part is copied below:

“

  - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx

    - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO

      - Example band combinations:

        - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations

      - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution

    - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO

      - Example band combinations:

        - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations

      - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution

    - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs

”

In this contribution,  we share our considerations on the maximum output power (MOP), and SAR solution.

2. Discussion

So far, the PC1.5 is defined for NR single carrier, with maximum UE power of 29dBm, and a tolerance of  +2/-3dB. In order to keep the consistency of power class definition among different UL combinations, it is proposed to reuse the existing PC1.5 definition for NR single carrier onto PC1.5 for NR TDD Intra-band UL CA cases, i.e. 29dBm maximum power and +2/-3dB tolerance.

Proposal 1:  Keep the power class definition consistent by reusing PC1.5 values for NR single carrier MOP for NR TDD Intra-band UL CA, i.e. 29dBm maximum power and +2/-3dB tolerance.

The duty cycle reporting mechanism had been widely used as the SAR solution for various types of HPUE, and it was also introduced for PC2 TDD Intra-band CA, as stated in 6.2A.1.1 section in TS38.101-1. In order to keep the consistency for the SAR solution, it is proposed to reuse the available solution by reporting UE capability maxUplinkDutyCycle-PC2-FR1, but the default duty cycle value (i.e. 50%) might need to be revisited to ensure SAR is not violated on higher transmit power at PC1.5.

Proposal 2: Reuse the existing duty cycle reporting as optional SAR solution for PC1.5 TDD Intra-band CA HPUE, and revisit the default duty cycle value if needed to ensure SAR is not violated on PC1.5.

3. Conclusion

In this contribution, we shared our views on the maximum output power (MOP) and SAR solution for NR PC1.5 TDD Intra-band CA HPUE, and made the following proposals:

Proposal 1: Keep the power class definition consistent by reusing PC1.5 values for NR single carrier MOP for NR TDD Intra-band UL CA, i.e. 29dBm maximum power and +2/-3dB tolerance.

Proposal 2: Reuse the existing duty cycle reporting as optional SAR solution for PC1.5 TDD Intra-band CA HPUE, and revisit the default duty cycle value if needed to ensure SAR is not violated on PC1.5.

3. Reference

[1] RP-240828	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)