3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404804

Changsha, China, 15th – 19th April, 2024

Title: 	Discussion on test case for Improvement on SCell/SCG setup delay

Source: 	Huawei, HiSilicon

Agenda item:	6.14.2.2

Document for:	Discussion

Introduction

This contribution provides considerations on performance requirements for Improvement on SCell/SCG setup delay.

Discussion

As per the RANP agreement, only the solution based on existing measurement is specified for R18 fast CA/DC setup. The key point of R18 fast CA/DC setup is validity check. During access to connected mode, UE is supposed to report the “valid” measurement results, and the conditions of “valid” results are defined as below. From UE implementation perspective, validity decision is to be made. If a measurement result is not “valid”, it is not supposed to be reported.

- 4.7.3 Measurement Report Requirements For a UE which supports [solution based on existing measurement] the UE shall be able to report valid measurement results upon RRC setup complete. The measurement results are considered valid if the following conditions are met: - the measurements are performed within the last [X] seconds before msg1 transmission for RRC resume/setup request, where [X] is configured by [TBD], and - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results. If network doesn’t provide configuration of [X], UE is not required to perform validity check but the measurement results satisfy measurement accuracy requirement at the measurement instance.

In last meeting, there is no consensus on whether to specify test case for SCG setup delay.

Issue 2-3: Whether define test case for Improvement on SCell/SCG setup delay FFS the following options: - Option 1: do not define test case (QC) - Msg1 transmission occasion is unknown to TE. RSRP variation from X sec before T1 based test design is not definable. - If test is for validating UE not to report, TE does not differentiate wrong UE who is doing nothing and good UE who is performing validity check. Both good and wrong UE will pass by not reporting. - Option 2: define test case

One idea of verify UE executing validity check is set as below:

Step 1: prior to [X] seconds before MSG1 transmission, UE can normally measure on target cell, e.g., RSRP0;

Step 2: at the starts of the last [X] seconds before MSG1 transmission, the target cell is turn off.

Figure 1. test step for verifying validity check function

When the UE is paged for connection setup and requested by the network to send idle mode measurements, the correct UE behaviour is that NO valid measurement result is reported. If RSRP0 is reported, it means that UE doesn’t execute validity check.

In last meeting, one concern on the above idea is that TE does not differentiate wrong UE who is doing nothing and good UE who is performing validity check, and both good and wrong UE will pass the test. However we think for UE capable of 39-8 (Measurement validation based on EMR measurement during connection setup/resume), the prerequisite is UE shall support idleInactiveNR-MeasReport-r16. It means that UE shall pass the R16 EMR test.  R16 EMR test can guarantee UE correctly performs and reports EMR measurement rather than doing nothing. Therefore the concern of “wrong UE” can be excluded in the test.

Observation 1: To verify UE execute “validity check” correctly, the following test steps can work for UE capable of 39-8 (Measurement validation based on EMR measurement), as UE has already pass by R16 EMR test.

- Step 1: prior to [X] seconds before MSG1 transmission, UE normally measures on target cell, e.g., RSRP0;

- Step 2: at the starts of the last [X] seconds before MSG1 transmission, the target cell is turn off. When the UE is paged for connection setup and requested by the network to send idle mode measurements, NO valid measurement result is reported.

For UE who supports 39-9 (Measurement validation based on non-EMR measurement), there is no test to verify the reselection UE can report measurement results as far. RAN4 can define a sperate test to verify the non-EMR UE can report measurement results during connection setup, and then define another test to verify whether the UE perform validity check.

Observation 2: For UE capable of 39-9(Measurement validation based on non-EMR measurement), the UE shall first pass by a sperate test to verify the non-EMR UE can correctly report measurement results during connection setup (new test), and then the validity check test is to be verified.

Based on the above analysis, to verify the validity check function, the test scheme for different UE capability is various. Considering the complexity and work load, we have no strong view on whether to specify the corresponding tests.

Conclusions

This contribution provides analysis on performance requirements for Improvement on SCell/SCG setup delay. The following proposals are provided:

Observation 1: To verify UE execute “validity check” correctly, the following test steps can work for UE capable of 39-8 (Measurement validation based on EMR measurement), as UE has already pass by R16 EMR test.

- Step 1: prior to [X] seconds before MSG1 transmission, UE normally measures on target cell, e.g., RSRP0;

- Step 2: at the starts of the last [X] seconds before MSG1 transmission, the target cell is turn off. When the UE is paged for connection setup and requested by the network to send idle mode measurements, NO valid measurement result is reported.

Observation 2: For UE capable of 39-9(Measurement validation based on non-EMR measurement), the UE shall first pass by a sperate test to verify the non-EMR UE can correctly report measurement results during connection setup (new test), and then the validity check test is to be verified.

Considering the complexity and work load, we have no strong view on whether to specify the corresponding tests.

Reference

[1] R4-2403548, WF on R18 Further NR mobility enhancement – part 2, Apple