3GPP TSG-RAN WG4 Meeting#110bis										                  R4-2404843

Changsha, China, 15th – 19th, April 2024

Agenda item:			10.3

Source:	Moderator (Apple)

Title:	Topic summary for [110bis][232] Reply_LS

Document for:	Information

# Introduction

This topic summary covers AI 10.1 (RRM part) and 10.2 (RRM part), including the following incoming LS.

- LS on combination of HST and RRM relaxation (R2-2311435)

- LS on defining the missing relative angular offsets and UE gain-related parameters for different power classes (R5-237837)

# Topic #1: LS on combination of HST and RRM relaxation (R2-2311435)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2404325 | Discussion on RAN2 LS on combination of HST and RRM relaxation | Apple | Observation 1: in the approved LS, RAN4 intentionally limited the agreement to FR1 only. Observation 2: FR2 HST flag (highSpeedMeasFlagFR2-r17) was mistakenly used in the CR, which should be replaced by highSpeedMeasInterFreq-r17 as captured in the approved LS. Proposal 1: update highSpeedMeasFlagFR2-r17 with highSpeedMeasInterFreq-r17 in the clarification for combination of HST and relaxed RRM measurement. |
| R4-2404326 | Reply LS on combination of HST and RRM relaxation | Apple | Placeholder for LS out. |
| R4-2404327 | CR on combination of HST and RRM relaxation - R17 | Apple | RAN4#110 agreed some new clarification on combination of HST and relaxed RRM measurement. RAN4 agreement was limited to FR1 HST. However, FR2 HST flag was mistakenly used. This CR is to correct the misused signaling name. |
| R4-2404328 | CR on combination of HST and RRM relaxation - R18 | Apple | Focus on the same issue in R18. |
| R4-2404728 | CR on combination of HST and RRM relaxation | CMCC | Correct the signalling name and in addition added the following calrification High speed requirements specified in this clause are applied provided that the relaxed measurement criteria in section x.x.x.x. are not fulfilled. |
| R4-2404729 | CR on combination of HST and RRM relaxation | CMCC | Same issue. |

## Open issues summary

### Sub-topic 1-1: Maintainance for the requirement on combination of HST and RRM relaxation

In the approved LS, RAN4 intentionally limited the agreement to FR1 only. FR2 HST flag (highSpeedMeasFlagFR2-r17) was mistakenly used in the CR, which should be replaced by highSpeedMeasInterFreq-r17 as captured in the approved LS.

Issue 1-1-1: which set of CRs is agreeable?

- Proposals

  - Proposal 1: Apple CRs in R4-2404327/28

  - Proposal 2: CMCC CRs in R4-2404728/29

- Recommended WF

# Topic#2:

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2405011 | Discussion on missing parameters for different power class | Huawei, HiSilicon | Observation 1: The value of “Y” and “Z” are defined based on the assumption that the number element for PC1/5/6 are 64/36/36. Observation 2: Taking PC3 as reference, Gmax for PC1/PC5/PC6 are proposed to be 29dB/26.5dB/26.5dB. Observation 3: Considering the worst case for the minimum gain for rough beam, Gmin are similar for different power class. Proposal 1: The UE gain G for PC1/5/6 are defined as follows: UE gain G, Rx beam peak direction UE Power class. 1; 2; 3; 4; 5; 6. Minimum, dBi; -10; FFS; -10; FFS; -10; -10. Maximum, dBi; 29; FFS; +20; FFS; 26.5; 26.5. Observation 4: The beam misalignment between fine beam and rough beam will lead to severer rough beam gain reduction for PC1/5/6 since: - Narrower beam with more antenna elements - Looser requirements for spherical coverage performance Proposal 2: The rough beam gain reduction for PC1/5/6 are defined as: Rough Beam gain reduction “D” in Rx Beam Peak direction UE Power class. 1; 2; 3; 4; 5; 6. Maximum gain reduction, dB; 10.5; FFS; 5.5; FFS; 8.5; 8.5. |
| R4-2405440 | Discussion on defining the missing testing parameter for PC1/5/6 and reply LS | Samsung | Observation 1: The antenna elements assumed for PC6 is 16, N=4, M=4 Observation 2: The range of UE gain G for PC5, that is [-5 dB, 57 dB], is too wide for PC6, which will make easy for the UE to pass RRM tests. Observation 3: Theoretically, about 316228 antenna elements to yield the 60 dB (57+3 dB loss) gain, the antenna system is impractical for a real HST UE. Proposal 1: The value Y and Z of PC1, PC5, PC6 are Value “Y” in dB, for each UE power class. 1; 2; 3; 4; 5; 6; 7. 16; 9.0; 7.0; FFS; 13; 13; FFS. Value “Z” in dB, for each UE power class. 1; 2; 3; 4; 5; 6; 7. 16; 9.0; 7.0; FFS; 13; 13; FFS. Proposal 2: The minimum allowable gain of G for power class 6 can be assumed as -23 dBi, and the maximum allowable gain of G for power class 6 can be assumed as 20 dBi Proposal 3: The minimum allowable gain of G for power class 1 can be assumed as -18 dBi, the maximum allowable gain of G for power class 1 can be assumed as 23 dBi. Proposal 4: There is no need to define G gain in Clause 2.1.6 Gain to PRS-RSRP measurement point for FR2 for PC6 Proposal 5: There is no need to define the CSI-RS_RP (Section B.2.4.2) for PC6, current PC6 UE test cases in RAN4 are defined for SSB-based measurements only Proposal 6: For UE gain difference between inter-frequencies Ginter, 3dB for PC1 and PC6 Proposal 7: There is no need to define the value of rough beam gain reduction “D” in B.2.1.5 for PC1/PC5/PC6 |
| R4-2405733 | On Missing RAN4 PC1, 5, and 6 Parameters for RAN5 | Nokia | Observation 1: Even though PC6-specific/enhanced test cases are based on SSB-based L1-RSRP measurements, regular CSI-RS based L1-RSRP test cases are still applicable to PC6 UEs. The introduction of Minimum CSI-RS_RSRP in Table B.2.4.2-2 is requested by RAN5 and is straightforward. Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2 Parameter; Angle of arrival; NR operating bands; Minimum CSI-RS_RP Note 2, Note 3; CSI-RS Ês/Iot. dBm / SCSCSI-RS; dB. SCSCSI-RS = 60 kHz; SCSCSI-RS = 120 kHz. UE Power class; UE Power class. 1; 2; 3; 4; 5; 6; 1, 2, 3, 4, 5, 6. Conditions; Rx Beam Peak; n257; -128.3+Y1; -113.8; -112.1; -127.8+Y4; -123.4+Y5; -123.4+Y6; (Value for SCSCSI-RS = 60 kHz) +3dB; ≥-3. n258; -128.3+Y1; -113.8; -112.1; -127.8+Y4; -123.6+Y5; -123.6+Y6. n259; -108.5; -120.5+Y5. n260; -125.3+Y1; -109.5; -125.8+Y4. n261; -128.3+Y1; -113.8; -112.1; -127.8+Y4; 123.4+Y6. n262; -123.3+Y1; -108.6; -106.6; -121.8+Y4. Spherical coverage Note 1; n257; -120.3+Z1; -102.8; -101.2; -118.8+Z4; -115.4+Z5; -115.4+Z6; (Value for SCSCSI-RS = 60 kHz) +3dB; ≥-3. n258; -120.3+Z1; -102.8; -101.2; -118.8+Z4; -115.6+Z5; -115.6+Z6. n259; -95.7. n260; -117.3+Z1; -96.9; -113.8+Z4. n261; -120.3+Z1; -102.8; -101.2; -118.8+Z4; -115.4+Z6. n262; -115.1+Z1; -96.7; -93.5; -109.7+Z4. NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19].. - RAN4 to define Minimum CSI-RS_RSRP for PC6 UEs as shown in the modified Table B.2.4.2-2 above. Observation 2: For PC6 devices, there is also a TC (A.7.6.1.5 SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17) that is based on L3 measurements. Table B.2.2-2: Conditions for intra-frequency measurements in FR2 Parameter; Angle of arrival; NR operating bands; Minimum SSB_RP Note 2, Note 3; SSB Ês/Iot. dBm / SCSSSB; dB. SCSSSB = 120 kHz; SCSSSB = 240 kHz. UE Power class; UE Power class. 1; 2; 3; 4; 5; 6; 1, 2, 3, 4, 5, 6. Conditions; Rx Beam Peak; n257; -128.3+Y1; -113.8; -112.1; -127.8+Y4; -123.4+Y5; -123.4+Y6; (Value for SCSSSB = 120 kHz) +3dB; ≥-6. n258; -128.3+Y1; -113.8; -112.1; -127.8+Y4; -123.6+Y5; -123.6+Y6. n259; -108.5; -120.5+Y5. n260; -125.3+Y1; -109.5; -125.8+Y4. n261; -128.3+Y1; -113.8; -112.1; -127.8+Y4; -123.4+Y6. n262; -123.3+Y1; -108,6; -106.6; -121.8+Y4. Spherical coverage Note 1; n257; -120.3+Z1; -102.8; -101.2; -118.8+Z4; -115.4+Z5; -115.4+Z6; (Value for SCSSSB = 120 kHz) +3dB; ≥-6. n258; -120.3+Z1; -102.8; -101.2; -118.8+Z4; -115.6+Z5; -115.6+Z6. n259; -95.7. n260; -117.3+Z1; -96.9; -113.8+Z4. n261; -120.3+Z1; -102.8; -101.2; -118.8+Z4; -115.4+Z6. n262; -115.1+Z1; -96.7; -93.5; -109.7+Z4. Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19].. RAN4 to define the values of Minimum SSB_RP in Table B.2.2-2: Conditions for intra-frequency measurements in FR2 for PC6 devices as shown in the table above and inform RAN5 about the change. Observation 3: The values of UE gain G, Rx beam peak direction (Table B.2.1.5.1-1) and Rough Beam gain reduction “D” in Rx Beam Peak direction (Table B.2.1.5.3-1) requested by RAN5 were still not defined in RAN4. Observation 4: RAN4 have not discussed PRS-RSRP measurement requirements for PC6 devices and PRS-RSRP may not be relevant for FWA PC1 and PC5 devices either. RAN4 to discuss whether UE gain to PRS-RSRP measurement point for FR2 (Table B.2.1.5.3-1) needs to be defined for PC1, PC5, and PC6. Ask for further clarification from RAN5, if needed. |
| R4-2405734 | Reply LS on defining the missing relative angular offsets and UE gain-related parameters for different power classes | Nokia | Draft LS out. |
| R4-2405735 | DraftCR to 38.133 on Missing PC1 Test Parameters for RAN5 | Nokia |  |
| R4-2405736 | Draft CR to 38.133 CatF R17 on PC5,6 RRM Test Configuration Parameters for RAN5 | Nokia |  |
| R4-2405760 | Draft CR to 38.133 CatF R17 on PC5,6 RRM Test Configuration Parameters for RAN5 | Nokia |  |

## Open issues summary

### Sub-topic 1-1

Issue 1-1-1: what is the assumption for Gain difference Y and Z between fine beam and rough beam for PC1/5/6？

- Proposals

  - Proposal 1: Huawei

The value of Y and Z are defined based on the assumption that then number of elements for PC1/5/6 are 64/36/36.

  - Proposal 2: Samsung

The value of Y and Z are defined based on the assumption that then number of elements for PC1/5/6 are 36/36/16. And the corresponding gain calculated are

|  | Value “Y” in dB, for each UE power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 16 | 9.0 | 7.0 | FFS | 13 | 13 | FFS |

|  | Value “Z” in dB, for each UE power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 16 | 9.0 | 7.0 | FFS | 13 | 13 | FFS |

  - Proposal 3: Qualcomm (R4-2401607)

| Value “Y” in dB, for each UE power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 7 |
| 18 (10*log(64)) | 9.0 | 7.0 | FFS | 15.5 (10*log(36)) | FFS |

| Value “Z” in dB, for each UE power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 7 |
| FFS (10*log(64)) | 9.0 | 7.0 | FFS | 15.5 (10*log(36)) | FFS |

- Recommended WF

Issue 1-1-2: what is the UE gain for PC1/5/6

- Proposals

  - Proposal 1: Huawei

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Minimum, dBi | -10 | FFS | -10 | FFS | -10 | -10 |
| Maximum, dBi | 29 | FFS | +20 | FFS | 26.5 | 26.5 |

  - Proposal 2: Samsung

- The minimum allowable gain of G for power class 6 can be assumed as -23 dBi, and the maximum allowable gain of G for power class 6 can be assumed as 20 dBi

-  The minimum allowable gain of G for power class 1 can be assumed as -18 dBi, the maximum allowable gain of G for power class 1 can be assumed as 23 dBi.

  - Proposal 3: Qualcomm (R4-2401607)

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 7 |
| Minimum, dBi | 0 | FFS | -10 | FFS | -5 | FFS |
| Maximum, dBi | 57 | FFS | +20 | FFS | 57 | FFS |

- Recommended WF

Issue 1-1-3: what is the UE rough beam gain reduction D for PC 1/5/6?

- Proposals

  - Proposal 1: Huawei

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Maximum gain reduction, dB | 10.5 | FFS | 5.5 | FFS | 8.5 | 8.5 |

  - Proposal 2: Samsung

There is no need to define the value of rough beam gain reduction “D” in B.2.1.5 for PC1/PC5/PC6

- Recommended WF

Issue 1-1-4: what is Ginter for PC 1/6?

- Proposals

  - Proposal 1: Samsung

For UE gain difference between inter-frequencies Ginter, 3dB for PC1 and PC6

  - Proposal 2: other, please specify

- Recommended WF

Issue 1-1-5: minimum CSI-RS_RP for PC6 UE?

- Proposals

  - Proposal 1: Samsung

There is no need to define the CSI-RS_RP (Section B.2.4.2) for PC6, current PC6 UE test cases in RAN4 are defined for SSB-based measurements only

  - Proposal 2: Nokia

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  |  |  | CSI-RS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  |  |  | dB |  |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  |  |  | SCSCSI-RS = 120 kHz |  |  |
|  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 1, 2, 3, 4, 5, 6 |  |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.4+Y5 | -123.4+Y6 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |  |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.6+Y5 | -123.6+Y6 |  |  |  |
|  |  | n259 |  |  | -108.5 |  | -120.5+Y5 |  |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |  |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  | 123.4+Y6 |  |  |  |
|  |  | n262 | -123.3+Y1 | -108.6 | -106.6 | -121.8+Y4 |  |  |  |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.4+Z5 | -115.4+Z6 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |  |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.6+Z5 | -115.6+Z6 |  |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |  |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  | -115.4+Z6 |  |  |  |
|  |  | n262 | -115.1+Z1 | -96.7 | -93.5 | -109.7+Z4 |  |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

Issue 1-1-6: minimum SSB_RP in Table B.2.2-2?

- Proposals

  - Proposal 1: Nokia

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 1, 2, 3, 4, 5, 6 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.4+Y5 | -123.4+Y6 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.6+Y5 | -123.6+Y6 |  |  |
|  |  | n259 |  |  | -108.5 |  | -120.5+Y5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  | -123.4+Y6 |  |  |
|  |  | n262 | -123.3+Y1 | -108,6 | -106.6 | -121.8+Y4 |  |  |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.4+Z5 | -115.4+Z6 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.6+Z5 | -115.6+Z6 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  | -115.4+Z6 |  |  |
|  |  | n262 | -115.1+Z1 | -96.7 | -93.5 | -109.7+Z4 |  |  |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |  |

  - Proposal 2: other, please specify

- Recommended WF

Issue 1-1-7: whether UE gain to PRS-RSRP measurement point for FR2 (Table B.2.1.5.3-1) needs to be defined for PC1, PC5, and PC6?

- Proposals

  - Proposal 1: Yes

  - Proposal 2: No

  - Proposal 3: Ask for further clarification from RAN5.

- Recommended WF

Issue 1-1-8: LS in R4-2405734 agreeable?

- Proposals

  - Proposal 1: Yes

  - Proposal 2: No, please provide comments.

- Recommended WF

Issue 1-1-9: CRs in R4-2405735/36 agreeable?

- Proposals

  - Proposal 1: Yes

  - Proposal 2: No, please provide comments.

- Recommended WF