3GPP TSG-RAN4 Meeting #110bis	R4-2404862

# Changsha, China, April 15 – 19, 2024

Agenda item:			6.20.1.1

Source:					LG Electronics

Title:	Remaining A-MPR NS values for SL-U

Document for:	Discussion

# Introduction

For Rel-18 NR SL-U  only 5 NS values, namely  NS_31, NS_53, NS_58, NS_60 and NS_61,were defined to complete the work in timely manner. For NR-U total of 17 NS values are used to ensure compatibility with the regulative requirements around the world in bands n46, n96 and n102. In the last meeting, A-MPR with NS_28 and NS_30 was submitted. This document initiates the discussion of the remaining 10 NS values for NR SL-U to ensure worldwide use of this feature.

# Remaining NS values for SL-U

Based on the agreement [2] and the approved Rel-19 SL CA WI, the remaining NS values need to be specified in Rel-18 maintenance.

Agreement: Remaining NS values for SL-U

- If SL is agreed as Rel-19 RAN4-led package and the remaining NS values are included, specify them in Rel-19. If not, specify them in Rel-18 maintenance.

A-MPR for 10 NS values among the remaining 12 NS values is discussed.

## SL-U PSSCH/PSCCH

Based on the simulation assumption and test scenario in [2], simulation results for A-MPR with NS_29, NS_54, NS_59, NS_63, NS_64, NS_65, NS_66, NS_67, NS_68 and NS_69 are shown below.

### 2.1.1 NS_29 (A-MPR)

For NS_29, Table 2-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-1: NS_29-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.17 | 2.15 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.47 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.49 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 3.53 | 6.32 | 2.47 | 3.17 | 3.52 | 6.30 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.49 | 3.52 | 6.31 | 2.47 | 3.17 | 3.52 | 6.30 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.53 | 6.31 | 3.17 | 3.17 | 3.52 | 6.30 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 6.32 | 5.47 | 5.07 | 5.88 | 6.30 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 3.52 | 6.32 | 2.47 | 3.17 | 2.48 | 1.23 | 3.52 | 6.29 | 2.13 | 3.15 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 3.53 | 6.32 | 2.47 | 3.17 | 2.48 | 1.22 | 3.52 | 6.30 | 2.13 | 3.16 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.53 | 6.31 | 3.16 | 3.17 | 3.16 | 2.48 | 3.52 | 6.31 | 3.16 | 3.15 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 6.31 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 6.30 | 5.47 | 5.06 |  |  |  |  |  |  |

Table 2-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-2 : NS_29-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 3.53 | 6.32 | 2.48 | 3.17 | 2.48 | 2.48 | 2.48 | 2.48 |
|  | 16 QAM | 3.53 | 6.32 | 2.48 | 3.17 | 2.48 | 2.49 | 2.48 | 2.48 |
|  | 64 QAM | 3.53 | 6.31 | 3.17 | 3.17 | 3.17 | 2.48 | 2.82 | 2.48 |
|  | 256 QAM | 5.88 | 6.32 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.07 |

Considering implementation margin, Table 2-3 can be proposed for SL-U NS_29 PSSCH/PSCCH A-MPR.

Table 2-3. NS_29 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 7.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 7.5 | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 7.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 3: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |  |  |

#### Proposal 1: Define NS_29 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-3.

### 2.1.2 NS_54 (A-MPR)

For NS_54, Table 2-4 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-4: NS_54-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5955) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.16 | 2.15 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.15 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.49 | 3.18 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5975) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.17 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.89 | 5.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 3.18 | 2.81 | 0.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 3.17 | 2.81 | 0.39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 3.53 | 2.81 | 2.14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5985) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5975) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 2.82 | 3.53 | 2.14 | 0.14 | 2.47 | 3.17 | 2.13 | 0.10 |
|  | '16QAM' | 2.82 | 3.53 | 2.14 | 0.40 | 2.47 | 3.17 | 2.13 | 0.35 |
|  | '64QAM' | 3.17 | 3.54 | 3.17 | 2.16 | 3.17 | 3.17 | 3.15 | 2.12 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 |
| '60MHz' (5995) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.14 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.39 | 2.47 | 0.66 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.17 | 2.15 | 3.17 | 2.48 | 3.16 | 2.13 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.05 |  |  |  |  |  |  |  |  |  |  |

| '60MHz' (6055) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.39 | 2.47 | 0.66 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.15 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.49 | 3.16 | 2.15 | 3.16 | 2.48 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.06 | 5.88 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.82 | 3.53 | 2.14 | 0.14 | 2.47 | 3.16 | 2.82 | 3.53 | 2.46 | 0.10 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '16QAM' | 2.82 | 3.53 | 2.15 | 0.15 | 2.47 | 3.17 | 2.81 | 3.53 | 2.46 | 0.36 | 2.13 | 0.64 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 3.53 | 3.16 | 2.15 | 2.81 | 3.17 | 3.17 | 3.53 | 3.16 | 2.12 | 2.80 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.46 | 5.06 | 5.47 | 5.06 |  |  |  |  |  |  |
| '80MHz' (6065)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.14 | 2.47 | 0.39 | 2.47 | 0.40 | 2.46 | 0.11 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.39 | 2.47 | 0.66 | 2.46 | 0.36 | 2.13 | 0.63 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.15 | 2.81 | 2.48 | 3.17 | 2.47 | 3.16 | 2.13 | 2.80 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |

Table 2-5 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-5 : NS_54-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 40MHz : 5965 60MHz : 5975 80MHz : 5985 |  |  |  | 20MHz : 5955/5975 40MHz : 5985 60MHz : 5995/6055 80MHz : 6065 |  |  |  |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 2.82 | 3.53 | 2.46 | 0.37 | 2.81 | 2.49 | 2.46 | 0.37 |
|  | 16 QAM | 2.82 | 3.53 | 2.46 | 0.64 | 2.81 | 2.50 | 2.46 | 0.63 |
|  | 64 QAM | 3.18 | 3.54 | 3.16 | 2.46 | 3.18 | 2.50 | 3.16 | 2.46 |
|  | 256 QAM | 5.48 | 5.09 | 5.47 | 5.06 | 5.89 | 5.49 | 5.88 | 5.06 |

Considering implementation margin and the centre frequency, the right side A-MPR in Table 2-5, such as, 5955/5975MHz for CBW 20MHz, 5985MHz for CBW 40MHz, 5995/6055MHz for CBW 60MHz, and 6065MHz for CBW 80MHz, can be reused with SL-U PSSCH/PSCCH MPR requirement in Table 6.2E.2F-1 of TS 38.101-1.

As a result, considering implementation margin and the centre frequency, Table 2-6 can be proposed for SL-U NS_54 PSSCH/PSCCH A-MPR.

Table 2-6. NS_54 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 2.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 3.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 6.0 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.2E.2F-3 apply. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

#### Proposal 2: Define NS_54 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-6.

### 2.1.3 NS_59 (A-MPR)

For NS_59, Table 2-7 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-7: NS_59-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (7115) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 4.68 | 2.82 | 4.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 4.68 | 2.48 | 4.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 4.68 | 3.17 | 4.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.89 | 5.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 4.67 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 4.68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 4.68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (7095) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.82 | 5.07 | 2.47 | 1.83 | 2.80 | 5.05 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 5.48 | 2.47 | 2.15 | 2.46 | 5.06 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.17 | 5.48 | 3.17 | 2.47 | 3.16 | 5.06 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.47 | 5.08 | 2.47 | 1.83 | 2.47 | 0.40 | 2.46 | 5.06 | 2.13 | 2.13 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.82 | 5.07 | 2.47 | 2.15 | 2.47 | 0.66 | 2.80 | 5.46 | 2.13 | 2.13 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 5.48 | 3.16 | 2.47 | 3.16 | 2.47 | 3.16 | 5.47 | 3.15 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.46 | 5.46 | 5.47 | 5.06 |  |  |  |  |  |  |
| '100MHz' (7075) | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 2.48 | 2.48 | 2.47 | 5.08 | 2.47 | 2.14 | 2.47 | 0.39 | 2.14 | 0.66 | 2.80 | 5.05 | 2.46 | 1.81 | 2.47 | 0.38 | 2.80 | 5.06 |
|  | '16QAM' | 2.48 | 2.48 | 2.82 | 5.48 | 2.47 | 2.15 | 2.47 | 0.66 | 2.14 | 0.66 | 2.80 | 5.06 | 2.46 | 1.81 | 2.47 | 0.64 | 2.80 | 5.05 |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 5.48 | 2.81 | 2.48 | 3.16 | 2.47 | 2.81 | 2.48 | 3.16 | 5.06 | 2.80 | 2.46 | 3.16 | 2.46 | 3.16 | 5.06 |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.48 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.06 | 5.47 | 5.47 | 5.47 | 5.05 | 5.47 | 5.47 | 5.88 | 5.06 |

Table 2-8 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-8 : NS_59-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 2.82 | 5.08 | 2.80 | 5.06 |
|  | 16 QAM | 2.82 | 5.48 | 2.80 | 5.46 |
|  | 64 QAM | 3.17 | 5.48 | 3.16 | 5.47 |
|  | 256 QAM | 5.89 | 5.48 | 5.88 | 5.47 |

Considering implementation margin, Table 2-9 can be proposed for SL-U NS_59 PSSCH/PSCCH A-MPR.

Table 2-9. NS_59 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration3 |  | Inner RB set configuration3 |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 7.0 | ≤ 4.0 | ≤ 7.0 |
|  | 16 QAM | ≤ 4.0 | ≤ 7.5 | ≤ 4.0 | ≤ 7.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.2E.2F-3 apply. |  |  |  |  |  |

#### Proposal 3: Define NS_59 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-9.

### 2.1.4 NS_63 (A-MPR)

For NS_63, Table 2-10 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-10: NS_63-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5955) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.17 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.49 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (6415) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.17 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.18 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.83 | 2.81 | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.82 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.83 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |
|  | 'QPSK' | 2.48 | 3.17 | 2.14 | 0.14 | 2.47 | 3.17 | 2.47 | 3.17 | 2.46 | 0.11 | 2.13 | 0.37 |  |  |  |  |
|  | '16QAM' | 2.48 | 3.17 | 2.15 | 0.40 | 2.47 | 2.82 | 2.47 | 3.17 | 2.46 | 0.10 | 2.13 | 0.36 |  |  |  |  |
|  | '64QAM' | 2.82 | 3.17 | 3.17 | 2.15 | 3.16 | 2.81 | 3.16 | 3.17 | 3.15 | 2.12 | 3.16 | 2.46 |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.05 | 5.47 | 5.06 |  |  |  |  |
| '80MHz' (6385)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.15 | 2.47 | 0.39 | 2.47 | 0.40 | 2.46 | 0.10 | 2.13 | 0.37 |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.66 | 2.47 | 0.66 | 2.46 | 0.35 | 2.13 | 0.37 |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.15 | 2.81 | 2.48 | 3.16 | 2.48 | 3.16 | 2.13 | 2.80 | 2.46 |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.06 |  |  |  |  |

Table 2-11 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-11 : NS_63-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz : 6415 40MHz : 5965 80MHz : 5985/6385 |  |  |  | 20MHz : 5955 |  |  |  |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 2.81 | 3.17 | 2.46 | 0.37 | 2.48 | 2.50 | - | - |
|  | 16 QAM | 2.81 | 3.17 | 2.46 | 0.36 | 2.48 | 2.50 | - | - |
|  | 64 QAM | 3.17 | 3.17 | 3.16 | 2.46 | 3.18 | 2.50 | - | - |
|  | 256 QAM | 5.89 | 5.08 | 5.47 | 5.06 | 5.89 | 5.08 | - | - |

Considering implementation margin and the centre frequency, the right side A-MPR in Table 2-11, including the centre frequencies other than the centre frequencies in the left side(such as, the centre frequency other than 6415MHz for 20MHz, 5965MHz for 40MHz, 5985/6385 MHz for 80MHz), can be reused with SL-U PSSCH/PSCCH MPR requirement in Table 6.2E.2F-1 of TS 38.101-1.

As a result, considering implementation margin and the centre frequency, Table 2-12 can be proposed for SL-U NS_63 PSSCH/PSCCH A-MPR.

Table 2-12. NS_63 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 | ≤ 2.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 3.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 6415 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 and 6405 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 and 6385 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |  |  |

#### Proposal 4: Define NS_63 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-12.

### 2.1.5 NS_64 (A-MPR)

For NS_64, Table 2-13 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-13: NS_64-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5955) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 8.96 | 9.44 | 12.74 | 13.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 8.96 | 9.43 | 12.74 | 12.76 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 8.96 | 9.43 | 12.74 | 13.24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 8.96 | 8.97 | 12.74 | 12.76 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5975) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 6.31 | 8.98 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 6.31 | 8.98 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 6.31 | 8.97 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 6.31 | 8.97 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 8.05 | 8.50 | 8.96 | 9.43 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 8.05 | 8.51 | 8.50 | 9.43 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 8.05 | 8.51 | 8.96 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 8.05 | 8.51 | 8.96 | 9.43 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (6005) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.17 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.17 | 5.89 | 6.30 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5975) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 8.50 | 8.50 | 12.73 | 13.23 | 9.88 | 9.89 | 6.73 | 9.42 |
|  | '16QAM' | 8.50 | 8.50 | 12.74 | 12.75 | 9.88 | 9.89 | 6.73 | 9.42 |
|  | '64QAM' | 8.50 | 8.51 | 12.73 | 13.23 | 9.42 | 9.89 | 6.73 | 9.42 |
|  | '256QAM' | 8.50 | 8.51 | 12.73 | 13.23 | 9.88 | 9.89 | 6.73 | 8.96 |
| '60MHz' (5995) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 7.17 | 7.17 | 6.73 | 9.44 | 5.88 | 6.31 | 6.73 | 8.95 |
|  | '16QAM' | 7.17 | 7.17 | 6.73 | 9.43 | 6.30 | 6.31 | 6.73 | 9.42 |
|  | '64QAM' | 7.17 | 7.17 | 6.73 | 9.43 | 5.88 | 6.30 | 6.73 | 8.96 |
|  | '256QAM' | 7.17 | 7.17 | 6.73 | 9.43 | 5.88 | 6.31 | 6.73 | 9.42 |

| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 8.05 | 8.05 | 12.74 | 13.73 | 9.88 | 9.89 | 8.50 | 8.51 | 6.73 | 9.42 | 5.88 | 6.30 |  |  |  |  |  |  |
|  | '16QAM' | 8.05 | 8.05 | 12.74 | 13.24 | 9.42 | 9.42 | 8.50 | 8.50 | 6.73 | 9.41 | 5.88 | 6.30 |  |  |  |  |  |  |
|  | '64QAM' | 8.05 | 8.05 | 13.22 | 13.72 | 9.88 | 12.26 | 8.50 | 8.50 | 6.73 | 9.42 | 5.88 | 6.30 |  |  |  |  |  |  |
|  | '256QAM' | 8.05 | 8.06 | 12.73 | 13.72 | 9.88 | 9.42 | 8.50 | 8.50 | 6.73 | 9.42 | 5.88 | 6.30 |  |  |  |  |  |  |
| '80MHz' (6065) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.82 | 6.73 | 9.43 | 3.53 | 5.89 | 2.47 | 4.28 | 6.73 | 9.41 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.82 | 6.73 | 9.43 | 3.53 | 5.89 | 2.47 | 4.28 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.82 | 6.73 | 9.43 | 3.53 | 5.88 | 3.17 | 4.28 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 6.73 | 9.44 | 5.47 | 5.89 | 5.47 | 5.07 | 6.73 | 9.42 | 5.47 | 5.88 |  |  |  |  |  |  |
| '100MHz' (5995) | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 8.05 | 8.05 | 13.22 | 12.75 | 9.88 | 9.89 | 8.50 | 8.51 | 8.05 | 8.05 | 6.73 | 9.42 | 5.88 | 6.30 | 7.16 | 7.17 | 6.73 | 8.95 |
|  | '16QAM' | 8.05 | 8.05 | 12.73 | 13.23 | 9.42 | 10.83 | 8.50 | 8.51 | 8.05 | 8.05 | 6.73 | 9.42 | 5.88 | 6.30 | 7.16 | 7.60 | 6.73 | 9.42 |
|  | '64QAM' | 8.05 | 8.05 | 13.22 | 13.23 | 9.88 | 9.89 | 8.50 | 8.51 | 8.05 | 8.05 | 6.73 | 9.42 | 5.88 | 6.30 | 7.16 | 7.16 | 6.73 | 9.42 |
|  | '256QAM' | 8.05 | 8.05 | 12.73 | 13.23 | 9.88 | 9.89 | 8.50 | 8.51 | 8.05 | 8.05 | 6.73 | 9.41 | 5.88 | 6.30 | 7.17 | 7.16 | 6.73 | 8.96 |
| '100MHz' (6055) | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 5.06 | 5.89 | 6.73 | 9.43 | 3.52 | 5.89 | 2.47 | 4.28 | 3.16 | 4.28 | 6.73 | 9.41 | 3.52 | 5.88 | 2.47 | 3.89 | 6.73 | 8.96 |
|  | '16QAM' | 5.06 | 5.88 | 6.73 | 9.43 | 3.53 | 5.89 | 2.47 | 4.28 | 3.16 | 4.28 | 6.73 | 9.41 | 3.52 | 5.88 | 2.47 | 3.89 | 6.72 | 9.41 |
|  | '64QAM' | 5.06 | 5.88 | 6.73 | 9.43 | 3.53 | 5.88 | 3.16 | 4.28 | 3.16 | 4.28 | 6.73 | 9.42 | 3.52 | 5.88 | 3.16 | 3.89 | 6.73 | 9.41 |
|  | '256QAM' | 5.47 | 5.88 | 6.73 | 9.43 | 5.47 | 5.89 | 5.47 | 5.07 | 5.47 | 5.07 | 6.73 | 9.42 | 5.47 | 5.88 | 5.47 | 5.06 | 6.73 | 9.42 |

Table 2-14 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-14 : NS_64-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz : 5955 40MHz : 5965 60MHz : 5975 80MHz : 5985 100MHz: 5995 |  |  |  | 20MHz : 5975 40MHz : 6005 60MHz : 5995 80MHz : 6065 100MHz: 6055 |  |  |  |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 13.22 | 13.73 | 7.16 | 9.42 | 7.17 | 9.44 | 6.73 | 9.41 |
|  | 16 QAM | 12.74 | 13.24 | 7.16 | 9.42 | 7.17 | 9.43 | 6.73 | 9.42 |
|  | 64 QAM | 13.22 | 13.72 | 7.16 | 9.42 | 7.17 | 9.43 | 6.73 | 9.42 |
|  | 256 QAM | 12.74 | 13.72 | 7.17 | 9.42 | 7.17 | 9.44 | 6.73 | 9.42 |

Considering implementation margin and VLP UE, Table 2-15 can be proposed for SL-U NS_64 PSSCH/PSCCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 2-15. NS_64 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation3 |  |  |  | RB Allocation4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 14.0 | ≤ 15.0 | ≤ 9.0 | ≤ 11.5 | ≤ 9.0 | ≤ 11.5 | ≤ 8.5 | ≤ 11.5 |
|  | 16 QAM | ≤ 14.0 | ≤ 15.0 | ≤ 9.0 | ≤ 11.5 | ≤ 9.0 | ≤ 11.5 | ≤ 8.5 | ≤ 11.5 |
|  | 64 QAM | ≤ 14.0 | ≤ 15.0 | ≤ 9.0 | ≤ 11.5 | ≤ 9.0 | ≤ 11.5 | ≤ 8.5 | ≤ 11.5 |
|  | 256 QAM | ≤ 15.0 | ≤ 15.0 | ≤ 9.0 | ≤ 11.5 | ≤ 9.0 | ≤ 11.5 | ≤ 8.5 | ≤ 11.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz, and 100 MHz channels centered at the nearest NR-ARFCN corresponding to 5995 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. |  |  |  |  |  |  |  |  |  |

#### Proposal 5: Define NS_64 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-15.

### 2.1.6 NS_65 (A-MPR)

For NS_65, Table 2-16 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-16: NS_65-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5955) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.17 | 3.18 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.49 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5975) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.15 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.39 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.17 | 2.15 | 3.16 | 2.48 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.07 | 5.88 | 5.06 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.40 | 2.48 | 0.66 | 2.47 | 0.40 | 2.46 | 0.10 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.39 | 2.47 | 0.39 | 2.47 | 0.66 | 2.46 | 0.35 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.15 | 2.81 | 2.48 | 3.16 | 2.47 | 3.16 | 2.11 | 3.16 | 2.47 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.05 |  |  |  |  |  |  |
| '100MHz' (5995) | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.39 | 2.47 | 0.66 | 2.47 | 0.40 | 2.14 | 0.67 | 2.13 | 0.36 | 2.46 | 0.11 | 2.47 | 0.38 | 2.13 | 0.10 |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.66 | 2.47 | 0.65 | 2.14 | 0.66 | 2.13 | 0.36 | 2.46 | 0.36 | 2.47 | 0.64 | 2.13 | 0.09 |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.15 | 2.81 | 2.48 | 3.16 | 2.47 | 2.81 | 2.47 | 3.16 | 2.12 | 3.16 | 2.46 | 3.16 | 2.47 | 3.16 | 2.12 |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.06 | 5.47 | 5.06 | 5.46 | 5.05 |

Table 2-17 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-17 : NS_65-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 2.81 | 2.50 | 2.47 | 0.38 |
|  | 16 QAM | 2.81 | 2.49 | 2.47 | 0.64 |
|  | 64 QAM | 3.18 | 2.50 | 3.16 | 2.47 |
|  | 256 QAM | 5.89 | 5.08 | 5.88 | 5.06 |

Considering implementation margin and VLP UE, Table 2-18 can be proposed for SL-U NS_65 PSSCH/PSCCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 2-18. NS_65 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration3 |  | Inner RB set configuration3 |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.5 | ≤ 7.0 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.2E.2F-3 apply. |  |  |  |  |  |

#### Proposal 6: Define NS_65 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-18.

### 2.1.7 NS_66 (A-MPR)

For NS_66, Table 2-19 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-19: NS_66-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (7115) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 15.66 | 17.65 | 15.66 | 17.66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 15.66 | 17.65 | 15.67 | 17.65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 15.66 | 17.65 | 15.67 | 17.66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 15.66 | 17.65 | 15.67 | 17.66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 12.25 | 14.68 | 15.66 | 17.65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 12.25 | 14.68 | 15.66 | 18.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 12.25 | 14.68 | 15.66 | 18.14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 12.25 | 14.69 | 15.66 | 17.65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (7095) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 10.82 | 13.22 | 15.66 | 18.15 | 12.73 | 15.18 | 15.66 | 18.13 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 10.82 | 13.22 | 15.66 | 18.15 | 12.74 | 15.18 | 15.66 | 18.13 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 10.82 | 13.22 | 15.66 | 18.15 | 12.73 | 15.18 | 15.66 | 18.13 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 10.82 | 13.22 | 15.66 | 18.15 | 12.74 | 15.18 | 15.66 | 18.12 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 9.42 | 11.78 | 15.66 | 18.15 | 12.73 | 15.18 | 10.82 | 13.22 | 15.65 | 18.13 | 12.73 | 15.17 |  |  |  |  |  |  |
|  | '16QAM' | 9.42 | 11.77 | 15.66 | 18.15 | 12.73 | 15.18 | 10.82 | 13.22 | 15.66 | 18.13 | 12.73 | 15.16 |  |  |  |  |  |  |
|  | '64QAM' | 9.42 | 11.77 | 15.66 | 18.14 | 12.74 | 15.18 | 10.82 | 13.23 | 15.65 | 18.14 | 12.73 | 15.17 |  |  |  |  |  |  |
|  | '256QAM' | 9.42 | 11.78 | 15.67 | 18.15 | 12.73 | 15.17 | 10.82 | 13.22 | 15.65 | 18.12 | 12.73 | 15.16 |  |  |  |  |  |  |
| '100MHz' (7075) | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 8.50 | 10.82 | 15.66 | 18.14 | 12.73 | 15.17 | 10.82 | 13.22 | 9.42 | 11.78 | 15.65 | 18.13 | 12.73 | 15.17 | 10.82 | 13.21 | 15.66 | 18.13 |
|  | '16QAM' | 8.50 | 10.82 | 15.66 | 18.14 | 12.73 | 15.18 | 10.82 | 13.22 | 9.42 | 11.78 | 15.66 | 18.13 | 12.73 | 15.16 | 10.82 | 13.21 | 15.66 | 18.13 |
|  | '64QAM' | 8.50 | 10.82 | 15.66 | 18.15 | 12.73 | 15.18 | 10.82 | 13.22 | 9.42 | 11.78 | 15.66 | 18.13 | 12.73 | 15.17 | 10.82 | 13.22 | 15.65 | 18.13 |
|  | '256QAM' | 8.50 | 10.83 | 15.66 | 18.15 | 12.73 | 15.17 | 10.82 | 13.22 | 9.42 | 11.77 | 15.66 | 18.12 | 12.73 | 15.17 | 10.82 | 13.22 | 15.66 | 18.13 |

Table 2-20 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-20 : NS_66-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 15.66 | 18.15 | 12.73 | 15.18 | 10.82 | 13.22 | 9.42 | 11.78 | 8.50 | 10.82 |
|  | 16 QAM | 15.67 | 18.15 | 12.74 | 15.18 | 10.82 | 13.22 | 9.42 | 11.78 | 8.50 | 10.82 |
|  | 64 QAM | 15.67 | 18.15 | 12.74 | 15.18 | 10.82 | 13.23 | 9.42 | 11.78 | 8.50 | 10.82 |
|  | 256 QAM | 15.67 | 18.15 | 12.74 | 15.18 | 10.82 | 13.22 | 9.42 | 11.78 | 8.50 | 10.83 |

Considering implementation margin and VLP UE, Table 2-21 can be proposed for SL-U NS_66 PSSCH/PSCCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 2-21. NS_66 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 16.5 | ≤ 19.0 | ≤ 13.5 | ≤ 16.0 | ≤ 11.5 | ≤ 14.0 | ≤ 10.0 | ≤ 12.5 | ≤ 9.0 | ≤ 11.5 |
|  | 16 QAM | ≤ 16.5 | ≤ 19.0 | ≤ 13.5 | ≤ 16.0 | ≤ 11.5 | ≤ 14.0 | ≤ 10.0 | ≤ 12.5 | ≤ 9.0 | ≤ 11.5 |
|  | 64 QAM | ≤ 16.5 | ≤ 19.0 | ≤ 13.5 | ≤ 16.0 | ≤ 11.5 | ≤ 14.0 | ≤ 10.0 | ≤ 12.5 | ≤ 9.0 | ≤ 11.5 |
|  | 256 QAM | ≤ 16.5 | ≤ 19.0 | ≤ 13.5 | ≤ 16.0 | ≤ 11.5 | ≤ 14.0 | ≤ 10.0 | ≤ 12.5 | ≤ 9.0 | ≤ 11.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 7: Define NS_66 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-21.

### 2.1.8 NS_67 (A-MPR)

For NS_67, Table 2-22 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-22: NS_67-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (7115) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 12.74 | 14.69 | 12.74 | 14.70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 12.73 | 14.69 | 12.74 | 14.70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 12.74 | 14.70 | 12.74 | 14.71 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 12.74 | 14.70 | 12.74 | 14.70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 9.42 | 11.78 | 12.74 | 14.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 9.42 | 11.77 | 12.74 | 14.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 9.42 | 11.78 | 12.74 | 14.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 9.42 | 11.78 | 12.74 | 14.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (7095) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 7.61 | 9.89 | 12.74 | 15.18 | 9.42 | 12.26 | 12.73 | 15.17 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 7.61 | 9.89 | 12.74 | 15.18 | 9.42 | 12.26 | 12.73 | 15.17 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 7.61 | 9.89 | 12.74 | 15.19 | 9.42 | 12.26 | 12.73 | 15.17 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 7.61 | 10.35 | 12.74 | 15.18 | 9.42 | 12.26 | 12.73 | 15.18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 6.30 | 8.96 | 12.73 | 15.19 | 9.42 | 12.26 | 7.61 | 10.35 | 12.73 | 15.17 | 9.42 | 12.25 |  |  |  |  |  |  |
|  | '16QAM' | 6.30 | 8.96 | 12.74 | 15.19 | 9.42 | 12.26 | 7.61 | 10.35 | 12.73 | 15.17 | 9.42 | 12.25 |  |  |  |  |  |  |
|  | '64QAM' | 6.30 | 8.96 | 12.74 | 15.19 | 9.42 | 12.26 | 7.61 | 10.35 | 12.73 | 15.17 | 9.42 | 12.25 |  |  |  |  |  |  |
|  | '256QAM' | 6.30 | 8.96 | 12.74 | 15.18 | 9.42 | 12.26 | 7.60 | 10.36 | 12.73 | 15.17 | 9.42 | 12.25 |  |  |  |  |  |  |
| '100MHz' (7075) | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 5.47 | 8.05 | 12.74 | 15.18 | 9.42 | 12.26 | 7.60 | 10.35 | 6.30 | 8.96 | 12.73 | 15.17 | 9.42 | 12.25 | 7.60 | 9.88 | 12.73 | 15.17 |
|  | '16QAM' | 5.47 | 8.05 | 12.74 | 15.18 | 9.42 | 12.26 | 7.60 | 10.35 | 6.30 | 8.96 | 12.73 | 15.16 | 9.42 | 12.25 | 7.60 | 10.34 | 12.73 | 15.18 |
|  | '64QAM' | 5.47 | 8.05 | 12.74 | 15.20 | 9.42 | 12.26 | 7.61 | 10.35 | 6.30 | 8.96 | 12.73 | 15.17 | 9.41 | 12.25 | 7.60 | 9.88 | 12.73 | 15.17 |
|  | '256QAM' | 5.47 | 8.05 | 12.73 | 15.19 | 9.42 | 12.26 | 7.60 | 10.35 | 6.30 | 8.96 | 12.74 | 15.17 | 9.42 | 12.25 | 7.60 | 9.88 | 12.73 | 15.16 |

Table 2-23 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-23 : NS_67-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 12.74 | 15.19 | 9.42 | 12.26 | 7.61 | 10.35 | 6.30 | 8.96 | 5.47 | 8.05 |
|  | 16 QAM | 12.74 | 15.19 | 9.42 | 12.26 | 7.61 | 10.35 | 6.30 | 8.96 | 5.47 | 8.05 |
|  | 64 QAM | 12.74 | 15.20 | 9.42 | 12.26 | 7.61 | 10.35 | 6.30 | 8.96 | 5.47 | 8.05 |
|  | 256 QAM | 12.74 | 15.19 | 9.42 | 12.26 | 7.61 | 10.36 | 6.30 | 8.96 | 5.47 | 8.05 |

Considering implementation margin and VLP UE, Table 2-24 can be proposed for SL-U NS_67 PSSCH/PSCCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 2-24. NS_67 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 13.5 | ≤ 16.0 | ≤ 10.5 | ≤ 13.5 | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 9.0 |
|  | 16 QAM | ≤ 13.5 | ≤ 16.0 | ≤ 10.5 | ≤ 13.5 | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 9.0 |
|  | 64 QAM | ≤ 13.5 | ≤ 16.0 | ≤ 10.5 | ≤ 13.5 | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 9.0 |
|  | 256 QAM | ≤ 13.5 | ≤ 16.0 | ≤ 10.5 | ≤ 13.5 | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 8: Define NS_67 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-24.

### 2.1.9 NS_68 (A-MPR)

For NS_68, Table 2-25 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-25: NS_68-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5955) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 6.30 | 8.97 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 6.31 | 8.97 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 6.30 | 8.97 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 6.31 | 8.97 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.17 | 5.88 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.17 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5975) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 3.91 | 6.73 | 9.43 | 3.53 | 5.88 | 6.73 | 8.95 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 3.90 | 6.73 | 9.43 | 3.53 | 5.89 | 6.73 | 8.95 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 3.90 | 6.74 | 9.43 | 3.53 | 5.89 | 6.73 | 8.95 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 6.73 | 9.44 | 5.47 | 6.31 | 6.73 | 9.42 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.82 | 6.73 | 9.43 | 3.53 | 5.89 | 2.47 | 4.28 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.82 | 6.73 | 9.42 | 3.53 | 6.31 | 2.47 | 4.28 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.82 | 6.74 | 9.43 | 3.53 | 5.89 | 3.16 | 4.28 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 6.73 | 9.43 | 5.47 | 5.89 | 5.47 | 5.07 | 6.73 | 9.42 | 5.47 | 5.88 |  |  |  |  |  |  |
| '100MHz' (5995) | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 2.48 | 2.48 | 6.73 | 9.43 | 3.53 | 5.89 | 2.47 | 4.28 | 2.14 | 2.82 | 6.73 | 9.42 | 3.52 | 5.88 | 2.47 | 3.89 | 6.73 | 8.95 |
|  | '16QAM' | 2.48 | 2.48 | 6.73 | 9.43 | 3.52 | 5.89 | 2.47 | 4.28 | 2.14 | 2.82 | 6.73 | 9.42 | 3.52 | 5.88 | 2.47 | 3.89 | 6.73 | 9.42 |
|  | '64QAM' | 2.82 | 2.48 | 6.74 | 9.43 | 3.53 | 6.31 | 3.16 | 4.28 | 2.81 | 2.82 | 6.73 | 9.42 | 3.52 | 5.88 | 3.16 | 3.89 | 6.73 | 8.96 |
|  | '256QAM' | 5.47 | 5.07 | 6.73 | 9.43 | 5.47 | 5.89 | 5.47 | 5.07 | 5.47 | 5.06 | 6.73 | 9.42 | 5.47 | 5.88 | 5.47 | 5.47 | 6.73 | 8.95 |

Table 2-26 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-26 : NS_68-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 6.74 | 9.43 | 3.53 | 5.89 | 2.48 | 4.28 | 2.48 | 2.82 | 2.48 | 2.48 |
|  | 16 QAM | 6.74 | 9.43 | 3.53 | 6.31 | 2.48 | 4.28 | 2.48 | 2.82 | 2.48 | 2.48 |
|  | 64 QAM | 6.74 | 9.43 | 3.53 | 6.31 | 3.17 | 4.28 | 2.82 | 2.82 | 2.82 | 2.48 |
|  | 256 QAM | 6.74 | 9.44 | 5.47 | 6.31 | 5.47 | 5.47 | 5.47 | 5.07 | 5.47 | 5.07 |

Considering implementation margin and VLP UE, Table 2-27 can be proposed for SL-U NS_68 PSSCH/PSCCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 2-27. NS_68 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.0 | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.0 | ≤ 6.0 | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.0 | ≤ 6.0 | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.0 | ≤ 6.0 | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 9: Define NS_68 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-27.

### 2.1.10 NS_69 (A-MPR)

For NS_69, Table 2-28 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 2-28: NS_69-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5955) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.17 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.17 | 2.15 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.49 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (6415) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.82 | 3.54 | 3.17 | 3.92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.82 | 3.54 | 2.83 | 3.92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 3.54 | 3.17 | 3.92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5965) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.82 | 2.81 | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.83 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.82 | 2.81 | 2.48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |
|  | 'QPSK' | 2.48 | 3.17 | 2.14 | 0.14 | 2.47 | 3.17 | 2.47 | 3.17 | 2.46 | 0.09 | 2.13 | 0.37 |  |  |  |  |
|  | '16QAM' | 2.48 | 3.17 | 2.14 | 0.40 | 2.47 | 2.82 | 2.47 | 3.17 | 2.46 | 0.10 | 2.13 | 0.63 |  |  |  |  |
|  | '64QAM' | 2.82 | 3.17 | 3.17 | 2.15 | 3.16 | 2.82 | 3.16 | 3.16 | 3.16 | 2.13 | 3.16 | 2.46 |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.06 |  |  |  |  |
| '80MHz' (6385)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.14 | 2.47 | 0.39 | 2.47 | 0.66 | 2.46 | 0.10 | 2.13 | 0.37 |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.15 | 0.40 | 2.47 | 0.39 | 2.47 | 0.66 | 2.46 | 0.36 | 2.13 | 0.63 |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.15 | 3.17 | 2.47 | 3.16 | 2.48 | 3.16 | 2.12 | 3.16 | 2.46 |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.06 |  |  |  |  |

Table 2-29 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 2-29 : NS_69-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 80MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 2.81 | 2.50 | 2.48 | 3.17 | 2.48 | 3.17 |
|  | 16 QAM | 2.81 | 2.50 | 2.48 | 2.83 | 2.48 | 3.17 |
|  | 64 QAM | 3.17 | 2.50 | 3.16 | 2.82 | 2.82 | 3.17 |
|  | 256 QAM | 5.48 | 5.08 | 5.47 | 5.07 | 5.47 | 5.07 |

Considering implementation margin and VLP UE, Table 2-30 can be proposed for SL-U NS_69 PSSCH/PSCCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 2-30. NS_69 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 80MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 3: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |

#### Proposal 10: Define NS_69 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-30.

## SL-U PSFCH

Based on the simulation assumption and test scenario in [2], simulation results for A-MPR with NS_29, NS_54, NS_59, NS_63, NS_64, NS_65, NS_66, NS_67, NS_68 and NS_69 are shown below.

### 2.2.1 NS_29 (A-MPR)

For NS_29, Table 3-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-1: NS_29-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5239.98) | 8.27 | 7.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5230.02) | 10.01 | 6.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5220) | 11.13 | 8.65 | 7.30 | 7.00 | 9.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5210) | 12.23 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.54 | 9.26 | 9.66 |  |  |  |  |  |  |  |  |  |  |

Table 3-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-2 : NS_29-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz (Full/Partial) | 40MHz (Full/Partial) | 60MHz (Full/Partial) | 80MHz (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 8.27 | 10.01 | 11.13 | 12.23 |

Considering implementation margin, Table 3-3 can be proposed for SL-U NS_29 PSFCH A-MPR.

Table 3-3. NS_29 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz (Full/Partial) | 40MHz (Full/Partial) | 60MHz (Full/Partial) | 80MHz (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 11.0 | ≤12.5 | ≤13.5 | ≤15.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |

#### Proposal 11: Define NS_29 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-3.

### 2.2.2 NS_54 (A-MPR)

For NS_54, Table 3-4 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-4: NS_54-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 8.27 | 7.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 10.01 | 6.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 11.13 | 8.65 | 7.30 | 7.00 | 9.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 12.23 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.54 | 9.26 | 9.66 |  |  |  |  |  |  |  |  |  |  |

Table 3-5 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-5 : NS_54-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | 12.23 | 9.96 |

Considering implementation margin, Table 3-6 can be proposed for SL-U NS_54 PSFCH A-MPR.

Table 3-6. NS_54 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |
| --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer/Inner RB sets |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 15.0 | ≤ 12.5 | Table 6.2E.2F-4(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |

#### Proposal 12: Define NS_54 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-6.

### 2.2.3 NS_59 (A-MPR)

For NS_59, Table 3-7 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-7: NS_59-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 8.27 | 7.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 10.01 | 6.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 11.13 | 8.65 | 7.30 | 7.00 | 9.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 12.23 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.54 | 9.26 | 9.66 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 13.28 | 9.85 | 9.67 | 8.69 | 7.42 | 9.44 | 7.96 | 7.09 | 6.49 | 12.15 | 10.18 | 10.14 | 10.33 | 10.60 | 9.33 | 8.97 | 9.67 | 8.81 | 11.33 |

Table 3-8 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-8 : NS_59-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | 13.28 | 9.96 |

Considering implementation margin, Table 3-9 can be proposed for SL-U NS_59 PSFCH A-MPR.

Table 3-9. NS_59 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 16.0 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |

#### Proposal 13: Define NS_59 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-9.

### 2.2.4 NS_63 (A-MPR)

For NS_63, Table 3-10 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-10: NS_63-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (6415) | 8.27 | 7.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (6405) | 10.01 | 6.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (6385) | 12.23 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.54 | 9.26 | 9.66 |  |  |  |  |  |  |  |  |  |  |

Table 3-11 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-11 : NS_63-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | 12.23 | 9.96 |

Considering implementation margin, Table 3-12 can be proposed for SL-U NS_63 PSFCH A-MPR. Here, same A-MPR can be assumed for the centred frequency for each edge side, as follows.

: 20MHz – A-MPR for 6415 MHz = A-MPR for 5955 MHz

: 40MHz – A-MPR for 6405 MHz = A-MPR for 5965 MHz

: 80MHz – A-MPR for 6385 MHz = A-MPR for 5985 MHz

Table 3-12. NS_63 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |
| --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer/Inner RB sets |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 15.0 | ≤ 12.5 | Table 6.2E.2F-4(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz and 6405 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz and 6385 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release, CBWs of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |

#### Proposal 14: Define NS_63 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-12.

### 2.2.5 NS_64 (A-MPR)

For NS_64, Table 3-13 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-13: NS_64-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # ‘20MHz’ | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 25.67 | 20.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5975) | 8.74 | 11.14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5995) | 8.74 | 8.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 32.37 | 25.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (6005) | 7.85 | 8.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (6045) | 7.85 | 8.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5975) | 34.40 | 31.13 | 19.97 | 11.27 | 30.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5995) | 24.77 | 15.64 | 11.23 | 8.91 | 30.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (6055) | 9.69 | 8.65 | 8.93 | 8.91 | 9.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | 33.48 | 32.10 | 31.04 | 20.83 | 15.48 | 11.07 | 31.77 | 30.73 | 30.81 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (6065) | 10.43 | 10.01 | 8.43 | 8.86 | 9.96 | 8.64 | 10.17 | 9.26 | 8.73 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (6415) | 10.43 | 10.01 | 8.43 | 8.86 | 9.96 | 8.64 | 10.17 | 9.26 | 8.73 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' (5995) | 34.37 | 33.10 | 34.28 | 31.06 | 20.18 | 24.48 | 15.30 | 11.37 | 8.55 | 32.00 | 31.74 | 31.21 | 32.25 | 31.40 | 30.68 | 30.66 | 30.71 | 30.42 | 32.95 |
| '100MHz' (6055) | 10.35 | 9.85 | 9.67 | 8.69 | 8.93 | 9.44 | 7.96 | 8.89 | 8.55 | 16.46 | 15.27 | 12.30 | 24.64 | 13.48 | 9.33 | 8.71 | 30.71 | 8.81 | 16.09 |
| '100MHz' (6155) | 10.32 | 9.85 | 9.67 | 8.69 | 8.93 | 9.44 | 7.96 | 8.89 | 8.55 | 10.78 | 10.18 | 9.84 | 10.19 | 10.60 | 9.33 | 8.71 | 8.76 | 8.81 | 10.67 |

Table 3-14 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-14 : NS_64-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz : 5955 40MHz : 5965 60MHz : 5975/5995 80MHz : 5985 100MHz: 5995/6055 |  | 20MHz : 5975/5995 40MHz : 6005/6045 60MHz : 6055 80MHz : 6065/6145 100MHz: 6155 |  |
|  | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 34.4 | 30.42 | 11.14 | 9.66 |

Considering implementation margin and VLP UE, Table 3-15 can be proposed for SL-U NS_64 PSFCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 3-15. NS_64 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤36.0 | ≤32.0 | ≤13.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 and 5995 MHz, 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz, and 100 MHz channels centered at the nearest NR-ARFCN corresponding to 5995 and 6055 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. |  |  |  |  |

#### Proposal 15: Define NS_64 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-15.

### 2.2.6 NS_65 (A-MPR)

For NS_65, Table 3-16 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-16: NS_65-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 8.27 | 7.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 10.01 | 6.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5975) | 11.13 | 8.65 | 7.30 | 7.00 | 9.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | 12.23 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.54 | 9.26 | 9.66 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' (5995) | 13.28 | 9.85 | 9.67 | 8.69 | 7.42 | 9.44 | 7.96 | 7.09 | 6.49 | 12.15 | 10.18 | 10.14 | 10.33 | 10.60 | 9.33 | 8.97 | 9.67 | 8.81 | 11.33 |

Table 3-17 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-17 : NS_65-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 13.28 | 9.96 |

Considering implementation margin and VLP UE, Table 3-18 can be proposed for SL-U NS_65 PSFCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 3-18. NS_65 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 16.0 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |

#### Proposal 16: Define NS_65 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-18.

### 2.2.7 NS_66 (A-MPR)

For NS_66, Table 3-19 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-19: NS_66-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 17.52 | 17.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 14.46 | 17.46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 13.10 | 14.77 | 17.78 | 17.75 | 14.80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 11.71 | 13.05 | 14.88 | 17.84 | 14.85 | 17.84 | 13.04 | 14.85 | 14.88 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 11.33 | 11.70 | 13.08 | 14.82 | 17.84 | 12.93 | 14.75 | 17.72 | 17.72 | 11.67 | 13.01 | 13.04 | 13.01 | 12.92 | 14.82 | 14.82 | 14.85 | 14.76 | 11.69 |

Table 3-20 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-20 : NS_66-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz (Full/Partial) | 40MHz (Full/Partial) | 60MHz (Full/Partial) | 80MHz (Full/Partial) | 100MHz (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 17.84 | 14.88 | 13.10 | 11.71 | 11.33 |

Considering implementation margin and VLP UE, Table 3-21 can be proposed for SL-U NS_66 PSFCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 3-21. NS_66 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤20.5 | ≤17.5 | ≤15.5 | ≤14.5 | ≤14.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### Proposal 17: Define NS_66 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-21.

### 2.2.8 NS_67 (A-MPR)

For NS_67, Table 3-22 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-22: NS_67-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 14.44 | 14.78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.26 | 14.42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 11.13 | 11.66 | 14.74 | 14.71 | 11.68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 12.23 | 10.32 | 11.73 | 14.82 | 11.70 | 14.83 | 10.54 | 11.72 | 11.73 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 11.33 | 9.85 | 10.40 | 11.71 | 14.79 | 10.34 | 11.65 | 14.71 | 14.71 | 10.78 | 10.22 | 10.18 | 10.19 | 10.60 | 11.72 | 11.72 | 11.72 | 11.67 | 10.67 |

Table 3-23 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-23 : NS_67-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz (Full/Partial) | 40MHz (Full/Partial) | 60MHz (Full/Partial) | 80MHz (Full/Partial) | 100MHz (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 14.83 | 11.73 | 11.13 | 12.23 | 11.33 |

Considering implementation margin and VLP UE, Table 3-24 can be proposed for SL-U NS_67 PSFCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 3-24. NS_67 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤17.5 | ≤14.5 | ≤14.0 | ≤14.0 | ≤14.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### Proposal 18: Define NS_67 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-24.

### 2.2.9 NS_68 (A-MPR)

For NS_68, Table 3-25 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-25: NS_68-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 8.74 | 8.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 7.85 | 8.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5975) | 9.69 | 8.65 | 8.93 | 8.91 | 9.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | 10.43 | 10.01 | 8.43 | 8.86 | 9.96 | 8.64 | 10.17 | 9.26 | 8.73 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' (5995) | 10.32 | 9.85 | 9.67 | 8.69 | 8.93 | 9.44 | 7.96 | 8.89 | 8.55 | 10.78 | 10.18 | 9.84 | 10.19 | 10.60 | 9.33 | 8.71 | 8.76 | 8.81 | 10.67 |

Table 3-26 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-26 : NS_68-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz (Full/Partial) | 40MHz (Full/Partial) | 60MHz (Full/Partial) | 80MHz (Full/Partial) | 100MHz (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 8.94 | 9.96 | 10.60 | 10.78 | 10.32 |

Considering implementation margin and VLP UE, Table 3-27 can be proposed for SL-U NS_68 PSFCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 3-27. NS_68 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤ 13.5 | ≤ 13.5 | ≤ 13.5 | ≤ 13.5 | ≤ 13.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### Proposal 19: Define NS_68 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-27.

### 2.2.10 NS_69 (A-MPR)

For NS_69, Table 3-28 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 3-28: NS_69-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 8.27 | 7.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 10.01 | 6.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5985) | 12.23 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.54 | 9.26 | 9.66 |  |  |  |  |  |  |  |  |  |  |

Table 3-29 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 3-29 : NS_69-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |
| --- | --- | --- | --- |
|  | 20MHz (Full/Partial) | 40MHz (Full/Partial) | 80MHz (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 8.27 | 10.01 | 12.23 |

Considering implementation margin and VLP UE, Table 3-30 can be proposed for SL-U NS_69 PSFCH A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 3-30. NS_69 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |
| --- | --- | --- | --- |
|  | 20MHz | 40MHz | 80MHz |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 11.0 | ≤ 12.5 | ≤ 15.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |

#### Proposal 20: Define NS_69 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-30.

## SL-U S-SSB

Based on the simulation assumption and test scenario in [2], simulation results for A-MPR with NS_29, NS_54, NS_59, NS_63, NS_64, NS_65, NS_66, NS_67, NS_68 and NS_69 are shown below.

### 2.3.1 NS_29 (A-MPR)

For NS_29, Table 4-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-1: NS_29-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5239.98) | 8.90 | 7.13 | 5.64 | 6.39 | 5.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5230.02) | 10.90 | 8.84 | 6.45 | 8.74 | 7.31 | 7.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5220) | 9.20 | 7.14 | 7.29 | 6.10 | 6.35 | 7.21 | 6.43 | 7.20 | 8.22 | 7.30 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5210) | 9.81 | 7.53 | 7.31 | 6.14 | 7.08 | 6.21 | 6.34 | 7.19 | 7.50 | 6.02 | 6.26 | 7.21 | 8.39 | 7.37 | 8.34 | 7.08 | 7.58 | 6.81 |  |  |

Table 4-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-2 : NS_29-S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 8.90 | 5.64 | 10.90 | 7.31 | 9.20 | 7.30 | 9.81 | 7.53 |

Considering implementation margin, Table 4-3 shows proposal for SL-U NS_29 S-SSB A-MPR.

Table 4-3. NS_29 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤ 11.5 | ≤ 8.0 | ≤12.0 | ≤10.0 | ≤12.0 | ≤10.0 | ≤12.5 | ≤10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |

#### Proposal 21: Define NS_29 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-3.

### 2.3.2 NS_54 (A-MPR)

For NS_54, Table 4-4 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-4: NS_54-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 8.90 | 7.13 | 5.64 | 6.39 | 5.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 10.90 | 8.84 | 6.45 | 8.74 | 7.31 | 5.52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5975) | 9.20 | 7.14 | 7.29 | 6.10 | 6.35 | 5.52 | 6.43 | 4.95 | 8.22 | 7.30 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 9.81 | 7.53 | 7.31 | 6.14 | 7.08 | 6.27 | 6.34 | 5.74 | 7.50 | 6.02 | 6.26 | 5.04 | 8.39 | 7.26 | 8.34 | 7.00 | 7.58 | 7.17 |  |  |

Table 4-5 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-5 : NS_54-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation / (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 10.90 | 7.53 | 7.50 | 6.02 |

Considering implementation margin, Table 4-6 shows proposal for SL-U NS_54 S-SSB A-MPR.

Table 4-6. NS_54 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |
| --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer/Inner RB sets |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |  |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 | Table 6.2E.2F-5(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |

#### Proposal 22: Define NS_54 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-6.

### 2.3.3 NS_59 (A-MPR)

For NS_59, Table 4-7 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-7: NS_59-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (7115) | 8.90 | 7.13 | 9.15 | 6.39 | 6.18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 10.90 | 8.84 | 6.45 | 8.74 | 7.31 | 9.14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (7095) | 9.20 | 7.14 | 7.29 | 6.10 | 6.35 | 6.20 | 6.43 | 6.19 | 8.22 | 7.30 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 9.81 | 7.53 | 7.31 | 6.14 | 7.08 | 6.27 | 6.34 | 6.16 | 7.50 | 6.02 | 6.26 | 6.19 | 8.39 | 7.26 | 8.34 | 7.00 | 7.57 | 6.85 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' (7075) | 10.69 | 7.18 | 7.08 | 5.65 | 7.87 | 5.93 | 7.09 | 6.21 | 6.45 | 6.10 | 7.26 | 5.03 | 7.17 | 4.89 | 6.58 | 6.27 | 6.36 | 6.08 | 9.82 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' (7095) | 8.41 | 7.26 | 8.46 | 7.25 | 8.97 | 6.85 | 8.76 | 6.54 | 8.36 | 6.89 | 7.26 | 6.92 | 7.61 | 6.84 | 7.75 | 6.46 | 8.99 | 7.26 |  |  |

Table 4-8 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-8 : NS_59-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation / (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 10.90 | 9.15 | 7.75 | 6.46 |

Considering implementation margin, Table 4-9 shows proposal for SL-U NS_59 S-SSB A-MPR.

Table 4-9. NS_59 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 11.5 | ≤ 10.5 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |

#### Proposal 23: Define NS_59 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-9.

### 2.3.4 NS_63 (A-MPR)

For NS_63, Table 4-10 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-10: NS_63-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 8.90 | 7.13 | 5.64 | 6.39 | 5.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 10.90 | 8.84 | 6.45 | 8.74 | 7.31 | 5.52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 10.16 | 7.34 | 7.49 | 6.24 | 7.19 | 6.26 | 6.44 | 5.61 | 7.61 | 5.97 | 6.45 | 4.96 | 8.47 | 7.28 | 8.23 | 7.07 | 7.62 | 6.92 |  |  |

Table 4-11 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-11 : NS_63-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation / (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 10.90 | 7.34 | 7.61 | 5.97 |

Considering implementation margin, Table 4-12 shows proposal for SL-U NS_63 S-SSB A-MPR.

Table 4-12. NS_63 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |

#### Proposal 24: Define NS_63 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-12.

### 2.3.5 NS_64 (A-MPR)

For NS_64, Table 4-13 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-13: NS_64-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 19.86 | 24.46 | 27.75 | 11.93 | 24.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5975) | 8.79 | 9.20 | 13.16 | 7.21 | 10.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5995) | 8.88 | 9.19 | 13.06 | 7.22 | 10.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 32.62 | 31.97 | 29.55 | 20.84 | 24.92 | 27.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (6005) | 9.12 | 8.11 | 10.13 | 8.80 | 9.20 | 13.20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (6045) | 9.10 | 8.11 | 10.22 | 8.73 | 9.15 | 13.05 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5975) | 31.91 | 29.55 | 28.14 | 25.74 | 16.86 | 25.48 | 7.22 | 10.17 | 28.63 | 25.76 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5995) | 13.06 | 10.69 | 11.39 | 9.94 | 7.11 | 10.17 | 7.22 | 10.26 | 28.36 | 26.68 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (6055) | 7.98 | 7.11 | 7.17 | 7.10 | 7.24 | 10.13 | 7.25 | 10.13 | 8.31 | 7.21 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 31.43 | 27.72 | 31.07 | 26.57 | 28.14 | 25.17 | 19.46 | 25.15 | 12.71 | 9.30 | 7.15 | 10.18 | 28.31 | 26.27 | 27.95 | 26.88 | 27.89 | 26.00 |  |  |
| '80MHz' (6055) | 7.71 | 6.79 | 7.22 | 6.25 | 7.10 | 7.21 | 7.18 | 10.18 | 7.31 | 7.10 | 7.12 | 10.11 | 8.73 | 7.37 | 8.16 | 7.24 | 7.22 | 7.27 |  |  |
| '80MHz' (6145) | 7.46 | 6.87 | 7.45 | 6.19 | 7.16 | 7.19 | 7.13 | 10.17 | 7.53 | 7.21 | 7.16 | 10.17 | 8.45 | 7.34 | 8.24 | 7.16 | 6.97 | 7.27 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' (5995) | 32.11 | 27.94 | 30.84 | 26.80 | 31.55 | 26.23 | 28.52 | 25.30 | 16.69 | 26.04 | 21.59 | 9.74 | 13.85 | 8.53 | 7.20 | 10.17 | 7.26 | 10.14 | 28.51 | 26.97 |
| '100MHz' (6055) | 14.05 | 8.77 | 8.15 | 6.39 | 7.85 | 6.25 | 7.08 | 7.21 | 7.22 | 10.15 | 7.15 | 5.56 | 7.19 | 7.17 | 7.25 | 10.23 | 7.22 | 10.08 | 11.32 | 9.75 |
| '100MHz' (6115) | 7.56 | 6.68 | 7.06 | 5.66 | 7.80 | 6.29 | 7.15 | 7.12 | 7.24 | 10.16 | 7.36 | 5.46 | 7.21 | 7.14 | 7.17 | 10.14 | 7.23 | 10.16 | 8.62 | 7.50 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' (5995) | 28.78 | 25.66 | 28.27 | 25.81 | 29.44 | 25.21 | 29.49 | 23.01 | 27.41 | 25.41 | 27.65 | 25.39 | 28.29 | 25.62 | 28.41 | 26.21 | 29.68 | 24.07 |  |  |
| '100MHz' (6055) | 8.53 | 7.85 | 11.85 | 9.28 | 11.60 | 9.09 | 8.76 | 7.34 | 8.14 | 7.30 | 7.05 | 8.78 | 27.86 | 25.10 | 7.72 | 7.19 | 10.73 | 8.83 |  |  |
| '100MHz' (6155) | 8.68 | 7.24 | 8.43 | 7.38 | 8.63 | 6.94 | 8.74 | 6.58 | 8.23 | 7.17 | 7.00 | 7.23 | 7.09 | 7.19 | 7.75 | 7.24 | 9.06 | 7.25 |  |  |

Table 4-14 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-14 : NS_64-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation /Centre frequency of CBW (MHz)/ (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz : 5955 40MHz : 5965 60MHz : 5975/5995 80MHz : 5985 100MHz: 5995/6055 |  |  |  | 20MHz : 5975/5995 40MHz : 6005/6045 60MHz : 6055 80MHz : 6065/6145 100MHz: 6115 |  |  |  |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 32.62 | 29.55 | 21.59 | 10.26 | 9.20 | 13.16 | 7.53 | 10.17 |

Considering implementation margin and VLP UE, Table 4-15 can be proposed for SL-U NS_64 S-SSB A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 4-15. NS_64 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | ≤ 35.0 | ≤ 32.0 | ≤ 24.0 | ≤ 13.0 | ≤ 11.5 | ≤ 15.5 | ≤ 10.0 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 and 5995 MHz, 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz and 100 MHz channels centered at the nearest NR-ARFCN corresponding to 5995 and 6055 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. |  |  |  |  |  |  |  |  |

#### Proposal 25: Define NS_64 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-15.

### 2.3.6 NS_65 (A-MPR)

For NS_65, Table 4-16 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-16: NS_65-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 8.90 | 7.13 | 5.64 | 6.39 | 5.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 10.90 | 8.84 | 6.45 | 8.74 | 7.31 | 5.52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5975) | 9.20 | 7.14 | 7.29 | 6.10 | 6.35 | 5.52 | 6.43 | 4.95 | 8.22 | 7.30 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 9.81 | 7.53 | 7.31 | 6.14 | 7.08 | 6.27 | 6.34 | 5.74 | 7.50 | 6.02 | 6.26 | 5.04 | 8.39 | 7.26 | 8.34 | 7.00 | 7.58 | 7.17 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' (5995) | 10.69 | 7.18 | 7.08 | 5.65 | 7.93 | 5.93 | 7.09 | 5.98 | 6.36 | 5.51 | 7.26 | 5.03 | 7.17 | 4.89 | 6.58 | 5.02 | 6.09 | 4.84 | 9.82 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' (5995) | 8.41 | 7.26 | 8.43 | 7.39 | 8.97 | 6.85 | 8.88 | 6.56 | 8.17 | 7.18 | 7.16 | 6.87 | 7.61 | 6.88 | 7.82 | 6.47 | 8.99 | 7.08 |  |  |

Table 4-17 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-17 : NS_65-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation / (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 10.90 | 7.53 | 7.50 | 6.02 |

Considering implementation margin and VLP UE, Table 4-18 can be proposed for SL-U NS_65 S-SSB A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 4-18. NS_65 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |

#### Proposal 26: Define NS_65 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-18.

### 2.3.7 NS_66 (A-MPR)

For NS_66, Table 4-19 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-19: NS_66-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (7115) | 15.81 | 18.17 | 22.10 | 16.26 | 19.13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 12.75 | 15.34 | 19.15 | 15.78 | 18.33 | 22.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (7095) | 11.45 | 14.53 | 13.24 | 16.22 | 16.19 | 19.14 | 16.22 | 19.13 | 13.21 | 16.27 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 10.20 | 13.16 | 11.46 | 14.38 | 13.21 | 16.11 | 16.31 | 19.21 | 13.22 | 16.15 | 16.26 | 19.23 | 11.43 | 14.40 | 13.32 | 16.20 | 13.30 | 16.12 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' (7075) | 9.48 | 12.29 | 10.12 | 13.26 | 11.60 | 14.51 | 13.24 | 16.21 | 16.32 | 19.14 | 11.42 | 14.49 | 13.33 | 16.22 | 16.20 | 19.28 | 16.15 | 19.14 | 10.16 | 13.19 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' (7095) | 11.48 | 14.47 | 11.42 | 14.35 | 11.49 | 14.53 | 11.40 | 14.46 | 13.21 | 16.16 | 13.24 | 16.19 | 13.19 | 16.23 | 13.24 | 16.21 | 10.17 | 13.29 |  |  |

Table 4-20 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-20 : NS_66-S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 18.17 | 22.10 | 18.33 | 22.11 | 16.22 | 19.14 | 16.31 | 19.23 | 16.32 | 19.28 |

Considering implementation margin and VLP UE, Table 4-21 can be proposed for SL-U NS_66 S-SSB A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 4-21. NS_66 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤ 21.0 | ≤ 24.5 | ≤ 21.0 | ≤24.5 | ≤19.0 | ≤21.5 | ≤19.0 | ≤21.5 | ≤19.0 | ≤21.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### Proposal 27: Define NS_66 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-21.

### 2.3.8 NS_67 (A-MPR)

For NS_67, Table 4-22 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-22: NS_67-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (7115) | 12.70 | 15.23 | 19.13 | 13.26 | 16.18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 9.74 | 12.25 | 16.14 | 12.65 | 15.28 | 19.13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (7095) | 9.20 | 11.46 | 10.24 | 13.16 | 13.17 | 16.18 | 13.22 | 16.18 | 10.20 | 13.27 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 9.83 | 10.17 | 8.52 | 11.43 | 10.11 | 13.16 | 13.30 | 16.15 | 10.16 | 13.24 | 13.18 | 16.17 | 8.61 | 11.44 | 10.11 | 13.24 | 10.19 | 13.24 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' (7075) | 10.65 | 9.20 | 7.52 | 10.10 | 8.52 | 11.45 | 10.21 | 13.17 | 13.28 | 16.30 | 8.49 | 11.38 | 10.13 | 13.25 | 13.16 | 16.20 | 13.23 | 16.22 | 10.05 | 10.21 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' (7095) | 8.50 | 11.34 | 8.52 | 11.44 | 8.68 | 11.48 | 8.70 | 11.40 | 10.23 | 13.26 | 10.17 | 13.15 | 10.19 | 13.14 | 10.21 | 13.26 | 9.07 | 10.26 |  |  |

Table 4-23 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-23 : NS_67-S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 15.23 | 19.13 | 15.28 | 19.13 | 13.22 | 16.18 | 13.30 | 16.17 | 13.28 | 16.30 |

Considering implementation margin and VLP UE, Table 4-24 can be proposed for SL-U NS_67 S-SSB A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 4-24. NS_67 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤ 18.5 | ≤ 21.5 | ≤18.0 | ≤21.5 | ≤16.0 | ≤18.5 | ≤16.0 | ≤18.5 | ≤16.0 | ≤18.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### Proposal 28: Define NS_67 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-24.

### 2.3.9 NS_68 (A-MPR)

For NS_68, Table 4-25 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-25: NS_68-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 8.90 | 9.08 | 13.05 | 7.17 | 10.13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 10.90 | 8.84 | 10.15 | 8.74 | 9.17 | 13.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5975) | 9.20 | 7.14 | 7.29 | 7.11 | 7.22 | 10.14 | 7.22 | 10.14 | 8.22 | 7.30 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 9.81 | 7.53 | 7.31 | 6.24 | 7.08 | 7.21 | 7.20 | 10.26 | 7.50 | 7.12 | 7.22 | 10.13 | 8.61 | 7.26 | 8.37 | 7.12 | 7.53 | 7.16 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' (5995) | 10.69 | 7.18 | 7.16 | 5.59 | 7.88 | 6.35 | 7.08 | 7.20 | 7.19 | 10.15 | 7.22 | 5.48 | 7.19 | 7.18 | 7.23 | 10.18 | 7.16 | 10.14 | 9.75 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' (5995) | 8.61 | 7.33 | 8.45 | 7.37 | 8.66 | 7.07 | 8.65 | 6.55 | 8.24 | 7.10 | 7.15 | 7.12 | 7.70 | 7.13 | 7.94 | 7.18 | 9.17 | 7.25 |  |  |

Table 4-26 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-26 : NS_68-S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 9.08 | 13.05 | 10.90 | 13.17 | 9.20 | 10.14 | 9.81 | 10.26 | 10.69 | 10.18 |

Considering implementation margin and VLP UE, Table 4-27 can be proposed for SL-U NS_68 S-SSB A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 4-27. NS_68 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤ 13.5 | ≤ 15.5 | ≤ 13.5 | ≤ 15.5 | ≤12.0 | ≤12.5 | ≤12.0 | ≤12.5 | ≤12.0 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### Proposal 29: Define NS_68 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-27.

### 2.3.10 NS_69 (A-MPR)

For NS_69, Table 4-28 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 4-28: NS_69-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5955) | 8.90 | 7.13 | 5.64 | 6.39 | 5.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5965) | 8.90 | 7.13 | 5.64 | 6.39 | 5.50 | 5.52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5985) | 10.16 | 7.34 | 7.49 | 6.24 | 7.19 | 6.26 | 6.44 | 5.61 | 7.61 | 5.97 | 6.45 | 4.96 | 8.47 | 7.28 | 8.23 | 7.07 | 7.62 | 6.81 |  |  |

Table 4-29 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 4-29 : NS_69-S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 80MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 8.90 | 5.64 | 8.90 | 5.64 | 10.16 | 7.34 |

Considering implementation margin and VLP UE, Table 4-30 can be proposed for SL-U NS_69 S-SSB A-MPR.

- maximum (6dB, simulated A-MPR + implementation margin)

Table 4-30. NS_69 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 80MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | ≤ 11.5 | ≤ 8.0 | ≤ 11.5 | ≤ 8.0 | ≤12.5 | ≤10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |

#### Proposal 30: Define NS_69 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-30.

# Conclusions

For the remaining NS values (NS_29, NS_54, NS_59, NS_63, NS_64, NS_65, NS_66, NS_67, NS_68 and NS_69), we provided our view and simulation results. Based on these, the followings are proposed.

#### Proposal 1~10: Define NS_29/54/59/63/64/65/66/67/68/69 PSSCH/PSCCH A-MPR for SL-U UE power class 5 as shown in Table 2-3/6/9/12/15/18/21/24/27/30 respectively.

#### Proposal 11~20: Define NS_29/54/59/63/64/65/66/67/68/69 PSFCH A-MPR for SL-U UE power class 5 as shown in Table 3-3/6/9/12/15/18/21/24/27/30 respectively.

#### Proposal 21~30: Define NS_29/54/59/63/64/65/66/67/68/69 S-SSB A-MPR for SL-U UE power class 5 as shown in Table 4-3/6/9/12/15/18/21/24/27/30 respectively.

## A-MPR for SL-U PSSCH/PSCCH

Table 2-3. NS_29 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 7.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 7.5 | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 7.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 3: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |  |  |

Table 2-6. NS_54 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 2.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 3.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 6.0 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.2E.2F-3 apply. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

Table 2-9. NS_59 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration3 |  | Inner RB set configuration3 |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 7.0 | ≤ 4.0 | ≤ 7.0 |
|  | 16 QAM | ≤ 4.0 | ≤ 7.5 | ≤ 4.0 | ≤ 7.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.2E.2F-3 apply. |  |  |  |  |  |

Table 2-12. NS_63 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 | ≤ 2.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 3.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 6415 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 and 6405 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 and 6385 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |  |  |

Table 2-15. NS_64 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation3 |  |  |  | RB Allocation4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 14.0 | ≤ 15.0 | ≤ 9.0 | ≤ 11.5 | ≤ 9.0 | ≤ 11.5 | ≤ 8.5 | ≤ 11.5 |
|  | 16 QAM | ≤ 14.0 | ≤ 15.0 | ≤ 9.0 | ≤ 11.5 | ≤ 9.0 | ≤ 11.5 | ≤ 8.5 | ≤ 11.5 |
|  | 64 QAM | ≤ 14.0 | ≤ 15.0 | ≤ 9.0 | ≤ 11.5 | ≤ 9.0 | ≤ 11.5 | ≤ 8.5 | ≤ 11.5 |
|  | 256 QAM | ≤ 15.0 | ≤ 15.0 | ≤ 9.0 | ≤ 11.5 | ≤ 9.0 | ≤ 11.5 | ≤ 8.5 | ≤ 11.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz, and 100 MHz channels centered at the nearest NR-ARFCN corresponding to 5995 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. |  |  |  |  |  |  |  |  |  |

Table 2-18. NS_65 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration3 |  | Inner RB set configuration3 |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.5 | ≤ 7.0 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.2E.2F-3 apply. |  |  |  |  |  |

Table 2-21. NS_66 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 16.5 | ≤ 19.0 | ≤ 13.5 | ≤ 16.0 | ≤ 11.5 | ≤ 14.0 | ≤ 10.0 | ≤ 12.5 | ≤ 9.0 | ≤ 11.5 |
|  | 16 QAM | ≤ 16.5 | ≤ 19.0 | ≤ 13.5 | ≤ 16.0 | ≤ 11.5 | ≤ 14.0 | ≤ 10.0 | ≤ 12.5 | ≤ 9.0 | ≤ 11.5 |
|  | 64 QAM | ≤ 16.5 | ≤ 19.0 | ≤ 13.5 | ≤ 16.0 | ≤ 11.5 | ≤ 14.0 | ≤ 10.0 | ≤ 12.5 | ≤ 9.0 | ≤ 11.5 |
|  | 256 QAM | ≤ 16.5 | ≤ 19.0 | ≤ 13.5 | ≤ 16.0 | ≤ 11.5 | ≤ 14.0 | ≤ 10.0 | ≤ 12.5 | ≤ 9.0 | ≤ 11.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

Table 2-24. NS_67 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 13.5 | ≤ 16.0 | ≤ 10.5 | ≤ 13.5 | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 9.0 |
|  | 16 QAM | ≤ 13.5 | ≤ 16.0 | ≤ 10.5 | ≤ 13.5 | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 9.0 |
|  | 64 QAM | ≤ 13.5 | ≤ 16.0 | ≤ 10.5 | ≤ 13.5 | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 9.0 |
|  | 256 QAM | ≤ 13.5 | ≤ 16.0 | ≤ 10.5 | ≤ 13.5 | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

Table 2-27. NS_68 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.0 | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.0 | ≤ 6.0 | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.0 | ≤ 6.0 | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.0 | ≤ 6.0 | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

Table 2-30. NS_69 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 80MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 3: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |

## A-MPR for SL-U PSFCH

Table 3-3. NS_29 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz (Full/Partial) | 40MHz (Full/Partial) | 60MHz (Full/Partial) | 80MHz (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 11.0 | ≤12.5 | ≤13.5 | ≤15.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |

Table 3-6. NS_54 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |
| --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer/Inner RB sets |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 15.0 | ≤ 12.5 | Table 6.2E.2F-4(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |

Table 3-9. NS_59 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 16.0 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |

Table 3-12. NS_63 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |
| --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer/Inner RB sets |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 15.0 | ≤ 12.5 | Table 6.2E.2F-4(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz and 6405 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz and 6385 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release, CBWs of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |

Table 3-15. NS_64 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤36.0 | ≤32.0 | ≤13.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 and 5995 MHz, 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz, and 100 MHz channels centered at the nearest NR-ARFCN corresponding to 5995 and 6055 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. |  |  |  |  |

Table 3-18. NS_65 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 16.0 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |

Table 3-21. NS_66 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤20.5 | ≤17.5 | ≤15.5 | ≤14.5 | ≤14.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 3-24. NS_67 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤17.5 | ≤14.5 | ≤14.0 | ≤14.0 | ≤14.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 3-27. NS_68 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤ 13.5 | ≤ 13.5 | ≤ 13.5 | ≤ 13.5 | ≤ 13.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 3-30. NS_69 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |
| --- | --- | --- | --- |
|  | 20MHz | 40MHz | 80MHz |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 11.0 | ≤ 12.5 | ≤ 15.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |

## A-MPR for SL-U S-SSB

Table 4-3. NS_29 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤ 11.5 | ≤ 8.0 | ≤12.0 | ≤10.0 | ≤12.0 | ≤10.0 | ≤12.5 | ≤10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |

Table 4-6. NS_54 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |
| --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer/Inner RB sets |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |  |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 | Table 6.2E.2F-5(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |

Table 4-9. NS_59 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 11.5 | ≤ 10.5 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |

Table 4-12. NS_63 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |

Table 4-15. NS_64 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | ≤ 35.0 | ≤ 32.0 | ≤ 24.0 | ≤ 13.0 | ≤ 11.5 | ≤ 15.5 | ≤ 10.0 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 and 5995 MHz, 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz and 100 MHz channels centered at the nearest NR-ARFCN corresponding to 5995 and 6055 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. |  |  |  |  |  |  |  |  |

Table 4-18. NS_65 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |

Table 4-21. NS_66 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤ 21.0 | ≤ 24.5 | ≤ 21.0 | ≤24.5 | ≤19.0 | ≤21.5 | ≤19.0 | ≤21.5 | ≤19.0 | ≤21.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 4-24. NS_67 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤ 18.5 | ≤ 21.5 | ≤18.0 | ≤21.5 | ≤16.0 | ≤18.5 | ≤16.0 | ≤18.5 | ≤16.0 | ≤18.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 4-27. NS_68 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤ 13.5 | ≤ 15.5 | ≤ 13.5 | ≤ 15.5 | ≤12.0 | ≤12.5 | ≤12.0 | ≤12.5 | ≤12.0 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 4-30. NS_69 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 80MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | ≤ 11.5 | ≤ 8.0 | ≤ 11.5 | ≤ 8.0 | ≤12.5 | ≤10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |

# References

[1] TR 38.786 V18.0.0

[2] R4-2315542, On Tx requirements for NR sidelink evolution, LG Electronics

[3] R4-2403868, WF on NR sidelink UE RF requirements, LG Electronics