3GPP TSG-RAN WG4 Meeting #110bis	R4- 2404879

Changsha, China, 15 April – 19 April, 2024

Source: 	Huawei, HiSilicon

Title:	Discussion on FDD HPUE MSD

Agenda Item:	5.18.2

Document for:	Approval

# 1	Introduction

In RAN4#110, a WF [1] was agreed on PC2 HPUE MSD, where a number of open issues were listed regarding Intra-band/Inter-band REFSENS tables, evaluation of 2Tx MSD, etc. In the sequel, we share our views.

# 2	Discussion

### 2.1 MSD for FDD intra-band CA

According to the latest WID [2], the following FDD intra-band CA combos are requested, which have PC2 single UL and intra-band contiguous/non-contiguous CA in the DL.

| NR CA configuration | Uplink NR CA configuration | contact name, company | Contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | n25A | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Deutsche Telekom, Ericsson, Nokia, Nokia Shainghai Bell | new |
| CA_n66(2A) | n66A | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Deutsche Telekom, Ericsson, Nokia, Nokia Shainghai Bell, DISH Network | new |
| CA_n71(2A) | n71A | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Deutsche Telekom, Ericsson, Nokia, Nokia Shainghai Bell | new |
| CA_n71B | n71A | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Deutsche Telekom, Ericsson, Nokia, Nokia Shainghai Bell | new |

In the current TS 38.101-1 clause 7.3A.2.1, the allowed REFSENS relaxation for intra-band contiguous CA is defined for the case when both PCC and SCC are active. When only one UL carrier is configured/scheduled, the following statement is specified.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency.

Proposal 1: Clarify for intra-band contiguous CA whether “the downlink PCC carrier center frequency shall be configured close to uplink operating band than any of the downlink SCC center frequency” is a scheduling restriction for the network.

Proposal 2: Clarify for intra-band contiguous CA the REFSENS requirements for SCC, e.g. whether PC2 RSD defined in Table 7.3.2-1c/1d are applicable.

In the current TS 38.101-1 clause 7.3A.2.2, the allowed REFSENS relaxation for intra-band non-contiguous CA with one uplink is defined as follows:

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands. (Source: TS 38.101-1)

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 |  |  |  |  |  |  |

Proposal 3: Clarify for intra-band non-contiguous CA the relative position in frequency for PCC and SCC, e.g. which NOTE applies in general, NOTE 4 or NOTE 7.

### 2.2 MSD for PC2 FDD with inter-band CA

The following proposals are captured in the WF [1]:

WF3 on inter-band CA REFSENS tables:

For inter-band CA, capture the 1Tx and the 2Tx FDD PC2 REFSENS requirements into a single table for:

- MSD due to UL harmonics,

- MSD due to Rx harmonic mixing

- MSD due to cross-band isolation,

The PC2 FDD table number uses the PC3 Table number and suffix “a”.

The 1Tx vs 2Tx MSD test points are distinguished using footnotes.

For CA_n3-n78 MSD due to Harmonic interference UL2/DL1, merge Table 7.3A.4-2a and Table 7.3A.4-2b into Table 7.3A.4-2a and adopt footnotes “X” and “Y” to distinguish 1Tx vs 2Tx MSD.

WF4: For PC2 FDD, interested companies are encouraged to bring guidelines to simplify 2Tx MSD analyses.

In TR 38.850 v1.2.0, a number of inter-band CA combos with PC2 FDD UL are added, such as CA_n25A-n77A, CA_n41A-n71A, CA_n66A-n77A and CA_n71A-n77A. They are combinations of LB/MB with HB. And the HB DL suffers harmonic interference from the LB/MB UL.

The captured MSD analysis for 1Tx PC2 UL assumes that the harmonic interference power increases by 3dB*harmonic order. For example, for CA_n41-n71:

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n71 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |

For PC3 MSD we have N+IPC3. For PC2 MSD we have N+IPC2. So, for PC2 compared to PC3, I increases by harmonic order * 3 dB or 12 dB. For our other PC2 and PC1.5 MSD analysis we have been using the following approach:

As a result, the PC2 MSD is increased significantly, e.g. from 10.8dB to 22.4dB.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1 for UE not supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n71 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 22.4 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 8.5 | NOTE 4 | UL4/DL1 direct-hit |

However, we think that this assumption on harmonic interference power is questionable. It might be true for the passive components in the RF front-end. But based on our MSD analysis [3] as well as the contribution from Apple [4] on CA_n3A-n78A, the harmonic interference is dominated by the harmonic at the PA output. Typically, a fixed harmonic suppression ratio is assumed for the PA, regardless of the power class. As a result, the PA harmonic output power is assumed to increase by only 3dB when the power class is upgraded from PC3 to PC2.

For CA_n3A-n78A, it can be seen that the specified MSD for UL2/DL1 is increased from 24dB to 27.1dB only.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1 for UE not supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 16.6 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Around 3dB MSD increase can also be seen for some PC2 TDD UL, which causes harmonic mixing of UL2/DL3.

Table 7.3A.4-4a: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n30 | 10 | 15 | 12 (RBstart=0) | 5 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 25 (RBstart=0) | 10 | 10.6 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 4.4 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 8.8 | NOTE 3 | UL2/DL3 |

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n30 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 24 (RBstart=0) | 10 | 8.0 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 0.9 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 6.3 | NOTE 2 | UL2/DL3 |

Proposal 4: The assumption that the harmonic interference power is increased by 3dB*harmonic order may result in over-estimated MSD. Consider adjust the assumption and re-evaluate the MSD caused by 1Tx PC2 FDD UL.

For the 2Tx PC2 FDD UL case, we have submitted a contribution for CA_n8-n41 in this meeting. So far, we have not found a simplified model for 2Tx PC2, hence the conventional method for MSD analysis is used, which is based on RF components parameters.

Proposal 5: Before a simplified model for 2Tx PC2 MSD is found, use the conventional MSD analysis method based on RF components’ isolation and linearity parameters.

# 3	Conclusion

In this paper, we have shared our views on PC2 FDD MSD.

Proposal 1: Clarify for intra-band contiguous CA whether “the downlink PCC carrier center frequency shall be configured close to uplink operating band than any of the downlink SCC center frequency” is a scheduling restriction for the network.

Proposal 2: Clarify for intra-band contiguous CA the REFSENS requirements for SCC, e.g. whether PC2 RSD defined in Table 7.3.2-1c/1d are applicable.

Proposal 3: Clarify for intra-band non-contiguous CA the relative position in frequency for PCC and SCC, e.g. which NOTE applies in general, NOTE 4 or NOTE 7.

Proposal 4: The assumption that the harmonic interference power is increased by 3dB*harmonic order may result in over-estimated MSD. Consider adjust the assumption and re-evaluate the MSD caused by 1Tx PC2 FDD UL.

Proposal 5: Before a simplified model for 2Tx PC2 MSD is found, use the conventional MSD analysis method based on RF components’ isolation and linearity parameters.

# References

- R4-2403628 WF on PC2 HPUE MSD, Skyworks, Huawei, Qualcomm, RAN4#110

- RP-240123 Revised WID: Rel-18 High power UE (power class 2) for a single FR1 NR FDD band in UL of NR intra-band and inter-band CA/DC combinations with y bands downlink (y=1,2,3,4,5,6) and x bands uplink (x=1), China Unicom, RAN#103

- R4-2302731 MSD for CA_n3-n78 with PC2 n3 UL, Huawei, HiSilicon, RAN4#106

- R4-2215660 MSD analysis for CA_n3-n78 with PC2 n3 UL, Apple, RAN4#104bis-e