3GPP TSG-RAN WG4 Meeting#109					R4-2404900

15th Apr, 2024 – 19th Apr, 2024

Source: 	Qualcomm Incorporated

Title: 	PDSCH demodulation performance with R18 DMRS: Simulation results

Agenda item:	 6.19.4

Document for:	Discussion

# 1 Introduction

In this paper we submit R18 DMRS PDSCH demodulation performance simulation results for alignment purposes.

# 2 PDSCH Demodulation Requirements

## 2.1 Rank4 requirements

The following results are obtained with eType1PTI = 53, i.e., number of CDM groups without data = 1, and PDSCH DMRS ports being {1008,1009,1010, 1011}.

## 2.1.1 TDD

With settings in Sub-topic 2-3 of [1], the following table has the simulation results:

| DMRS | MCS (Table1) | Channel | SNR requirement (dB) |
| --- | --- | --- | --- |
| R15 | 13 | TDLA30-10 | 12.8 |
| R18 | 13 | TDLA30-10 | 13.3 |

## 2.1.2 FDD

With settings in Sub-topic 2-3 of [1], the following table has the simulation results:

| DMRS | MCS (Table1) | Channel | SNR requirement (dB) |
| --- | --- | --- | --- |
| R15 | 13 | TDLA30-10 | 12.6 |
| R18 | 13 | TDLA30-10 | 13.2 |

Observation: For Rank4 cases, R18 DMRS needs higher SNR than that for R15 DMRS.

Proposal1: RAN4 to use simulation results for rank4 PDSCH demod. requirements.

## 2.2 Rank2 requirements

The following results are obtained with eType1PTI = 10, i.e., number of CDM groups without data = 1, and PDSCH DMRS ports being {1008,1009}.

## 2.1.1 TDD

With settings in Sub-topic 2-3 of [1], the following table has the simulation results:

| DMRS | MCS (Table1) | Channel | SNR requirement (dB) |
| --- | --- | --- | --- |
| R15 | 19 | TDLA30-10 | 16.9 |
| R18 | 19 | TDLA30-10 | 16.9 |

## 2.1.2 FDD

With settings in Sub-topic 2-3 of [1], the following table has the simulation results:

| DMRS | MCS (Table1) | Channel | SNR requirement (dB) |
| --- | --- | --- | --- |
| R15 | 19 | TDLA30-10 | 16.8 |
| R18 | 19 | TDLA30-10 | 16.8 |

Observation: In rank2 cases, there is no performance difference noticed between R15 DMRS and R18 DMRS.

From above observation, requirements of Test 2-1 of Chapter 5.2.2.1.1, 5.2.2.2.1 can be extended to Rank2 R18 cases too.

Proposal2: For Rank2 cases, RAN4 to reuse requirements of Test 2-1 of Clause 5.2.2.1.1, 5.2.2.2.1.

Proposal3: RAN4 to discuss applicability rules for rank2 requirements.

# Conclusion

- Proposal1: For Rank4 cases, RAN4 to use new simulation results for R18 DMRS rank4 PDSCH demod. requirements.

- Proposal2: For Rank2 cases, RAN4 to reuse requirements of Test 2-1 of Clause 5.2.2.1.1, 5.2.2.2.1.

- Proposal3: RAN4 to discuss applicability rules for rank2 requirements.

- Simulation results for rank2, rank4 cases are submitted.

# References

[1] WF R4-2402868, “WF for UE 8Rx performance requirements”, 3GPP TSG RAN WG4 meeting, #110, Feb. 26 – Mar. 1, 2024, Athens, Greece.