3GPP TSG-RAN4 Meeting #110bis	R4-2404941

# Changsha, China,  April 15th - April 19th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR |  | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5 on top of the running CR R4-2401116: Tx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| 7 |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | This draft CR further updates the endorsed running CR R4-2401116 for Tx requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of a generic name (NTN VSAT) which would indicate any UE classe/type to simplify some wording in sub-clauses. Introduction of max TRP requirement, needed for OoBE. Simplification of the SEM Addition of FCC 25.209 which was missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN Ka-band won’t be correctly supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

# 1	Scope

The present document establishes the minimum RF and performance requirements for NR User Equipment (UE) supporting satellite access operation.

The Mobile VSAT communicating with non-GSO is not considered in this release.

<End of the change>

<Start of the change>

# 2	References

……

[15]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE.

[16]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[17]	ITU-R Recommendation SM.329-12, "Unwanted emissions in the spurious domain"

[18]	EN 303 978, Satellite Earth Stations and Systems (SES);Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP)  transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.2, 2016-10

[19]	EN 301 459, Satellite Earth Stations and Systems (SES);Harmonised Standard for Satellite Interactive Terminals (SIT) and Satellite User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.1, 2016-05

<End of the change>

<Start of the change>

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Co-polarized transmission: when the DUT transmission antenna polarization is aligned with test antenna polarization.

Cross-polarized transmission: when the DUT transmission antenna polarization is aligned with the tangent of the test antenna polarization.

Emissions disables state: Radio state in which the ESOMP is not emitting (e.g. before system monitoring pass, before the control channel is received, when a failure is detected, when an ESOMP is commanded to disable, and

when the ESOMP is in a location requiring cessation of emissions).

Enhanced channel raster: channel raster with a 10 kHz granularity in bands with a 100 kHz channel raster.

…

Non-terrestrial networks: Networks, or segments of networks, using an airborne or space-borne vehicle to embark a transmission equipment relay node or base station.

NTN VSAT: a UE operating in FR2-NTN which could be a Fixed VSAT or a Mobile VSAT.

Plane perpendicular to the GSO arc: The plane that is perpendicular to the “plane tangent to the GSO arc,” as defined below, and includes a line between the earth station in question and the GSO space station that it is communicating with (FCC 47 CFR 25.103).

Plane tangent to the GSO arc: The plane defined by the location of an earth station's transmitting antenna and a line in the equatorial plane that is tangent to the GSO arc at the location of the GSO space station that the earth station is communicating with (FCC 47 CFR 25.103).

Satellite: A space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO), Medium-Earth Orbit (MEO), or Geostationary Earth Orbit (GEO).

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

BWChannel	Channel bandwidth

BWinterferer	Bandwidth of the interferer

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FInterferer	Frequency of the interferer

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PInterferer	Modulated mean power of the interferer

PUEType	Minimum UE type peak EIPR  (i.e. no tolerance) as specified in sub-clause 9.2.1

Puw	Power of an unwanted DL signal

θ	Angle in degrees from a line from the earth station antenna to the assigned orbital location of the target satellite

TRPmax	The maximum TRP for the NTN VSAT as specified in sub-clause 9.2.1

<End of the change>

<Start of the change>

## 9.2	Transmitter power

### 9.2.1	NTN VSAT maximum output power

#### 9.2.1.0	General

The NTN VSAT classes are specified based on the assumptions of certain NTN VSAT types with specific device architectures including antenna beam steering types. The requirements are specified for different NTN VSAT types. And for the the hybrid beam steering capable NTN VSAT, which can adjust its antenna(s) or beam(s) in both electronic steering and mechanical steering ways, the applicable requirements should follow either electronic or mechanical beam steering requirements depending on the NTN VSAT type it declared. The NTN VSAT types can be found in Table 9.2.1.0-1 below.

Table 9.2.1.0-1: Assumptions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| Note 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. |  |  |

#### 9.2.1.1	Minimum requirements for Fixed VSAT

The following requirements define the maximum output power radiated by the Fixed VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 9.2.1.1-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 9.2.1.1-1: Minimum peak EIRP for Fixed VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511, n510 | 1 | 70 |
|  | 2 | 70 |
|  | 3 | 61 |
| Note: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

The maximum output power values for TRP and EIRP are found in Table 9.2.1.1-2 below.

Table 9.2.1.1-2: Maximum output power limits for Fixed VSAT

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n512, n511, n510 | [35] | 76.2 |

#### 9.2.1.2	Minimum requirements for Mobile VSAT

The following requirements define the maximum output power radiated by the Mobile VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 9.2.1.2-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 9.2.1.2-1: Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511 | 4 | 70 |
|  | 5 | 70 |
| Note: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

The maximum output power values for TRP and EIRP are found in Table 9.2.1.2-2 below.

Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n512, n511, n510 | [35] | 76.2 |

### 9.2.2	Off-axis EIRP limit

#### 9.2.2.1	General

The Off-axis EIRP density envelope is applicable within the band to NTN VSAT transmitting to a SAN.

#### 9.2.2.2	Minimum requirement for bands n510 and n511

For co-polarized transmissions in the plane tangent to the GSO arc, the requirements specified in table 9.2.2.2-1 apply to NTN VSAT.

Table 9.2.2.2-1: Off-axis EIRP density limits for co-polarized transmissions in the plane tangent to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 62.5 – 25log(θ) | 1 |
| 7° ≤ θ ≤ 9.2° | 41.5 | 1 |
| 9.2° ≤ θ ≤ 19.1° | 65.5 – 25log(θ) | 1 |
| 19.1° < θ ≤ 180° | 33.5 | 1 |

For co-polarized transmissions in the plane perpendicular to the GSO arc, the requirements specified in table 9.2.2.2-2 apply to NTN VSAT.

Table 9.2.2.2-2: Off-axis EIRP density limits for co-polarized transmissions in the plane perpendicular to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 3.5° ≤ θ ≤ 7° | 65.5 – 25log(θ) | 1 |
| 7° ≤ θ ≤ 9.2° | 44.5 | 1 |
| 9.2° ≤ θ ≤ 19.1° | 68.5 – 25log(θ) | 1 |
| 19.1° < θ ≤ 180° | 36.5 | 1 |

The EIRP density levels specified in table 9.2.2.2-1 and table 9.2.2.2-2 may be exceeded by up to 3 dB, for values of θ > 7°, over 10% of the range of theta (θ) angles from 7–180° on each side of the line from the  NTN VSAT to the target SAN.

For cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc, the requirements specified in table 9.2.2.2-3 apply to NTN VSAT.

Table 9.2.2.2-2: Off-axis EIRP density limits for cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 52.5 – 25log(θ) | 1 |

#### 9.2.2.3	Minimum requirement for band n512

##### 9.2.2.3.1	Fixed VSAT

For co-polarized transmissions, the requirements specified in table 9.2.2.3.1-1 apply to Fixed VSAT.

Table 9.2.2.3.1-1: Off-axis EIRP density limits for co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.8° ≤ θ ≤ 7° | 49 – 25log(θ) - K | 40 |
| 7° ≤ θ ≤ 9.2° | 28 - K | 40 |
| 9.2° ≤ θ ≤ 48° | 52 – 25log(θ) - K | 40 |
| 48° < θ ≤ 180° | 20 - K | 40 |
| Note: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Fixed VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.4.2 in [19]. |  |  |

For cross-polarized transmissions, the requirements specified in table 9.2.2.3.1-2 apply to Fixed VSAT.

Table 9.2.2.3.1-2: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.8° ≤ θ ≤ 7° | 39 – 25log(θ) - K | 40 |
| 7° ≤ θ ≤ 9.2° | 18 - K | 40 |
| Note: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Fixed VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.4.2 in [19]. |  |  |

##### 9.2.2.3.2	Mobile VSAT

For co-polarized transmissions, the requirements specified in table 9.2.2.3.2-1 apply to Mobile VSAT.

Table 9.2.2.3.2-1: Off-axis EIRP density limits for co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 49 – 25log(θ) - K | 40 |
| 7° ≤ θ ≤ 9.2° | 28 - K | 40 |
| 9.2° ≤ θ ≤ 48° | 52 – 25log(θ) - K | 40 |
| 48° < θ ≤ 180° | 20- K | 40 |
| Note: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [18]. |  |  |

Mobile VSAT with low elevation angles may exceed the limits specified in Table 9.2.2.3.2-1 by the amount specified in Table 9.2.2.3.2-2

Table 9.2.2.3.2-2: Off-axis EIRP density limits for co-polarized transmissions

| Elevation angle to Satellite (ε) | Increase in EIRP density (dB) |
| --- | --- |
| ε < 5o | 2.5 |
| 5o < ε < 30o | 3 – 0.1 * ε |

For cross-polarized transmissions, the requirements specified in table 9.2.2.3.2-3 apply to Mobile VSAT.

Table 9.2.2.3.2-3: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 39 – 25log(θ) - K | 40 |
| 7° ≤ θ ≤ 9.2° | 18 - K | 40 |
| Note: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [18]. |  |  |

##### 9.2.2.3.3	Additional Off-axis EIRP density requirements for protection of fixed services

For uncoordinated NTN VSAT, the requirements specified in table 9.2.2.3.3-1 apply.

Table 9.2.2.3.3-1: Off-axis EIRP density limits for protection of fixed services

| Frequency Range (GHz) | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 27.8285—28.4445 |  |  |
| 28.8365 – 28.9485 (NOTE1) | -5 | 1 |
| 28.9485 – 29.4525 |  |  |
| NOTE1: When applicable, if this frequency range is allocated to fixed service. |  |  |

### 9.2.3	Configured transmitted power

The NTN VSAT can configure its maximum output power. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11].

The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

PUEType ≤ PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by the maximum TRP limit for NTN VSAT defined in sub-clause 9.2.1.

with PUEType is the NTN VSAT minimum peak EIRP as specified in sub-clause 9.2.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 9.2.1. The requirement is verified in beam peak direction.

The tolerance T(∆P) for applicable values of ∆P (values in dB) is specified in Table 9.2.3-1.

Table 9.2.3-1: PUMAX,f,c tolerance for FR2-NTN

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n510, n511, n512 | P = 0] | [0] |
|  | [0 < P ≤ 2] | [1.5] |
|  | [2 < P ≤ 3] | [2.0] |
|  | [3 < P ≤ 4] | [3.0] |
|  | [4 < P ≤ 5] | [4.0] |
|  | [5 < P ≤ 10] | [5.0] |
|  | [10 < P ≤ 15] | [7.0] |
|  | [15 < P ≤ X] | [8.0] |
| NOTE: X is the value such that Pumax,f,c lower bound, PUEType - P – T(P) = minimum output power specified in clause 9.2.1 |  |  |

## 9.3	Output power dynamics

### 9.3.1	Minimum output power

#### 9.3.1.0	General

The minimum controlled output power of the NTN VSAT is defined as the EIRP in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

The minimum output power is defined as the mean power in at least one sub frame (1ms).

#### 9.3.1.1	Minimum output power for Mobile VSAT

For Mobile VSAT, the minimum output power shall not exceed the values specified in Table 9.3.1.1-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 9.3.1.1-1: Minimum output power for Mobile VSAT type 4 and type 5

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n512, n511 | 50 | [TBD] | 47.58 |
|  | 100 | [TBD] | 95.16 |
|  | 200 | [TBD] | 190.20 |
|  | 400 | [TBD] | 380.28 |

#### 9.3.1.2	Minimum output power for Fixed VSAT

For Fixed VSAT, the minimum output power shall not exceed the values specified in Table 9.3.1.2-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 9.3.1.2-1: Minimum output power for Fixed VSAT type 1, type 2 and type 3

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n512, n511, n510 | 50 | [TBD] | 47.58 |
|  | 100 | [TBD] | 95.16 |
|  | 200 | [TBD] | 190.20 |
|  | 400 | [TBD] | 380.28 |

### 9.3.2	Transmit OFF power

#### 9.3.2.1	General

The transmit OFF power is defined as the TRP in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the NTN VSAT is not allowed to transmit on any of its ports.

#### 9.3.2.2	Minimum output power for Mobile VSAT

The transmit OFF power shall not exceed the values specified in Table 9.3.2.2-1 for each operating band supported. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 9.3.2.2-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511 | [-35] | [-35] | [-35] | [-35] |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

#### 9.3.2.3	Minimum output power for Fixed VSAT

The transmit OFF power shall not exceed the values specified in Table 9.3.2.2-1 for each operating band supported. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 9.3.2.2-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | [-35] | [-35] | [-35] | [-35] |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

<End of the change>

<Start of the change>

### 9.3.4	Power control

#### 9.3.4.1	General

The requirements on power control accuracy apply under normal conditions and are defined as a directional requirement. The requirements are verified in beam locked mode on beam peak direction.

The applicant shall declare the method and accuracy of uplink power control.

#### 9.3.4.2	Absolute power tolerance

The absolute power tolerance is the ability of the NTN VSAT transmitter to set its initial output power to a specific value for the first sub-frame (1 ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error RSRP estimate.

The minimum requirements specified in Table 9.3.4.2-1 apply in the power range bounded by the minimum output power as specified in sub-clause 9.3.1 ('Pmin') and the maximum output power as specified in sub-clause 9.2.1 as minimum peak EIRP ('Pmax'). The intermediate power point 'Pint' is defined in table 9.3.4.2-2.

Table 9.3.4.2-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | [± 14.0 dB] |
| Pmax ≥ P > Pint | [± 12.0 dB] |

Table 9.3.4.2-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | [Pmax – 12.0 dB] |

#### 9.3.4.3	Relative power tolerance

The relative power tolerance is the ability of the NTN VSAT transmitter to set its output power in a target sub-frame (1 ms) relatively to the power of the most recently transmitted reference sub-frame (1 ms) if the transmission gap between these sub-frames is less than or equal to 20 ms.

The minimum requirements specified in Table 9.3.4.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in sub-clause 9.3.1 and Pint as defined in sub-clause 9.3.4.2. The minimum requirements specified in Table 9.3.4.3-2 apply when the power of the target and reference sub-frames are within the power range bounded by Pint as defined in sub-clause 9.3.4.2 and the measured PUMAX as defined in sub-clause 9.2.4.

For a test pattern that is either a monotonically increasing or monotonically decreasing power sweep over the range specified for Tables 9.3.4.3-1 and 9.3.4.3-2, 3 exceptions are allowed for each of the test patterns. For these exceptions, the power tolerance limit is a maximum of ±11.0 dB.

Table 9.3.4.3-1: Relative power tolerance, Pint ≥ P ≥ Pmin

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | [±5.0] |
| 2 ≤ ΔP < 3 | [±6.0] |
| 3 ≤ ΔP < 4 | [±7.0] |
| 4 ≤ ΔP < 10 | [±8.0] |
| 10 ≤ ΔP < 15 | [±10.0] |
| 15 ≤ ΔP | [±11.0] |
| NOTE: The requirements apply with ue-BeamLockFunction enabled. |  |

Table 9.3.4.3-2: Relative power tolerance, PUMAX ≥ P > Pint

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | [± 3.0] |
| 2 ≤ ΔP < 3 | [± 4.0] |
| 3 ≤ ΔP < 4 | [± 5.0] |
| 4 ≤ ΔP < 10 | [± 6.0] |
| 10 ≤ ΔP < 15 | [± 8.0] |
| 15 ≤ ΔP | [± 9.0] |
| NOTE 1: The requirements apply with ue-BeamLockFunction enabled. NOTE 2: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, guard periods: for a power step ΔP = 1 dB, the relative power tolerance for transmission is ± 1.0 dB. |  |

#### 9.3.4.4	Aggregate power tolerance

The aggregate power control tolerance is the ability of the NTN VSAT transmitter to maintain its power in a sub-frame (1 ms) during non-contiguous transmissions within 21ms in response to 0 dB TPC commands with respect to the first NTN VSAT transmission and all other power control parameters as specified in 38.213 kept constant.

The minimum requirements specified in Table 9.3.4.4-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in sub-clause 9.3.1 and Pint as defined in sub-clause 9.3.4.2. The minimum requirements specified in Table 9.3.4.4-2 apply when the power of the target and reference sub-frames are within the power range bounded by Pint as defined in sub-clause 9.3.4.2 and the maximum output power as specified in sub-clause 9.2.1.

Table 9.3.4.4-1: Aggregate power tolerance, Pint ≥ P ≥ Pmin

| TPC command | UL channel | Aggregate power tolerance within 21 ms |
| --- | --- | --- |
| 0 dB | PUCCH | [± 5.5 dB] |
| 0 dB | PUSCH | [± 5.5 dB] |

Table 9.3.4.4-2: Aggregate power tolerance, Pmax ≥ P > Pint

| TPC command | UL channel | Aggregate power tolerance within 21 ms |
| --- | --- | --- |
| 0 dB | PUCCH | [± 3.5 dB] |
| 0 dB | PUSCH | [± 3.5 dB] |

## 9.4	Transmitter signal quality

### 9.4.1	Frequency Error

The NTN VSAT basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN VSAT pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN VSAT modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the NTN VSAT by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the NTN VSAT is explicitly provided to the NTN VSAT from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions.

The frequency error is defined as a directional requirement. The requirement is verified in beam locked mode with the test metric of Frequency (Link=TX beam peak direction, Meas=Link angle).

### 9.4.2	Transmit modulation quality

The requirements for transmit modulation quality defined in 3GPP TS 38.101-2 [x] clause 6.4.2 except clause 6.4.2.2, 6.4.2.3, 6.4.2.4, 6.4.2.5, 6.4.2.6 shall apply for NTN VSAT.

## 9.5	Output RF spectrum emissions

### 9.5.1	Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 9.5.1-1.

The occupied bandwidth is defined as a directional requirement. The requirement is verified in beam locked mode with the test metric of OBW (Link=TX beam peak direction, Meas=Link angle).

Table 9.5.1-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Channel bandwidth (MHz) | 50 | 100 | 200 | 400 |

### 9.5.2	Out of Band Emissions

#### 9.5.2.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio. Additional requirements to protect specific bands are also considered.

The requirements in clause 9.5.2.2 only apply when both UL and DL of a NTN VSAT are configured for single CC operation, and they are of the same bandwidth.

All out of band emissions for FR2-NTN are TRP.

The spectrum emission mask of the NTN VSAT applies to frequencies starting from the  edge of the assigned NR channel bandwidth.

#### 9.5.2.2	Spectrum emission mask

#### 9.5.2.2.1	General NR spectrum emission mask

The power of any NTN VSAT emission shall not exceed the levels specified in Table 9.5.2.2-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 9.5.2.2.1-1: General NR spectrum emission mask for NTN-FR2

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BW | 0.5 MHz f_offset < 2× BW + 0.5 MHz | max11SE limit, TRPPrated,UEmax – 10log10(BW) –40×log10 f_offset-0.5BW×2+1dBm | 1 MHz |
| NOTE 1: TRPmax is specified in clause 9.2.1. NOTE 2: Transmission BW is in the unit of MHz. NOTE 3: The 11dBm/1MHz value corresponds to the spurious emission limit specified in spurious emission clause 9.5.3, and is converted from the SE limit requirement defined on 4 kHz to a value defined over 1 MHz. NOTE 4: PSD attenuation as in ITU-R SM.1541-6 [6], Annex 5 OoB domain emission limits for earth stations. |  |  |  |

#### 9.5.2.2.2	Additional spectrum emission mask

For bands n511 and n510 the mean power of emissions shall be attenuated below the mean output power of the transmitter (measured in dBm) in accordance with [FCC 25.202].

The power of any NTN VSAT emission shall not exceed the levels specified in Table 9.5.2.2.2-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 9.5.2.2.2-1: Additional spectrum emission mask

| Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0.002MHz+0.5xBW f_offset < 1xBW-0.002MHz | TRPmax (dBm) - 25 dB | 4 kHz |
| 0.002MHz+1xBW f_offset < 2.5xBW-0.002MHz | TRPmax (dBm) - 35 dB | 4 kHz |
| 0.002MHz+2.5xBW f_offset < 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 4 kHz |
| NOTE 1: TRPmax is specified in clause 9.2.1. NOTE 2: Transmission BW is in the unit of MHz. NOTE 3: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 4: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. |  |  |

<End of the change>

<Start of the change>

##### 9.5.3.2.3		“Carrier-on” state

The requirements specified in tables 9.5.3.2.3-1 and 9.5.3.2.3-2 apply to NTN VSAT in “Carrier-on”.

The requirement specified in table requirements specified in table 9.5.3.2.3-1 apply outside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency.

The requirement specified in table requirements specified in table 9.5.3.2.3-2 apply inside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency, and outside the transmission bandwidth.

Note: The on-axis spurious radiations, outside the bands 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz, are

indirectly limited by subclause 9.5.3.2.3. Consequently no specification is needed.

Table 9.5.5.2.2.3-1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 44 - K (Note) | 1 |
|  | 1, 2, 3 | 4 - K (Note) | 1 |
| Note: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [18] for mobile VSAT or sub-clause 4.2.4.2 in [19] for fixed VSAT. |  |  |  |

Table 9.5.5.2.2.3-2: On-axis spurious limits in “Carrier-on” state - inside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 58 - K (Note) | 1 |
|  | 1, 2 ,3 | 48 - K (Note) | 1 |
| Note: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [18] for mobile VSAT or sub-clause 4.2.4.2 in [19] for fixed VSAT. |  |  |  |

<End of the change>

<Start of the change>

## 9.6	Antenna performance and pointing accuracy

## 9.6.1	Antenna pointing accuracy

#### 9.6.1.1	Minimum requirement for Mobile VSAT

The applicant shall declare the peak pointing accuracy () and the associated statistical basis.

The antenna shall maintain the declared peak pointing accuracy (), such that the off-axis EIRP emission density pattern projected onto the geostationary arc remains within the mask specified in clauses 9.2.2.2 and 9.2.2.3 when shifted by an angle of ±(°), taking into account the following factors [EN 303 978]:

the worst case operational environmental conditions;

maximum ESOMP dynamics; and

the range of latitude, longitude and altitude relative to the satellite orbital position.

For linearly polarized ESOMPs, the following specification is required. The applicant shall declare the on-axis cross polarization isolation of the ESOMP [EN 303 978]:

The polarization angle shall be continuously adjustable within the operational range as declared by the applicant.

It shall be possible to fix the transmit antenna polarization angle with an accuracy of at least 1°.

When linear polarization is used for both transmission and reception, the angle between the receive and corresponding transmit polarization planes shall not deviate by more than 1° from the nominal value declared by the applicant.

- For circularly polarized ESOMPs, the applicant shall declare the voltage axial ratio.

#### 9.6.1.2	Minimum requirement for Fixed VSAT

The minimum requirements are defined in terms of a) pointing stability, b) pointing accuracy capability and c) polarization angle alignment capability for linear polarization, as follows:

a) Pointing stability: Under the condition of 100 km/h maximum wind speed, with gusts of 130 km/h lasting 3 seconds, the installation shall not show any sign of permanent distortion and shall not need repointing after the application of the wind load.

b) Pointing accuracy capability: The applicant shall declare the usage area in terms of the range of latitude and longitude relative to the satellite orbital position where the alignments specified below are possible. Two sets of specifications are further defined:

- Specification 1: Main beam pointing accuracy. The antenna sub-system alignment facilities shall enable the main beam axis to be adjusted and fixed with a pointing accuracy (δφ) of either:

  - 1) 0,1º; or

  - 2) a greater value declared by the applicant, subject to the following restrictions:

    - the pointing accuracy (δφ) shall not exceed 30 % of the antenna transmit main beam half power beamwidth;

    - the off-axis e.i.r.p. emission density pattern remains within the mask specified in clause 9.2.2.3 when shifted by an angle of ±(δφ - 0,1º).

- Specification 2: Alignment with the geostationary satellite orbit. For antennas with asymmetric main beam, the antenna shall be capable of having the plane defined by the antenna main beam axis and its major axis aligned with the tangent to the geostationary orbit in accordance with the method declared by the applicant.

c) Polarization angle alignment capability for linear polarization. Following conditions will apply:

-	The polarization angle shall be continuously adjustable within the operational range as declared by the applicant.

-	It shall be possible to fix the transmit antenna polarization angle with an accuracy of at least 1°.

-	When linear polarization is used for both transmission and reception, the angle between the receive and corresponding transmit polarization planes shall not deviate by more than 1° from the nominal value declared by the applicant.

### 9.6.2	Antenna performance

The following requirements are applicable to NTN VSAT type 1, type 2, type 4 or type 5 operating in band n511 or in band n510.

The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2-1: Co-polarization gain limit in the plane tangent to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 29 – 25log(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log(θ) |
| 19.1° < θ ≤ 180° | 0 |

The co-polarization gain in the plane perpendicular to the GSO arc  shall not exceed the levels specified in Table 9.6.2-2. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2-2: Co-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3.5° ≤ θ ≤ 7° | 32 – 25log(θ) |
| 7° ≤ θ ≤ 9.2° | 10.9 |
| 9.2° ≤ θ ≤ 19.1° | 35 – 25log(θ) |
| 19.1° < θ ≤ 180° | 3 |

The off-axis cross-polarization gain in the plane tangent and in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2-3.

Table 9.6.2-3: Cross-polarization gain limit

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 19 – 25log(θ) |

<End of the change>