# 3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404974

# Changsha, China, 15th – 19th April, 2024

Agenda Item:	6.24.3.2

Source:	Ericsson

Title:	Discussion on NES test case

Document for:	Discussion

# Introduction

In last meeting, RAN4 agreed the SSB-less SCell activation and NES-based CHO test case in NES. The only remaining issue in test case design is whether to test Cell DTX/DRX. In this contribution, we discuss the cell DTX/DRX test case.

Issue 3-2-1: RRM performance impacts of Cell DTX/DRX - Further discuss: - Option 2: No test case for Cell DTX/DRX. - Not support: E///, Nokia, ZTE - Option 3: Define one test case for Cell DTX: FFS the details - Option 3a: SA event triggered reporting tests without gap under Cell DTX. - QC, vivo, MTK, Apple, OPPO want to check whether option 3 is feasible.

# Cell DTX/DRX test case

In last meeting, UE vendors have concern to test UE’s behaviour in Cell DTX/DRX mode since RAN4 agreed no measurement impact due to Cell DTX/DRX. We think such observation is valid. However, from NW’s perspective, our concern is whether UE will fulfil the same behaviour when NW configures Cell DTX. For example, whether UE may also have some sleep mode when NW configures Cell DTX which is different with our requirement. Thus, at least test cases about the impact due to Cell DTX/DRX for both CSI-RS RLM and measurement shall be defined. To move forward, we think it’s fine to define applicability rule that if UE passes the Cell DTX test can skip the non Cell DTX test from Rel-18.

Proposal 1: RAN4 to define the test cases for Cell DTX/DRX procedure.

- SA event triggered reporting tests without gap under Cell DTX

Proposal 2: RAN4 to define the applicability rule for Cell DTX/DRX test as follow.

- A UE which fulfils the requirements in Cell DTX test case can skip the test case in A.6.6.1.1 SA event triggered reporting tests without gap under non-DRX.

# Conclusion

In the contribution, we discuss the test case design for NES. We have the following proposals:

Proposal 1: RAN4 to define the test cases for Cell DTX/DRX procedure.

- SA event triggered reporting tests without gap under Cell DTX

Proposal 2: RAN4 to define the applicability rule for Cell DTX/DRX test as follow.

- A UE which fulfils the requirements in Cell DTX test case can skip the test case in A.6.6.1.1 SA event triggered reporting tests without gap under non-DRX.

# Reference

[1] R4-2403526, “WF on RRM requirements for Netw_Energy_NR”, Huawei