3GPP TSG-RAN WG4 Meeting #110bis	R4-2404983

Changsha, China, April 15th – 19th 2024

Agenda Item:	6.18.4

Source:	Ericsson

Title:	Discussion on NCR EMC

Document for:	Discussion

# 1	Introduction

In this contribution, we present our views and proposals on open issues listed in the WF [1] approved in RAN4#110.

# 2	Discussion

Radiated emission limits for NCR

The radiated emission test is applicable to NCR type 1-C and type 1-H, while the radiated emission requirement on NCR type 2-O is covered by radiated spurious emission requirement in TS 38.106 [2].

Regarding NCR type 1-C and type 1-H, the radiated emission limits for NCR need to follow the RSE limits defined in TS 38.106 [2]. Reuse the NR repeater’s RSE limits for NCR-Fwd and WA NCR-MT. Reuse the UE’s RSE limits for LA NCR-MT.

Regarding NCR type 1-C and type 1-H, reuse the NR repeater’s RSE limits for NCR-Fwd and WA NCR-MT. Reuse the UE’s RSE limits for LA NCR-MT.

Performance criteria for NCR-Fwd and NCR-MT

Considering NCR-Fwd has similar function to NR repeater, and the performance criteria for NR repeater is power accuracy (no more than ± 2 dB for type 1-C and no more than ± 3 dB for 2-O) as defined in TS 38.114 [3]. The existing power accuracy requirements can be reused for NCR-Fwd, i.e., no more than ± 2 dB for NCR-Fwd type 1-C and type 1-H, and no more than ± 3 dB for NCR-Fwd type 2-O.

Reuse the performance criteria for NR repeater, i.e., power accuracy as the performance criteria for NCR-Fwd, with the value no more than ± 2 dB for NCR-Fwd type 1-C and type 1-H, and no more than ± 3 dB for NCR-Fwd type 2-O.

Unlike NCR-Fwd, which simply amplifies RF power, NCR-MT plays a crucial role in decoding control information, and it is essential to check the signal quality. Therefore, throughput can be adopted as the performance criteria. The same requirement as for NR BS (throughput > 95 %) defined in TS 38.113 [4] can be reused for NCR-MT.

Reuse the performance criteria for NR BS, i.e., throughput > 95 % as the performance criteria for NCR-MT.

# Conclusion

In this contribution, views, and proposals on general aspects for SBFD WI are presented.:

Proposal 1	Regarding NCR type 1-C and type 1-H, reuse the NR repeater’s RSE limits for NCR-Fwd and WA NCR-MT. Reuse the UE’s RSE limits for LA NCR-MT.

Proposal 2	Reuse the performance criteria for NR repeater, i.e., power accuracy as the performance criteria for NCR-Fwd, with the value no more than ± 2 dB for NCR-Fwd type 1-C and type 1-H, and no more than ± 3 dB for NCR-Fwd type 2-O.

Proposal 3	Reuse the performance criteria for NR BS, i.e., throughput > 95 % as the performance criteria for NCR-MT.

# References

R4-2402961, WF on NCR EMC Conformance, ZTE

TS 38.106, NR repeater radio transmission and reception

TS 38.114, NR; Repeaters ElectroMagnetic Compatibility (EMC)

TS 38.113, NR; Base Station (BS) ElectroMagnetic Compatibility (EMC)