3GPP TSG-RAN WG4 Meeting #110bis	R4-2401598

Changsha, China, April 15-19, 2024

Source: 	Ericsson

Title:  	Discussion on test cases for above 10GHz bands

Agenda Item:	6.16.7.1

Document for:	Discussion

# Introduction

In this contribution, we herein initiate preliminary views on test scope and test cases for above 10GHz bands.

# Discussion

In core part, the below test scope has been agreed.

### Issue 6-1: Configuration of test cases Agreement: - Strive to reuse Rel-17 NR NTN test setup for satellite specific parameters. Any changes, if needed/identified for FR2-NTN, can be considered. ### Issue 6-2: NTN bands above 10 GHz Agreement: - For FR2-NTN Type 1 and Type 2 UEs, the below are baseline. Type 1 and Type 2 UEs can have different sets of test cases. - RRC Idle and Inactive mobility in intra-satellite scenario - UL timing accuracy - L1-RSRP - RLM - L3 measurements in intra-satellite scenario - Intra-satellite Handover - Blind inter-satellite Handover - For relative accuracy for intra-frequency measurement, FFS whether to define requirements for intra-sat only based on the assumption of same Rx beam.

Observation 1: Test configurations including satellite configuration/ephemeris data can reuse the existing ones.

Proposal 1: Regarding intra-satellite test cases for NTN bands above 10 GHz, Type 1 and Type 2 UEs have not difference from core requirement and test case/configuration/requirement perspective, e.g., in below test cases no need to differentiate between Type 1 and Type 2 UEs.

- RRC Idle and Inactive mobility in intra-satellite scenario

- UL timing accuracy

- L1-RSRP

- RLM

- L3 measurements in intra-satellite scenario

- Intra-satellite Handover

Proposal 2: Regarding blind inter-satellite Handover for NTN bands above 10 GHz, the angular distance between source satellite and target satellite shall be configured, it can be implemented by AoA setup for two cells.

# Conclusion

Observation 1: Test configurations including satellite configuration/ephemeris data can reuse the existing ones.

Proposal 1: Regarding intra-satellite test cases for NTN bands above 10 GHz, Type 1 and Type 2 UEs have not difference from core requirement and test case/configuration/requirement perspective, e.g., in below test cases no need to differentiate between Type 1 and Type 2 UEs.

- RRC Idle and Inactive mobility in intra-satellite scenario

- UL timing accuracy

- L1-RSRP

- RLM

- L3 measurements in intra-satellite scenario

- Intra-satellite Handover

Proposal 2: Regarding blind inter-satellite Handover for NTN bands above 10 GHz, the angular distance between source satellite and target satellite shall be configured, it can be implemented by AoA setup for two cells.

# References

- R4-2403492, WF on R18 NR NTN RRM requirements, Qualcomm