3GPP TSG-RAN WG4 Meeting # 110-bis					R4-2405040

China, Changsha, April 15 – April 19, 2024

Title:	TP to TR 37.718-11-21: Addition of DC_28A_n1A-n105A

Source:	Nokia, Spark

Agenda item:	5.6.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 37.718-11-21 to add DC_28A_n1A-n105A as specified in WID [1]. All fallbacks have been analysed.

************************************* Start of TP*****************************************

## 6.x	DC_28_n1-n105

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n1-n105 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.x.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_28A_n1A-n105A | DC_28A_n1A |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n1A-n105A |  |  | 28 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 105 |
|  |  |  | n1 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n105 |  | 15 |  |  | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

For UE coexistence study of UL DC_28_n1, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.x.3-1.

Based on the calculation, we do not identify any interference impact:

Table 6.x.3-1: The harmonic and IMD products caused by UL DC_28_n1

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 2110 | 2170 |
| 3rd Band DL | 612 | 652 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

### 6.x.4	∆TIB and ∆RIB values

For DC_28_n1-n105, ΔTIB,c and ΔRIB,c values are as follows (from CA_n1-n5-n28).

Table 6.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n1-n105 | 0.6 | 0.3 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.x.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28_n1-n105 | 0.2 | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 6.x.5	MSD requirements

There is no additional receiver requirements for MSD.

************************************* End of TP******************************************

# References

[1]	RP-233488，Rel-18 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)，LG Electronics，RAN#102, December 2023