# 3GPP TSG- RAN WG4 Meeting # 110bis	R4-2405062

# Changsha, China, 15th–19th Apr, 2024

# Source:	ZTE Corporation

# Title:	Discussion on UE RF requirements for inter-band NR CA/DC with 3MHz CBW

# Agenda Item:	9.10.2

# Document for:	Approval

- Introduction

In RAN#103 meeting, a new Work Item [1] was approved on NR channel BW less than 5MHz for FR1 Phase 2 with the following RAN4 objective. In this contribution, we want to share some initial views on UE RF requirements for inter-band NR CA/DC with 3MHz CBW.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

- Discussion

  - 2.1 System parameters

2.1.1 Operating band for CA

As listed in the objective, example band combination is CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101. So we can define the new inter-band CA operating band as follows:

Table 2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n100-n101 | n100, n101 |

Proposal 1: To define the new inter-band CA operating band as in Table 2.1-1.

2.1.2 Configurations for CA

As listed in the objective, example band combination is CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101. So we can define the new NR CA configurations and bandwidth combinations sets for inter-band CA as follows:

Table 2.1-2: NR CA configuration and bandwidth combination sets for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3 | 0 |
|  |  | n101 | 5, 10 |  |

Proposal 2: To define the new NR CA configuration and bandwidth combination sets for inter-band CA as in Table 2.1-2.

2.1.3 Configurations for DC

A new inter-band NR DC configuration needs to be defined as follows:

Table 2.1-3: Inter-band NR DC configurations (two bands)

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n100A-n101A | DC_n100A-n101A |

Proposal 3: To define the new inter-band NR DC configuration for inter-band CA as in Table 2.1-3.

  - 2.2 UE RF

2.2.1 RF architecture

One of the objective is to define co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band, and the example band combination is CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101. It is a low FDD band and middle TDD band combination, which differentiates from LB-LB and LB-HB combination. For inter-band CA_n100-n101, the possible RF architecture implementation can be assumed as is shown in Figure 2.2-1.

Figure 2.2-1: Possible RF architecture implementation for CA_n100-n101

Proposal 4: For inter-band CA_n100-n101, the possible RF architecture implementation can be assumed as is shown in Figure 2.2-1.

2.2.2 Transmitter power for CA/DC

One of the objective is to define co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band, and the example band combination is CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101. Band n100 and n101 all support PC 1 and PC 3. So it is proposed that PC 3 should be supported for CA/DC and UE maximum output power for CA/DC can be defined as Table 2.2-1 and 2.2-2.

Table 2.2-1: UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n100A-n101A |  |  |  |  | 23 | +2/-3 |  |  |

Table 2.2-2: UE Power Class for uplink inter-band NR-DC

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n100A-n101A |  |  |  |  | 23 | +2/-3 |  |  |

Proposal 5: Inter-band CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 should support PC 3 and UE maximum output power can be defined as Table 2.2-1 and 2.2-2.

2.2.3 Maximum Sensitivity Degradation (MSD) analysis

For inter-band CA_n100-n101, sensitivity degradation needs to be analysised, and the harmonic and IMD products can be calculated by the table shown in Annex. As can be seen in the table, no MSD issue needs to be discussed.

Proposal 6: For inter-band CA_n100-n101, no MSD issue needs to be discussed.

2.2.4 Power imbalance for CA

In current specification, power imbalance requirement is only applicable for a UE indicating  [intraBandNR-CA-non-collocated-r18] and if [nonCollocatedTypeNR-CA-r18] is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2. For intra-band CA, there exists a capability to distingish whether non-collocated is supported. Therefore, for inter band CA_n100-n101, whether a new signalling of indicating inter-band non-collocated is required needs discussion. Considering power imbalance parameters for non-co-located inter band CA_n100-n101, 25 dB power imbalance can be reused.

Observation 1: Whether a new signalling of indicating inter-band non-collocated is required needs discussion.

Proposal 7: Considering power imbalance parameters for non-co-located inter band CA_n100-n101, 25 dB power imbalance can be reused.

  - 2.3 Summary of corrections on TS 38.101-1

Based on the above, the summary of the corrections on TS 38.101-1 are given in Table 2.3-1.

Table 2.3-1: The summary of the corrections on TS 38.101-1

| General part |  |
| --- | --- |
| General part | Proposal for inter-band CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 |
| 5.2A Operating bands for CA | A new inter-band CA operating band needs to be defined. |
| 5.3A UE channel bandwidth for CA | No requirement impacts. |
| 5.4A Channel arrangement for CA | No requirement impacts. |
| 5.5A Configurations for CA | A new NR CA configurations and bandwidth combinations sets for inter-band CA needs to be defined. |
| 5.5B Configurations for DC | A new inter-band NR DC configuration needs to be defined. |
| UE RF part |  |
| NR UE Tx/Rx requirement | Proposal for inter-band CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 |
| 6.2A.1 UE maximum output power for CA | Inter-band CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 should support PC3 and UE maximum output power can be defined as table 2.2-1. |
| 6.2A.2 MPR for CA | No specification impact. |
| 6.2A.3 A-MPR for CA | No specification impact. |
| 6.2A.4 Configured transmitted power for CA | ΔTIB,c = [0.3] for Inter-band CA (two bands) needs to be defined. |
| 6.2B.1 UE maximum output power for NR-DC | Inter-band CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 should support PC3 and UE maximum output power can be defined as table 2.2-2. |
| 6.2B.2 MPR for NR-DC | No specification impact. |
| 6.2B.3 A-MPR for NR-DC | No specification impact. |
| 6.2B.4 Configured transmitted power for NR-DC | No specification impact. |
| 6.3A Output power dynamics for CA | No specification impact. |
| 6.4A Transmit modulation quality for CA | No specification impact. |
| 6.5A Output RF spectrum emissions for CA | No specification impact. |
| 7.3A.2 Reference sensitivity for CA | ΔRIB,c = [0] for Inter-band CA (two bands) needs to be defined. |
| 7.4A Max input level for CA | No specification impact. |
| 7.5A Adjacent channel selectivity for CA | No specification impact. |
| 7.6A Blocking characteristics for CA | No specification impact. |
| 7.7A Spurious response for CA | No specification impact. |
| 7.8A Intermodulation characteristics for CA | No specification impact. |
| 7.9A Spurious emissions for CA | No specification impact. |
| 7.10A Power imbalance for CA | Considering power imbalance parameters for non-co-located inter band CA_n100-n101, 25 dB power imbalance can be reused. |

- Conclusion

In this contribution, we want to share some views on UE RF requirements for inter-band NR CA/DC with 3MHz CBW and the proposals are made as following:

Proposal 1: To define the new inter-band CA operating band as in Table 2.1-1.

Proposal 2: To define the new NR CA configuration and bandwidth combination sets for inter-band CA as in Table 2.1-2.

Proposal 3: To define the new inter-band NR DC configuration for inter-band CA as in Table 2.1-3.

Proposal 4: For inter-band CA_n100-n101, the possible RF architecture implementation can be assumed as is shown in Figure 2.2-1.

Proposal 5: Inter-band CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 should support PC 3 and UE maximum output power can be defined as Table 2.2-1 and 2.2-2.

Proposal 6: For inter-band CA_n100-n101, no MSD issue needs to be discussed.

Observation 1: Whether a new signalling of indicating inter-band non-collocated is required needs discussion.

Proposal 7: Considering power imbalance parameters for non-co-located inter band CA_n100-n101, 25 dB power imbalance can be reused.

- Reference

RP-240832, New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN, vivo

- Annex

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 874.4 | 880 | 1900 | 1910 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1020 | 1035.6 | 2774.4 | 2790 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 161.2 | 140 | 2920 | 2945.6 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3648.8 | 3670 | 4674.4 | 4700 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2071.2 | 2040 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 713.2 | 740 | 4820 | 4855.6 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5548.8 | 5580 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4523.2 | 4550 | 6574.4 | 6610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6765.6 | 6720 | 1620 | 1587.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8474.4 | 8520 | 5397.6 | 5430 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3981.2 | 3940 | 1160 | 1196.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7448.8 | 7490 | 6423.2 | 6460 |