3GPP TSG-RAN WG4 Meeting #110bis	R4-2405098

Changsha, China, 15 April – 19 April 2024

Agenda Item:	6.21.3.1

Source:	Ericsson

Title:	Simulation results of UE demodulation requirements for RedCap enhancements

Document for:	Information

# 1	Introduction

RAN4#110 agreed with the way forward on UE demodulation and CSI reporting requirements for RedCap enhancements (eRedCap) [1]. This contribution provides our simulation results.

# 2	PDSCH demodulation

## 2.1	Test cases for UE not supporting eRedCapNotReducedBB-BW-r18

Table 1	Alignment results for UE not supporting eRedCapNotReducedBB-BW-r18 (FDD/HD-FDD).

| Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx | MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | 1.0 |
| 2Rx | MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | -3.0 |
| 1Rx | MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 9.1 |
| 2Rx | MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 4.8 |

Table 2	Alignment results for UE not supporting eRedCapNotReducedBB-BW-r18 (TDD).

| Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx | MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | 1.3 |
| 2Rx | MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | -2.7 |
| 1Rx | MCS13 in Table 1 | 16QAM, 0.47 | 16QAM, 0.47 | 12 | 3104 | TDLC300-100 | 9.4 |
| 2Rx | MCS13 in Table 1 | 16QAM, 0.47 | 16QAM, 0.47 | 12 | 3104 | TDLC300-100 | 5.2 |

## 2.2	Test cases for UE supporting eRedCapNotReducedBB-BW-r18

Table 3	Alignment results for UE supporting eRedCapNotReducedBB-BW-r18 (FDD/HD-FDD).

| Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx | MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | 0.6 |
| 2Rx | MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | -3.2 |
| 1Rx | MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 9.3 |
| 2Rx | MCS4 in Table 1 | 16QAM, 0.47 | Rank 1 | 52 | 3904 | TDLB100-400 | 4.9 |

Table 4	Alignment results for UE supporting eRedCapNotReducedBB-BW-r18 (TDD).

| Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR to achieve 70% of the maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx | MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | 0.5 |
| 2Rx | MCS4 in Table 1 | QPSK, 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | -3.1 |
| 1Rx | MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 8.9 |
| 2Rx | MCS13 in Table 1 | 16QAM, 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 4.9 |

# References

- R4-2402870, “Way forward on [110][325] NR_redcap_enh_demod”, Ericsson.