3GPP TSG-RAN WG4 Meeting #110bis	R4-2405102

# Changsha, China, April 15-19, 2024

Source:	Ericsson

Title:	On UE RF requirements

Agenda item:	9.7.2

Document for:	Approval

# 1	Introduction

In RAN#103 a new RAN4-led WI was approved,  New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s) [1].

… ### 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the core requirements for type 4 FWA UE that supports non-co-located scenarios for both FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and FR1 intra-band non-contiguous NR-CA for 4-layer DL MIMO, where MRTD>CP is assumed. - RF power imbalance requirements - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. Note: The requirements specified for type 2 UE should be reused as much as possible …

In this contribution we discuss the UE RF core part.

# 2	Discussion

The UE types are defined in R4-2217734 (WF on NonCol_intraB_ENDC_NR_CA for New Type UE [2]):

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

## 2.1	Power imbalance

In release 18 we have specified the following for EN_DC in TS 38-101-3:

… Table 7.10B.3-1: Test parameters for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Frequency relationship (Center of BWanother Relative to edge of BWwanted). 1; Wanted carrier; REFSENS + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz). Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS + 1; BWwanted > BWanother. Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25. NOTE 1: For NR carrier, the transmitter shall be set to 24dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 24 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 3: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with overlapping DL bands NOTE 4: REFSENS is the reference sensitivity level or two antenna port in Table 7.3.2-1a or in Table 7.3.2-1b of TS 38.101-1, or in Table 7.3.1-1 of TS 36.101. NOTE 5: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between E-UTRA and NR carriers for the EN-DC combinations in Table 7.10B.3-2. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 6: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used.. …

In a similar way we have, for release-18, specified for CA in TS 38.101-1:

… Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Center of BWanother Relative to edge of BWwanted. 1; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz. Another wanted carrier; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted > BWanother. Another wanted carrier; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS NOTE 4 + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier; Power of wanted carrier + 25. NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: The minimum requirements only apply for non simultaneous Rx/Tx between all carriers for TDD intra-band non-contiguous CA band combinations in Table 7.10A.2-2. …

Observation 1: For baseline approved specifications we have agreed a power imbalance of 25 dB for UE Type 2.

For the type 4a and 4b UE architectures we have no LNA sharing, similar to the type 2 architecture. It is also reasonable to make type 4 UE compatible with a network with type 2 UE

| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |

Observation 2: Type 4a and 4b UE architectures we have no LNA sharing, similar to the type 2 architecture.

Based on these observations, no LNA sharing and compatibility with networks deployed with Type 2 UE it is fair to propose that the power imbalance for Type 4a and 4b is 25 dB.

Proposal 1: RF power imbalance requirements for NRCA and ENDC is 25 dB.

# 3 	Summary

Observation 1: For baseline approved specifications we have agreed a power imbalance of 25 dB for UE Type 2.

Observation 2: Type 4a and 4b UE architectures we have no LNA sharing, similar to the type 2 architecture.

Proposal 1: RF power imbalance requirements for NRCA and ENDC is 25 dB.

# References

[1] 	RP-240728, New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Apple.

[2]	R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI