3GPP TSG-RAN WG4 Meeting #110b	R4-2405124

# Changsha, China, 15th - 19th Apr, 2024

Title: 	Simulation results on BS demodulation requirements for further coverage enhancements

Source: 	Huawei, HiSilicon

Agenda item:	6.17.2

Document for:	Discussion

Background

During RAN4#110 meeting, WF [1] on NR_cov_enh2_demod was approved. In this contribution, we provide our initial simulation results about BS demodulation requirements for further coverage enhancements.

Simulations

Table 2-1 Ideal simulation results for PRACH repetition number 2

| Antenna configuration | SCS | Propagation conditions | Frequency offset | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A2 | Burst format B4 | Burst format C2 |
| 1x2 | 15kHz | AWGN | 0 | -19.1 | -20.1 | -19.1 |
|  |  | TDLC300-100 Low | 400 Hz | -11.6 | -14.8 | -11.3 |
|  | 30kHz | AWGN | 0 | -19.2 | -20.6 | -19.3 |
|  |  | TDLC300-100 Low | 400 Hz | -11.7 | -15.2 | -11.8 |
|  | 60kHz | AWGN | 0 | -18.9 | -20 | -19.1 |
|  |  | TDLA30-300 Low | 4000 Hz | -9.9 | -12.9 | -10.1 |
|  | 120kHz | AWGN | 0 | -18.8 | -19.7 | -18.9 |
|  |  | TDLA30-300 Low | 4000 Hz | -11.4 | -14.7 | -11.2 |

Conclusions

In this contribution, we provide our initial simulation results about BS demodulation requirements for further coverage enhancements.

Reference

R4-2402866, WF on NR_cov_enh2_demod, RAN4#110, China Telecom