3GPP TSG-RAN WG4 Meeting #110b	R4-2405143

# Changsha, China, 15th - 19th Apr, 2024

Title: 	Simulation results on SAN demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	6.16.8.1

Document for:	Discussion

Background

During RAN4#110 meeting, Simulation assumption [1] on demodulation requirements for NR NTN enhancements was approved. In this contribution, we provide our initial simulation results on SAN demodulation requirements for NR NTN enhancements.

Simulation

Above 10GHz bands

PUSCH

Normal PUSCH with CP-OFDM

| Case number | MCS | Antenna configuration | SNR@70% of maximum throughput |
| --- | --- | --- | --- |
| 1 | 2 | 1x1 | -0.2 |
| 2 | 2 | 1x2 | -4.7 |
| 3 | 16 | 1x1 | 12.4 |
| 4 | 16 | 1x2 | 7.3 |
| 5 | 20 | 1x1 | 17.6 |
| 6 | 20 | 1x2 | 11.4 |

Normal PUSCH with DFT-s-OFDM

| Case number | MCS | Antenna configuration | SNR@70% of maximum throughput |
| --- | --- | --- | --- |
| 1 | 2 | 1x1 | 0.1 |
| 2 | 2 | 1x2 | -4.5 |

PUSCH repetition Type A

| Case number | MCS | Antenna configuration | SNR@70% of maximum throughput |
| --- | --- | --- | --- |
| 1 | 2 | 1x1 | -6.8 |
| 2 | 2 | 1x2 | -10.2 |

PUCCH

PUCCH format 0

| Case number | Number of OFDM symbols | Antenna configuration | SNR@1% of ACK missed detection |
| --- | --- | --- | --- |
| 1 | 1 | 1x1 | 16.8 |
| 2 | 1 | 1x2 | 6.2 |
| 3 | 2 | 1x1 | 12.8 |
| 4 | 2 | 1x2 | 3.6 |

PUCCH format 1

| Case number | Number of OFDM symbols | Antenna configuration | SNR |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0.1% of NACK to ACK | 1% of ACK missed detection |
| 1 | 1 | 1x1 | 1.2 | -0.4 |
| 2 | 1 | 1x2 | -5.3 | -6.5 |

PUCCH format 2 (ACK missed detection)

| Case number | Antenna configuration | SNR@1% of ACK missed detection |
| --- | --- | --- |
| 1 | 1x1 | 9.5 |
| 2 | 1x2 | 0.8 |

PUCCH format 2 (UCI BLER)

| Case number | Antenna configuration | SNR@1% of UCI BLER |  |
| --- | --- | --- | --- |
|  |  | 22bit | 16bit |
| 1 | 1x1 | 10.7 | 8.6 |
| 2 | 1x2 | 1.7 | -0.3 |

PUCCH format 3

| Case number | Number of PRBs | Number of symbols | Antenna configuration | SNR@1% of UCI BLER |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Additional DMRS | No additional DM-RS |
| 1 | 1 | 14 | 1x1 | 4.6 | 5.1 |
| 2 | 1 | 14 | 1x2 | -2.0 | -1.4 |
| 3 | 3 | 4 | 1x1 | 12.7 | 12.7 |
| 4 | 3 | 4 | 1x2 | 2.6 | 2.6 |

PUCCH format 4

| Case number | Antenna configuration | SNR@1% of UCI BLER |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Additional DMRS |  | No additional DM-RS |  |
|  |  | 22bit | 16bit | 22bit | 16bit |
| 1 | 1x1 | 14.6 | 12.0 | N/A | N/A |
| 2 | 1x2 | 2.5 | 0.6 | 3.1 | 1.2 |

PRACH

| Case number | Burst format | Antenna configuration | SNR@1% of ACK missed detection |
| --- | --- | --- | --- |
| 1 | B4 | 1x1 | -3.8 |
| 2 | B4 | 1x2 | -10.1 |
| 3 | C2 | 1x1 | 3.3 |
| 4 | C2 | 1x2 | -4.6 |

PUSCH with DMRS bundling

Normal PUSCH with CP-OFDM

| Case number | SCS | Channel bandwidth | PUSCH mapping type | PUSCH aggregation factor | Antenna configuration | SNR@70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | 5MHz | A | n8 | 1x1 | -9.0 |
| 2 | 15kHz | 5MHz | A | n8 | 1x2 | -11.9 |
| 3 | 15kHz | 5MHz | A | n4 | 1x1 | -5.5 |
| 4 | 15kHz | 5MHz | A | n4 | 1x2 | -8.9 |
| 5 | 15kHz | 5MHz | B | n8 | 1x1 | -9.0 |
| 6 | 15kHz | 5MHz | B | n8 | 1x2 | -11.9 |
| 7 | 15kHz | 5MHz | B | n4 | 1x1 | -5.5 |
| 8 | 15kHz | 5MHz | B | n4 | 1x2 | -8.9 |
| 9 | 30kHz | 10MHz | A | n8 | 1x1 | -8.8 |
| 10 | 30kHz | 10MHz | A | n8 | 1x2 | -12.0 |
| 11 | 30kHz | 10MHz | A | n4 | 1x1 | -5.1 |
| 12 | 30kHz | 10MHz | A | n4 | 1x2 | -8.8 |
| 13 | 30kHz | 10MHz | B | n8 | 1x1 | -8.8 |
| 14 | 30kHz | 10MHz | B | n8 | 1x2 | -12.0 |
| 15 | 30kHz | 10MHz | B | n4 | 1x1 | -5.1 |
| 16 | 30kHz | 10MHz | B | n4 | 1x2 | -8.8 |

Proposals

In this contribution, we provide our initial simulation results on SAN demodulation requirements for NR NTN enhancements.

Reference

R4-2401719, Simulation assumption on demodulation requirements for NR NTN enhancements, RAN4#110, Huawei, HiSilicon