3GPP TSG-RAN WG4 Meeting #110b	R4-2405145

# Changsha, China, 15th - 19th Apr, 2024

Title: 	Simulation results on UE demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	6.16.8.2

Document for:	Discussion

Background

During RAN4#110 meeting, Simulation assumption [1] on demodulation requirements for NR NTN enhancements was approved. In this contribution, we provide our initial simulation results on UE demodulation requirements for NR NTN enhancements.

Simulation

PDSCH

| Case number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of HARQ Processes | SNR@70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1x1 | 1x2 |
| 1-1 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-600 | 16 | 3.0 | -2.0 |
| 1-2 | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-600 | 16 | 11.0 | 5.6 |
| 1-3 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-600 | 32 | 3.0 | -2.0 |
| 1-4 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-600 | 16 (4 with feedback disabled) | 3.8 | -1.3 |

PDCCH

| Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleaver size | Shift index | DCI format | Payload (without CRC) | Propagation Condition | SNR@1% of Pm-dsg |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | 1x1 | 1x2 |
| 100 | 120 | 60 | 1 | 2 | Interleaved | 2 | 3 | 0 | 1_0 | 40 | NTN-TDLC5-600 | 16.6 | 6.7 |
| 100 | 120 | 60 | 1 | 4 | Interleaved | 6 | 2 | 0 | 1_1 | 56 | NTN-TDLC5-600 | 13.6 | 3.9 |
| 100 | 120 | 60 | 1 | 8 | Interleaved | 2 | 3 | 0 | 1_1 | 56 | NTN-TDLC5-600 | 12.1 | 3.0 |
| 100 | 120 | 60 | 2 | 16 | Interleaved | 2 | 3 | 0 | 1_0 | 40 | NTN-TDLC5-600 | 8.8 | 0.3 |

Proposals

In this contribution, we provide our initial simulation results on UE demodulation requirements for NR NTN enhancements.

Reference

R4-2401719, Simulation assumption on demodulation requirements for NR NTN enhancements, RAN4#110, Huawei, HiSilicon