3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405156

# Changsha, China, April 15th– 19th, 2024

Title: 	Discussions on for advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	6.11.2.2

Document for:	Discussion

Background

In last meeting, a WF [1] on the receiver for MU-MIMO scenario was approved. This contribution provides our views related to open issues.

- UE feature list

- It was agreed in last meeting that granularity of capability signaling is per UE with the note that “ UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA operation”. However,  the complexity of R-ML in EN-DC, NE-DC and NR-DC operation are similar with CA operation. Therefore, the optional conditions for R-ML should include EN-DC, NE-DC and NR-DC.

- Proposal 1: Modify the note “ UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA operation” to “ UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA, NE-DC, EN-DC and NR-DC operation”.

- It has been agreed to introduce additional 3 bit DCI to assist UE perform R-ML receiver on MU-MIMO scenario,  the presence of 3bit is further indicated by the RRC signaling “advReceiver-MU-MIMO-DCI-1-1”. However, identifying this DCI signaling is one capability needing additional implementation changes, there is potential PDCCH performance degradation if UE doesn’t implement the 3bit DCI identification while BS transmit that.

- Observation 1: BS should be clearly informed that whether UE support decoding PDCCH with additional 3 bits DCI assistant signaling.

- Based on the observation1, we propose the following：

- For the consequence if the feature group 36-1 is not supported by the UE, add following: UE is not capable of decoding PDCCH with DCI bits on presence of the co-scheduled UE information field in DCI format 1_1.

- Proposal 2: For the consequence if the feature group 36-1 is not supported by the UE, add following: UE is not capable of decoding PDCCH with DCI bits on presence of the co-scheduled UE information field in DCI format 1_1.

- Proposal 3: Update the feature 36-1 as following:

- In summary, we propose to update the feature 36-1 as following:

| 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when co-scheduled UE(s)’ modulation order is signaled | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO; UE not capable of decoding PDCCH with DCI bits of the co-scheduled UE information field in DCI format 1_1. | Per UE Note: UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA,NE-DC, EN-DC and NR-DC operation | No | FR1 only | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Test parameters

Test requirements without modulation order blind detection (DCI index 1-5 is indicated)

For case with 2T2R, we have following options:

- For Rank 1+1 with 2T2R, down select among the following cases: - Case#1: Random precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Case#5: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

The only difference between case 1 and case 5 is precoding scheme. Based on our simulations shown in Appendix, the performance gain for FDD is 8.4dB for case#1 and 6.4dB for case#5. Therefore, orthogonal precoding is adequate to verify the R-ML performance given the significant gain. Random precoding seems not be practical in the real deployment. Therefore, case#5 should be selected.

For Rank 2+2 with 4T4R, we have following options:

- For Rank 2+2 with 4T4R, down select among the following cases: - Case#7: Orthogonal precoding, TDLA30-10, ULA Low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Case#8: Orthogonal precoding, TDLA30-10, XP medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE - Case#9: Orthogonal precoding, TDLA30-10, ULA Low, MCS 17 (Table 1) for Target UE, 16QAM for co-UE. full FDRA for the co-UE - Case#10: Orthogonal precoding, TDLA30-10, XP medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE. full FDRA for the co-UE - Orthogonal precoding, TDLA30-10, XP medium, MCS 17 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

The performance gain for FDD is 2.6dB, 3.4dB, 1.3dB, 1.6dB and 3.5dB for case #7, #8,#9,#10 and last case respectively. Based on some BS vendor’s feedback in the last meeting, 64QAM+QPSK is not practical in the real network, so such case should be discarded. Based on the remaining cases, case8 has largest performance gain and should be selected.

Test requirements with modulation order blind detection

For rank 2+2, based on our simulation results, case of MCS13+QPSK, XP medium shows significant performance gain, which could be selected. However, all other companies shows that the gain is quite limited for all cases [2].  Considering the final requirements are defined based on the average results from all companies, which shows limited performance gain. Hence we propose to discard Rank 2+2.

For rank 1+1, all cases with orthogonal PMI show enough performance gain, which should be selected to be more practical. Regarding the MCS, we propose the MCS17+QPSK since MCS13+QPSK has been selected for case without modulation order detection. Meanwhile, for case 1+1, we don’t see the necessity to configure DCI assistant signaling since there is only one modulation order configured.

MCS Table

For MCS table, we have following options:

- Candidate options on the RRC assistant information configuration on the MCS table: - For UEs not supporting modulation order blind detection: - Option 1: No need for the network to inform such information to the UE - Option 2: Should be presented regardless of whether the UE supports MO BD - Option 2A: ‘64QAM MCS Table’ - For UEs supporting modulation order blind detection: - Option 1: RRC-based assistant signalling on MCS Table should be ‘256QAM MCS Table’ - Option 2: Align with the MCS Table configuration in the test

Our understanding is that for cases without MO detection, there is no necessity to enable RRC assistant signalling on MCS table since UE is not expected to perform MO detection.

For cases with MO detection, it is proposed to configure “256QAM MCS Table “ since 256QAM Table is widely used in real network to improve the system throughput.

Proposal 4: RAN4 to consider following test setup for cases without modulation order detection.

For Rank 1+1: TDLC300-100, Orthogonal PMI selection, 2T2R ULA medium, MCS13(Target UE) +QPSK(Co-UE)

For Rank 2+2: TDLA30-10, Orthogonal PMI selection, 4T4R, XPL medium, MCS13(Target UE) +QPSK(Co-UE)

DCI index 1 is configured and RRC assistant signaling on MCS table is disabled.

Proposal 5: RAN4 to consider following test setup for cases with modulation order detection.

For Rank 1+1: TDLC300-100, Orthogonal PMI selection, 2T2R/2T4R ULA medium, MCS17(Target UE) +16QAM(Co-UE)

DCI assistant signaling is disabled and RRC assistant signaling on MCS table indicates “256QAM Table”.

Test applicability rules

Based on our understanding, R-ML with MO detection is more complex than that without MO detection, so if UE passes the Rank 1+1 cases with MO detection, UE can skip the Rank 1+1 cases without MO detection.

Proposal 6: Introduce the test applicability rules that if UE passes the Rank 1+1 cases with MO detection, UE can skip the Rank 1+1 cases without MO detection.

- Reference

[1]    R4-2402864 WF on the advanced receiver for MU-MIMO scenario. China Telcom.

[2]    R4-2400805 Phase II simulation result collection for advanced receiver for MU-MIMO

[3]    R4-2403842 Rel-18 RAN4 UE feature list for NR. CMCC

- Conclusion

- Proposal 1: Modify the note “ UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA operation” to “ UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA, NE-DC, EN-DC and NR-DC operation”.

- Observation 1: BS should be clearly informed that whether UE support decoding PDCCH with additional 3 bits DCI assistant signaling.

- Proposal 2: For the consequence if the feature group 36-1 is not supported by the UE, add following: UE is not capable of decoding PDCCH with DCI bits on presence of the co-scheduled UE information field in DCI format 1_1.

- Proposal 3: Update the feature 36-1 as following:

| 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when co-scheduled UE(s)’ modulation order is signaled | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO; UE not capable of decoding PDCCH with DCI bits of the co-scheduled UE information field in DCI format 1_1. | Per UE Note: UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA,NE-DC, EN-DC and NR-DC operation | No | FR1 only | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: RAN4 to consider following test setup for cases without modulation order detection.

For Rank 1+1: TDLC300-100, Orthogonal PMI selection, 2T2R ULA medium, MCS13(Target UE) +QPSK(Co-UE)

For Rank 2+2: TDLA30-10, Orthogonal PMI selection, 4T4R, XPL medium, MCS13(Target UE) +QPSK(Co-UE)

DCI index 1 is configured and RRC assistant signaling on MCS table is disabled.

Proposal 5: RAN4 to consider following test setup for cases with modulation order detection.

For Rank 1+1: TDLC300-100, Orthogonal PMI selection, 2T2R/2T4R ULA medium, MCS17(Target UE) +16QAM(Co-UE)

DCI assistant signaling is disabled and RRC assistant signaling on MCS table indicates “256QAM Table”.

Proposal 6: Introduce the test applicability rules that if UE passes the Rank 1+1 cases with MO detection, UE can skip the Rank 1+1 cases without MO detection.

- Appendix

Table 6-1: Summary of simulation results without modulation order detection for FDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) | MMSE-IRC SNR@70% maxTP(dB) | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 13.7 | 22.1 | 8.4 |
|  | 2T4R |  |  |  |  |  | 12.9 | 25.0 | 12.1 |
|  | 2T2R | Orthogonal |  |  |  |  | 13.0 | 19.4 | 6.4 |
|  | 2T4R |  |  |  |  |  | 12.1 | 20.5 | 8.4 |
| 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 9.5 | 12.1 | 2.6 |
|  |  |  |  | XP medium |  |  | 10.5 | 13.9 | 3.4 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.1 | 16.4 | 1.3 |
|  |  |  |  | XP medium |  |  | 16.8 | 18.4 | 1.6 |
|  |  |  |  |  |  | QPSK | 14.8 | 18.3 | 3.5 |

Table 6-2: Summary of simulation results without modulation order detection for TDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) | MMSE-IRC SNR@70% maxTP(dB) | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 14.3 | 23.7 | 9.4 |
|  | 2T4R |  |  |  |  |  | 13.6 | 27.7 | 14.1 |
|  | 2T2R | Orthogonal |  |  |  |  | 13.4 | 20.4 | 7.0 |
|  | 2T4R |  |  |  |  |  | 12.5 | 22.3 | 9.8 |
| 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 9.6 | 12.3 | 2.7 |
|  |  |  |  | XP medium |  |  | 10.6 | 14.1 | 3.5 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.4 | 16.6 | 1.2 |
|  |  |  |  | XP medium |  |  | 17.0 | 18.7 | 1.7 |
|  |  |  |  |  |  | QPSK | 15.2 | 18.5 | 3.3 |

Table 6-3: Summary of simulation results for case with modulation order estimation for FDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR(dB)@70% max TP | MMSE-IRC SNR(dB)@70% max TP | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 14.5 | 22.1 | 7.6 |
|  |  |  |  | ULA Low |  |  | 11.2 | 15.0 | 3.8 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.0 | 7.4 | 1.4 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 14.6 | 25.0 | 10.4 |
|  | 2T2R |  | TDLC300-100 | ULA medium | 17 | 16QAM | 22.7 | N/A | N/A |
|  |  |  |  | ULA Low |  |  | 17.7 | 24.3 | 6.6 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 10.8 | 10.8 | 0.0 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 22.7 | N/A | N/A |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 13.6 | 19.4 | 5.8 |
|  | 2T4R |  |  |  |  |  | 13.5 | 20.5 | 7.0 |
|  | 2T2R |  |  |  | 17 | 16QAM | 20.9 | 26.1 | 5.2 |
|  | 2T4R |  |  |  |  |  | 20.9 | 27.9 | 7.0 |
| 2+2 | 4T4R |  | TDLA30-10 | ULA Low | 13 | QPSK | 10.6 | 12.1 | 1.5 |
|  |  |  |  | XP medium |  |  | 10.7 | 13.9 | 3.2 |

Table 6-4: Summary of simulation results for case with modulation order estimation for TDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR(dB)@70% max TP | MMSE-IRC SNR(dB)@70% max TP | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 15.2 | 23.7 | 8.5 |
|  |  |  |  | ULA Low |  |  | 11.5 | 16.3 | 4.8 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.1 | 7.6 | 1.5 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 15.9 | 27.7 | 11.8 |
|  | 2T2R |  | TDLC300-100 | ULA medium | 17 | 16QAM | 24.4 | N/A | N/A |
|  |  |  |  | ULA Low |  |  | 18.4 | N/A | N/A |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 11.1 | 11.0 | -0.1 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 25.4 | N/A | N/A |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 14.1 | 20.4 | 6.3 |
|  | 2T4R |  |  |  |  |  | 14.3 | 22.3 | 8.0 |
|  | 2T2R |  |  |  | 17 | 16QAM | 22.1 | 28.8 | 6.7 |
|  | 2T4R |  |  |  |  |  | 22.5 | 32.4 | 9.9 |
| 2+2 | 4T4R |  | TDLA30-10 | ULA Low | 13 | QPSK | 10.6 | 12.3 | 1.7 |
|  |  |  |  | XP medium |  |  | 10.6 | 14.1 | 3.5 |