3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405166

# Changsha, China, April 15th – 19th , 2024

Title: 	Discussion on remaining issues for UE demod for less than 5MHz

Source: 	Huawei, HiSilicon

Agenda item:	6.8.7.1

Document for:	Discussion

Background

In last meeting, WF [1] on FR1 less than 5MHz demodulation requirements was agreed and some open issues are left. This contribution provide our views on remaining issues.

- Discussions

One issue is whether to define PDCCH requirements in HST conditions:

Issue 1-3-3: PDCCH requirements in HST conditions Way forward: Further discussion is needed: - Option 1: Introduce PDCCH requirements at 3MHz CBW in HST conditions. - Option 2: Not to introduce HST scenario for PDCCH requirements.

PDCCH has dense DMRS symbols in time domain, low modulation order and low code-rate, which means performance can be guaranteed in high Doppler scenario, we don’t see the necessity to consider such case.

Proposal 1: Not introduce PDCCH requirements at 3MHz CBW in HST conditions.

The other issue is the number of Tx, all existing PDCCH test cases cover both 1Tx and 2Tx , we slightly keep consistency with existing PDCCH cases and cover both 1Tx and 2Tx.

Proposal 2: Cover both 1Tx and 2Tx for punctured PDCCH requirements.

- Conclusion

This contribution provides the simulation results for punctured PDCCH requirements. The proposals are:

Proposal 1: Not introduce PDCCH requirements at 3MHz CBW in HST conditions.

Proposal 2: Cover both 1Tx and 2Tx for punctured PDCCH requirements.

- Reference

[1]    R4-2402863 WF on NR_FR1_lessthan_5MHz_BW_demod. Nokia, Nokia Shanghai Bell.