# 3GPP TSG-RAN WG4 Meeting #110bis													R4-2405180
Changsha, China, 15th – 19th April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38101-1 | CR |  | rev |  | Current version: | V18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38101-1 Clarification of MIMO and TxD capability for PC3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | R18_3Tx_NR_CA_ENDC-Core |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no information about which band in the band combination supports UL MIMO or TxD for PC3 while it is clarified for other power classes. It causes questions on how PC3 is supported in 3Tx. For the PC2/PC1.5 band combination requested it is clear on the statement of which band is in UL MIMO and which band is in single Tx. And PC3 should follow same MIMO capability as PC2/PC1.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add notes to clarify the MIMO/TxD capability for the 3Tx band combinations in PC3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is still unclear on which band supports UL MIMO/TxD in PC3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.3.1, 6.2L.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This draft CR is based on ageed CR R4-2401790 in RAN4#110 meeting. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES >

6.2H.3.1	UE maximum output power for inter-band UL CA with UL MIMO

For inter-band UL CA with UL MIMO in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2H.3.1-1. The period of measurement shall be at least one sub frame (1 ms). The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2 and 6.2D.1-3 for 2-layer configuration and ULFPTx configuration respectively for the component carrier configured with UL MIMO.

Table 6.2H.3.1-1: UE Power Class for inter-band UL CA with UL MIMO in one frequency band

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23x | +2/-3 |
| CA_n5A-n77A | 293 | +2/-3 |  |  | 23x | +2/-3 |
| CA_n8A-n78A |  |  | 262 | +2/-3 | 23x | +2/-3 |
| CA_n25A-n41A | 293 | +2/-3 | 262 | +2/-3 | 23x | +2/-3 |
| CA_n25A-n77A | 293 | +2/-3 |  |  | 23x | +2/-3 |
| CA_n26A-n78A |  |  | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n28A-n41A |  |  | 262 | +2/-3 | 23x | +2/-3 |
| CA_n28A-n78A |  |  | 262 | +2/-3 | 23x | +2/-3 |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23x | +2/-3 |
| CA_n41A-n71A | 293 | +2/-3 | 262 | +2/-3 | 23x | +2/-3 |
| CA_n41A-n77A |  |  | 264 | +2/-3 | 23y | +2/-3 |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. NOTE 5: Power class 3 is default power class unless otherwise stated. NOTE 6: FWA form factor is targeted unless otherwise stated. NOTE x: The UE supports single Tx in FDD band and UL MIMO in TDD band. NOTE y: The UE supports UL MIMO in either one of the TDD bands. |  |  |  |  |  |  |

<Next change >

6.2L.3.1	UE maximum output power for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2L.3.1-1. The period of measurement shall be at least one sub frame (1 ms).

Table 6.2L.3.1-1: UE Power Class for inter-band UL CA with Tx Diversity in one frequency band

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n5A-n77A | 293 | +2/-3 |  |  | 23 x | +2/-3 |
| CA_n8A-n78A |  |  | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n25A-n41A | 293 | +2/-3 | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n25A-n77A | 293 | +2/-3 |  |  | 23 x | +2/-3 |
| CA_n26A-n78A |  |  | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n28A-n41A |  |  | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n28A-n78A |  |  | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n41A-n71A | 293 | +2/-3 | 262 | +2/-3 | 23 x | +2/-3 |
| CA_n41A-n77A |  |  | 26y | +2/-3 | 23 y | +2/-3 |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. NOTE 4: Power class 3 is the default power class unless otherwise stated. NOTE 5: FWA form factor is targeted unless otherwise stated. NOTE x: The UE supports single Tx in FDD band and Tx Diversity in TDD band. NOTE y: The UE supports Tx Diversity in either one of the TDD bands. |  |  |  |  |  |  |

<END OF CHANGES >