# 3GPP TSG-RAN WG4 Meeting # 110bis                                 R4-2405204

# Changsha, China, 15 April – 19 April, 2024

# Title: 	Simulation results for MU-MIMO with R-ML receiver

# Source: 	ZTE Corporation

# Agenda item:	6.11.2.2

# Document for:	Discussion

# 1 Introduction

- In RAN4#110 meeting, some conclusions has been reached for advanced receiver and a WF was agreed[1]. In this paper, we will provide our simulation results for MU-MIMO.

# 2 Discussion

# 2.1 Simulation assumptions

- The simulation assumptions are summarized in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 4,13,17 | Rank 1+1: QPSK Rank 2+2: QPSK, 16QAM and 64QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |
| MIMO configuration |  |  | For Rank 1+1: 2T2R ULA Medium For Rank 2+2: 4T4R ULA Low |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |
| HARQ process number |  |  | 4 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - Option 1: Select the precoding matrix to ensure orthogonality with target UE - Option 2: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100,TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC,E-MMSE-IRC, R-ML | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

  - 2.2 Simulation results

## 2.2.1 Without Blind detection

In this section, we provide the summary of link-level analysis of the co-schedule UE(s) without modulation order blind detection performance and advanced receiver throughput performance r in case of all information of co-schedule UE is known to target UE . The simulation results for the normalized throughput are illustrated below:

### 2.2.1.1 FDD

Table 2-2: Summary of simulation results without modulation order detection for FDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 12.8 |
|  |  |  |  | ULA Low |  |  | 9.6 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 5.1 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 15.7 |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium |  |  | 13.2 |
|  | 2T4R | Orthogonal | TDLC300-100 | ULA medium |  |  | 13.7 |
| 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 10.3 |
|  |  |  |  | XP medium |  |  |  |
|  |  |  |  | ULA Low | 17 | 16QAM | 16.3 |
|  |  |  |  | XP medium |  |  |  |

### 2.2.1.2 TDD

Table 2-3: Summary of simulation results without modulation order detection for TDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 13.6 |
|  |  |  |  | ULA Low |  |  | 10.3 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 5.2 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 16.4 |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium |  |  | 14.6 |
|  | 2T4R | Orthogonal | TDLC300-100 | ULA medium |  |  | 14.9 |
| 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 11.0 |
|  |  |  |  | XP medium |  |  |  |
|  |  |  |  | ULA Low | 17 | 16QAM | 17.1 |
|  |  |  |  | XP medium |  |  |  |

## 2.2.2 Blind detection

In this section, we provide the summary of link-level analysis of the co-schedule UE(s) modulation order blind detection performance and advanced receiver throughput performance. The simulation results for the normalized throughput are illustrated below:

### 2.2.1.1 FDD

Table 2-4: Summary of simulation results for case with modulation order estimation for FDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR(dB)@70% max TP | MMSE-IRC SNR(dB)@70% max TP | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 12.7 | 22.4 | 9.7 |
|  |  |  |  | ULA Low |  |  | 9.7 | 17.0 | 7.3 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.6 | 7.4 | 0.8 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 14.2 |  |  |
|  | 2T2R |  | TDLC300-100 | ULA medium | 17 | 16QAM | 23.8 | NA | Inf |
|  |  |  |  | ULA Low |  |  | 17.1 | 20.3 | 3.2 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 10.7 | 10.8 | 0.1 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 24.1 | NA | Inf |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 13.3 | 20.7 | 7.4 |
|  | 2T4R |  |  |  |  |  | 13.5 | 22.4 |  |
|  | 2T2R |  |  |  | 17 | 16QAM | 20.8 | 26.3 | 5.5 |
|  | 2T4R |  |  |  |  |  | 24.8 | 32.8 | 8.0 |

### 2.2.1.2 TDD

Table 2-5: Summary of simulation results for case with modulation order estimation for TDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR(dB)@70% max TP | MMSE-IRC SNR(dB)@70% max TP | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 13.6 | 23.1 | 9.5 |
|  |  |  |  | ULA Low |  |  | 10.9 | 19.3 | 8.4 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.8 | 7.9 | 1.1 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 16.5 |  |  |
|  | 2T2R |  | TDLC300-100 | ULA medium | 17 | 16QAM | 24.5 | NA | Inf |
|  |  |  |  | ULA Low |  |  | 17.7 | 21.2 |  |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 10.1 | 11.1 | 1.0 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 24.9 | NA | Inf |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 14.6 | 22.5 | 7.9 |
|  | 2T4R |  |  |  |  |  | 14.9 |  |  |
|  | 2T2R |  |  |  | 17 | 16QAM | 22.8 | 27.1 | 4.3 |
|  | 2T4R |  |  |  |  |  | 25.6 | 35.3 | 9.7 |

# 3 Conclusion

In this contribution, we give some simulation results on demodulation performance requirements for MU-MIMO.

# 4 References

- R4-2402864. WF on the advanced receiver for MU-MIMO scenario. China Telecom, RAN4#110 meeting.