3GPP TSG-RAN WG4 Meeting # 110bis                                R4-2405220

Changsha, China, 15th-19th April, 2024

Agenda item:	  6.14.2.2

Source: 	ZTE Corporation

Title: 	Discussion on performance requirements for NR mobility enhancements

Document for:	Approval

Introduction

The RRM core requirement on Mobility work item has been summarized, which include

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- CHO including target MCG and target SCG in NR-DC

- CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC

- To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay

This contribution provides our view on performance requirements for other RRM performance requirements, including RRM performance requirements for NR-DC with selective activation of cell groups via L3 enhancements, improvement on SCell/SCG setup delay and enhanced CHO configurations.

Discussion

In RAN4#110 meeting, we have reached consensus to define test cases for subsequent CPAC and enhanced CHO configurations. We can discuss details based on draftCR directly. However, there is no agreement on whether define test case for Improvement on SCell/SCG setup delay, and it should be further discussed based on WF [1].

Issue 2-3: Whether define test case for Improvement on SCell/SCG setup delay FFS the following options: - Option 1: do not define test case (QC) - Msg1 transmission occasion is unknown to TE. RSRP variation from X sec before T1 based test design is not definable. - If test is for validating UE not to report, TE does not differentiate wrong UE who is doing nothing and good UE who is performing validity check. Both good and wrong UE will pass by not reporting. - Option 2: define test case

For the solution based on existing measurements, it was introduced the validity check for measurement results obtained before Msg1 transmission and UE is just allowed to report vaild results to NW. For this new functionality, we prefer to define the corresponding test procedure to verify whether UE can correctly perform the validity check and the reported results are within the last [X] sec before msg1 transmission for RRC resume/setup request.

Proposal 1: RAN4 to define test case on existing measurement for verifying the procedure on validity check.

To verify UE execute “validity check” correctly, we can set different signal level for different time period and TE can tell from the reported signal level whether the measurements are older than X, such as mentained in R4-2401346. When the UE is paged for connection setup and requested by the network to send idle mode measurements, the correct UE behaviour is that NO valid measurement result is reported. If RSRP0 is reported, it means that UE doesn’t execute validity check.

Proposal 2: To verify UE execute “validity check” correctly, different signal level can be set for different time period and TE can tell from the reported signal level whether the measurements are older than X

In our understanding, when the UE reports measurements from idle/inactive mode, the reported measurement results need to fulfil the measurement accuracy requirements at the measurement instance. Meanwhile, the existing measurements include EMR and cell-reselection measurement results. Therefore, test case for verifying measurement accuracy of the idle/inactive mode measurements reported by the UE are needed, and the Rel-16 EMR configuration can be used as a baseline.

- Proposal 3: Define test case for verifying measurement accuracy of the idle/inactive mode measurements reported by the UE.

Conclusions

In this contribution, we put forward the following proposal on performance part for mobility enhancements.

Proposal 1:  RAN4 to define test case on existing measurement for verifying the procedure on validity check.

Proposal 2: To verify UE execute “validity check” correctly, different signal level can be set for different time period and TE can tell from the reported signal level whether the measurements are older than X

Proposal 3: Define test cases for verifying measurement accuracy of the idle/inactive mode measurements reported by the UE.

References

- R4-2403445, WF on RRM performance requirements of R18 Further NR mobility enhancement, Apple

- R4-2401346, Discussion on performance requirements for selective activation of cell groups via L3 enhancements, Improvement on SCell/SCG setup delay and Enhanced CHO, Huawei, HiSilicon