3GPP TSG-RAN WG4 Meeting # 110bis											R4-2405271

Changsha, China, 14 ‒ 19 April 2024

Agenda item:			6.8.8

Source:	Man Hung Ng (Nokia)

Title:	Topic summary for [110bis][118] NR_FR1_lessthan_5MHz_BW_R18

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Summary for contributions submitted under agenda items 6.8.1 and 6.8.2 for NR support for dedicated spectrum less than 5MHz for FR1.

List of candidate target of discussion for 1st round and 2nd round:

- 1st round: Discussion and agreement on open issues listed below.

- 2nd round: Continue discussion and agreement on open issues listed below.

# Topic #1: System parameters

Main technical topic overview. The structure can be done based on sub-agenda basis.

- R4-2405765 and R4-2405766 are moved to agenda item 9.10.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2404284 | Nokia | Proposal 1: Observation 1: There is no need to reserve a GSCN value in the RAN4 specifications to be used in the legacy list, as this is unlikely to be required in actual deployments and would cause the same issue on inter-frequency neighbour cells measurement to legacy UE. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1: Discussion paper in R4-2404284

Observation 1: There is no need to reserve a GSCN value in the RAN4 specifications to be used in the legacy list, as this is unlikely to be required in actual deployments and would cause the same issue on inter-frequency neighbour cells measurement to legacy UE.

- Proposals

  - Option 1: Note R4-2404284.

  - Option 2: Further discuss R4-2404284.

- Recommended WF

  - Option 1: Note R4-2400284 and wait for RAN2 discussion.

# Topic #2: UE RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2405066 | Rakuten Mobile, Inc | Proposal 1: Adds Asymmetric Channel BW combinations for n28. Observation 1: To introduce asymmetric UL DL channel BW combnations for n28 that is required to support 3Mhz Uplink with 5Mhz Downlink. |
| R4-2405067 | Rakuten Mobile, Inc | Proposal 1: Firstly, we propose that there is a need to define additional emission requirements for NR n28 3Mhz according to the latest addition of NS_17 and corrosponding emission requirements for B28 3Mhz in LTE. Proposal 2: Secondly, asymmetric bandwidth needs to be defined for frequency bands less than 5Mhz (for Rakuten n28 ). Proposal 3: Thirdly, a new feature capability would be needed for indicating support of asymmetric bandwidth for less than 5Mhz, its proposed to send LS to RAN2 as soon as Draft CR is agreed by RAN4. |
| R4-2405658 | Nokia, Skyworks Solutions Inc., Rakuten Mobile, Inc | Proposal 1: It is proposed to add 3 MHz channel bandwidth for NS_17 in the NR spec. Observation 1: NS_17 emission requirement can be fulfilled without A-MPR or RB restriction for NR band n28 3 MHz operation in Japan. |
| R4-2405659 | Nokia, Skyworks Solutions Inc., Rakuten Mobile, Inc | Proposal 1: 3 MHz channel bandwidth for NS_17 is only specified at 715-718 MHz. Observation 1: NS_17 for 3 MHz channel bandwidth in band n28 operation for Japan is not properly specified. It needs to be aligned with E-UTRA requirement, i.e., 3 MHz channel bandwidth for uplink only at 715-718 MHz without A-MPR. |
| R4-2405715 | Qualcomm Inc. | Proposal 1: No A-MPR is needed to support 3 MHz in band n28 at 715-718 MHz with NS_17 requirements. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1: Proposal 1 in R4-2405067 and A-MPR studies in R4-2405658 and R4-2405715

R4-2405067 Proposal 1: Firstly, we propose that there is a need to define additional emission requirements for NR n28 3Mhz according to the latest addition of NS_17 and corrosponding emission requirements for B28 3Mhz in LTE. R4-2405658 Proposal 1: It is proposed to add 3 MHz channel bandwidth for NS_17 in the NR spec. Observation 1: NS_17 emission requirement can be fulfilled without A-MPR or RB restriction for NR band n28 3 MHz operation in Japan. R4-2405715 Proposal 1: No A-MPR is needed to support 3 MHz in band n28 at 715-718 MHz with NS_17 requirements.

- Proposals

  - Option 1: No A-MPR or RB restriction is needed to support 3 MHz in band n28 at 715-718 MHz with NS_17 requirements.

  - Option 2: Further study on A-MPR or RB restriction to support 3 MHz in band n28 at 715-718 MHz with NS_17 requirements.

- Recommended WF

  - Option 1: No A-MPR or RB restriction is needed to support 3 MHz in band n28 at 715-718 MHz with NS_17 requirements.

### Sub-topic 2-2

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-2: Proposal 2 in R4-2405067 and draft CR in R4-2405066

R4-2405067 Proposal 2: Secondly, asymmetric bandwidth needs to be defined for frequency bands less than 5Mhz (for Rakuten n28 ). R4-2405066 Proposal 1: Adds Asymmetric Channel BW combinations for n28. Observation 1: To introduce asymmetric UL DL channel BW combnations for n28 that is required to support 3Mhz Uplink with 5Mhz Downlink.

- Proposals

  - Option 1: Agree proposal 2 in R4-2405067 and endorse draft CR in R4-2405066.

  - Option 2: Agree proposal 2 in R4-2405067 and revise draft CR in R4-2405066.

  - Option 3: Note proposal 2 in R4-2405067 and note draft CR in R4-2405066.

- Recommended WF

  - Option 1: Agree proposal 2 in R4-2405067 and endorse draft CR in R4-2405066.

### Sub-topic 2-3

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-3: Draft CR in R4-2405659

R4-2405659 Proposal 1: 3 MHz channel bandwidth for NS_17 is only specified at 715-718 MHz. Observation 1: NS_17 for 3 MHz channel bandwidth in band n28 operation for Japan is not properly specified. It needs to be aligned with E-UTRA requirement, i.e., 3 MHz channel bandwidth for uplink only at 715-718 MHz without A-MPR.

- Proposals

  - Option 1: Endorse draft CR in R4-2405659.

  - Option 2: Revise draft CR in R4-2405659.

  - Option 3: Note draft CR in R4-2405659.

- Recommended WF

  - Option 1: Endorse draft CR in R4-2405659.

### Sub-topic 2-4

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-4: Proposal 3 in R4-2405067

R4-2405067 Proposal 3: Thirdly, a new feature capability would be needed for indicating support of asymmetric bandwidth for less than 5Mhz, its proposed to send LS to RAN2 as soon as Draft CR is agreed by RAN4.

- Proposals

  - Option 1: Agree proposal 3 in R4-2405067 and send LS to RAN2.

  - Option 2: Note proposal 3 in R4-2405067.

- Recommended WF

  - Option 1: Agree proposal 3 in R4-2405067 and send LS to RAN2.

…