3GPP TSG-RAN WG4 Meeting #110bis	R4-2405299

# Changsha, China, Apr. 15 – Apr. 19, 2024

Source: 	Huawei, HiSilicon

Title: 	General discussion on A-IoT coexistence scenarios

Agenda Item:	9.13.2

Document for:	Discussion

Introduction

The new SID on solutions for Ambient IoT was approved at the RAN#102 meeting [1] and later revised [2] at the RAN#103 meeting. The RAN4 relative objectives in the revised SID [2] are copied as below:

General Scope

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope:

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

- X is to be decided in WGs.

- Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

- For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ.

NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement.

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

- FR1 licensed spectrum in FDD.

- Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

…

Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum.

The following objectives are set, within the General Scope:

- Evaluation assumptions

- …

- Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4]

…

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848.

…

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

RAN1 had made some progress in RAN1#116. In this paper we discuss the potential RAN4 coexistence scenarios for Ambient IoT based on the SID and RAN1#116 progress.

Discussion

In this discussion, we will go through factors related to coexistence scenarios in the SID one by one. Some of these factors are still being discussed by RAN1 and are not yet stable. However, we aim to provide a nearly complete overview of RAN4 coexistence scenarios for Ambient IoT.

- 2.1 Device type

According to SID (RP-240826), devices are categorized based on their peak power consumption levels into two types: one with a peak power consumption of ~1 µW, and the other with a peak power consumption of ≤ a few hundred µW. During RAN1#116, the device with a peak power consumption of around 1 µW was named Device 1, while the device with a peak power consumption of ≤a few hundred µW was further divided into Device 2a and Device 2b. Device 2a’s UL transmission is backscattered on a carrier wave provided externally, while Device 2b generates its UL transmission internally.

The following are copied from the Chair's notes RAN1#116 9.4 R19 Ambient IoT v07 [3]:

Agreement

For the purpose of the study, RAN1 uses the following terminologies:

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

To align with RAN1 terminology, we have three types of Ambient IoT devices to consider: Device 1, Device 2a, and Device 2b.

Device 1 and Device 2a share similar coexistence scenarios, as their UL transmissions are backscattered on an externally provided carrier wave. Device 2a has DL and/or UL amplification to enhance receiver sensitivity and/or increase backscattered signal power, which sets it apart from Device 1 that lacks an amplifier. Additionally, Device 2a may have a frequency shifter for backscatter modulation, which moves the backscattered signal tens of MHz away from the external carrier-wave frequency. This can help reduce interference to the uplink receiver. To make things easier, for Device 2a，it is recommend to use Device 2a without frequency shifter as the coexistence baseline since it experiences more severe coexistence issues.

Device 2b generates its UL transmission internally, independent of backscatter. As a result, its coexistence scenarios differ from that of device 1 and device 2a, and should be treated separately.

Proposal 1: Take Device 2a without a frequency shifter as the baseline for Device 2a coexistence analysis, because Device 2a without a frequency shifter brings more severe carrier-wave interference to the uplink receiver compared to the one with a frequency shifter.

Observation 1: Device 1 and Device 2a share similar coexistence scenarios, as their UL transmissions are backscattered on an externally provided carrier wave.

Proposal 2: Device 2b behaves differently from Device 1 and Device 2a, and requires separate treatment on co-existence scenarios.

- 2.2 Deployment-topology combinations

Two combinations of deployment -topology are included in the SID:

B. Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

In the TR 38.848[4]，characteristics of deployment scenario 1 are as follows：

With Ambient IoT device indoors and basestation indoors, this deployment scenario is characterized according to Table 4.2.2.1-1.

Table 4.2.2.1-1 in [4]: Characteristics of deployment scenario 1

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico-cell |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

NOTE 1:	Descriptions may not be applicable for some Devices (A, B).

Deployment scenario 1 with Topology 1 (D1T1 in short for convenience) is illustrated in Figure 1. Both the Ambient devices and the Ambient IoT micro-BS are located indoors. The Ambient IoT BS communicates directly with the device.

Figure 1: Deployment scenario 1 with Topology 1

In the TR 38.848[4]，characteristics of deployment scenario 2 are as follows：

With Ambient IoT device indoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.2-1.

Table 4.2.2.2-1: Characteristics of deployment scenario 2

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1), (2), (3) Note: The location of intermediate or assisting node (if any) is indoor or outdoor |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device C may support Topology (1), (2), (3), Device A may support Topology (2), Device B may support Topology (2), (3) |

NOTE 1:	Descriptions may not be applicable for some Devices (A, B).

Figure 2 illustrates Deployment Scenario 2 with Topology 2 (D2T2 in short for convenience). The NR macro base station is positioned outdoors while the Ambient devices are placed indoors. The NR macro base station does not directly communicate with the device but rather controls the indoor intermediate node UE, which in turn communicates with the indoor device.

Figure 2: Deployment scenario 2 with Topology 2

- 2.3 Meaning of co-site

The meaning of 'co-site' in the SID is not very clear. In Table 4.2.0-1 of TR 38.848, it refers to being deployed on the same site as an existing 3GPP deployment with the same base station type. The opposite of this is being deployed on new sites without assuming the existence of a 3GPP deployment.

Table 4.2.0-1: Characteristics of deployment scenarios

| Characteristic | Possible description entries |
| --- | --- |
| … | … |
| Coexistence with existing 3GPP technologies | Deployed on the same sites as an existing 3GPP deployment corresponding to the base station type. Deployed on new sites without an assumption of an existing 3GPP deployment. |
| … | … |

In the RAN1 [5], It is proposed that whether the co-located Ambient IoT BS and micro-cell NR BS at the same points (site) work in same band is up to RAN4 to identify coexistence scenarios. At least there could be the following case for co-site:

For example, at least there could be the following case for co-site:

- Indoor micro-cell Ambient IoT BS and indoor micro-cell NR BS are deployed in the same points (sites) in the layout, while Ambient IoT is operating at 900 MHz but NR uses C-band. In this case, the coexistence is mainly between the indoor micro-cell 900 MHz Ambient IoT with the outdoor macro-cell 900MHz NR.

From RAN4’s perspective, take Deployment scenario 1 with Topology 1 as an example, co-site can refer to two scenarios:

1) D1T1 co-site case 1: deploying different frequency bands for NR and Ambient IoT on the same site. In this case, Ambient IoT is operating at band n8 while NR is operating at C-band, for example.

Figure 3 D1T1 co-site case (1): deploying different frequency bands for NR and Ambient IoT on the same site

（Note: The picture shows that A-IoT micro-BS and NR micro-BS are separate for clarity, but they are actually in the same site.）

For this case，as shown in the Figure 3, indoor A-IoT micro- BS use the in-band or guard-band spectrum of an outdoor NR macro-cell. The coexistence is mainly between the indoor micro-cell band n8 Ambient IoT with the outdoor macro-cell band n8 NR as Figure 4. As NR is deployed in the different frequency bands with Ambient IoT, it's reasonable to assume that there aren't any indoor NR UEs operating in the band n8.

Figure 4 D1T1 co-site case (1): n8 NR indoor A-IoT micro- BS use the in-band or guard-band spectrum of an outdoor NR macro-cell（Note: The picture shows that A-IoT micro-BS and NR micro-BS are separate for clarity, but actually they are in the same site.）

2) deploying same frequency band for NR and Ambient IoT on the same site. For instance, both n8 Ambient IoT micro-BS and n8 NR micro-BS are on the same site. As shown in the Figure 5, indoor A-IoT micro- BS concurrent with NR.

Figure 5 D1T1 co-site case (2): deploying same frequency band for NR and Ambient IoT on the same site

（Note: The image may suggest that the A-IoT micro-BS and NR micro-BS are one and the same, but in reality, they can be distinct and situated in the same location.）

The Figure 4 may suggest that the A-IoT micro-BS and NR micro-BS are the same, but in reality, they can be distinct and located in the same site. The difference between the two is that their isolation may vary. There is a concern that deploying NR and A-IoT on the same site using the same frequency band may result in problematic interference issues. In this scenario, there may be a significant sensitivity difference between A-IoT devices (specifically Device 1 and Device 2a) and n8 NR UEs. For example, Ambient IoT Device 1 receiver sensitivity is -36dBm, while n8 NR UE sensitivity is -97dBm. Interference between indoor NR UEs and indoor A-IoT micro-BS may be problematic due to mutual blocking. When the interference source is the A-IoT micro-BS transmitter, and the victim is the indoor NR UE receiver, the interference signal from A-IoT micro-BS transmitter can reach levels about -36dBm.However, the blocking level for the NR UE receiver is -56dBm, which means that some NR UE receivers may be blocked by the A-IoT micro-BS transmitting signal. On the other hand, a few NR UE receivers that receive strong NR signals and are not blocked by the A-IoT may interfere with the A-IoT micro-BS receiver.

It is currently better to focus on co-site case (1), which involves deploying NR and Ambient IoT on the same site using the different frequency bands.

To ensure smooth co-existence, it is recommended deploying different frequency bands at the same site. It is also recommended to avoid deploying the same frequency band for both NR and Ambient IoT at the same site, unless it is absolutely necessary.

Proposal 3: To minimize the chance of interference, it is recommended to use D1T1 co-site case (1) as a coexistence start point, i.e. deploying different frequency bands for NR and Ambient IoT on the same site. D1T1 co-site case (2) can be considered later.

- 2.4 Spectrum usage

The SID describes that Ambient IoT uses FR1 licensed spectrum in FDD.

We believe that the assumption underlying FDD licensed spectrum is that base stations transmit on the downlink spectrum and receive on the uplink spectrum, while intermediate node UE transmits on the uplink spectrum and receives on the downlink spectrum. This ensures compliance with spectrum usage conventions and makes it easier to meet regulatory requirements.

Proposal 4: To meet regulations, in D1T1, the Ambient IoT indoor micro base station transmits on the downlink spectrum and receives on the uplink spectrum since it operates on FR1 licensed spectrum in FDD.

Proposal 5: To meet regulations, in D2T2, the Ambient IoT intermediate UE transmits on the uplink spectrum and receives on the downlink spectrum, just like a typical NR FDD UE.

For carrier wave spectrum usage, there are some agreements in RAN1’s discussion [6]:

Agreement

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied.

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Agreement

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied.

- Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

- Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum

- Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Our understanding is that for topology 1, CW inside the topology indicates that CW is transmitted from the BS. In terms of spectrum regulations, Case 1-1 seems reasonable, whereas Case 1-2, where CW is transmitted in the UL spectrum by the BS, does not appear to comply with regulations. Therefore, we recommend choosing Case 1-1 not Case 1-2.

For Case 1-4, carrier-wave is transmitted in the uplink spectrum. This allows reusing BS hardware design for the uplink receiver, as it also receives backscattered signals in the same spectrum.

For topology 2, CW inside the topology indicates that CW is transmitted from the intermediate UE. Case 2-2 is natural for the intermediate UE to transmit carrier-wave in uplink spectrum, considering the existing spectrum regulations.

For Case 2-3 and Case 2-4, considering that the carrier-wave is transmitted in uplink spectrum for Case 2-2, it is reasonable and beneficial to reuse the uplink receiver of intermediate UE for both cases where the carrier-wave node is inside and outside D2T2. Consequently, the carrier-wave node is recommended to also transmit carrier-wave in uplink spectrum.

If the external carrier-wave is assumed to be transmitted in downlink spectrum for CW outside Topology 2, as in Case 2-3, the intermediate UE has to support a different D2R receiver implementation in uplink spectrum for Case 2-2 and in downlink spectrum for Case 2-3, respectively.

It is recommended to choose Case 2-2 and Case 2-4 for inside and outside the topology, respectively.

Proposal 6: From regulation perspective, choose the following CW spectrum usage:

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied.

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied.

- Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

- Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

- 2.5 Spectrum deployment

In the SID, Ambient IoT spectrum deployment includes in-band to NR, in guard-band to LTE/NR, in standalone band(s).

As we know, in the case of sub-1GHz FDD SCS 15kHz, NR has a higher spectrum utilization rate with a smaller guard band compared to LTE. Therefore, the guard band requirement for NR technology is more challenge than that of LTE technology. Analysis coexistence in guard-band to NR will cover that to LTE.

Proposal 7: For the Ambient IoT spectrum deployment in guard-band to LTE/NR, only analysis in guard-band to NR.

When deploying Ambient IoT spectrum in-band to NR, it has a comparable effect on the PRB near NR. As a result, it can be considered a unique scenario for guard-band deployment where the evaluation from guard-band to NR can be reused.

From the perspective of operators, Ambient IoT spectrum deployment in guard-band to NR does not occupy additional spectrum resources, and at the same time has the minimum impact on NR performance.

Among the three spectrum deployment methods, operators can deploy Ambient IoT spectrum in the guard-band to NR without using up extra spectrum resources, and this deployment method has minimal impact on NR performance.

Proposal 8: Deploying NR Ambient IoT spectrum in guard-band to NR can be a top priority for coexistence analysis.

- 2.6 CW inside/outside topology

RAN1 has discussed about CW inside/outside topology for Device 1 and Device 2a which based on backscatter for D2R transmission. However, it's important to note that the proposal hasn't been approved yet. We can refer to it and make necessary adjustments as RAN1 continues to make progress. [7]

For Deployment scenario 1 with topology 1, the following scenarios are used for evaluation of coverage and coexistence:

(Copy from R1-2401874 [7])

D1T1-A: indoor BS + indoor AIoT device, CW inside topology,

- D1T1-A1: different node for CW2D/R2D and D2R

  - ‘CW’ in CW2D and ‘R’ in D2R are different

  - ‘CW’ in CW2D and ‘R’ in R2D are same

  - ‘R’ in R2D and ‘R’ in D2R are different

- D1T1-A2:

  - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D

- [R2D in at least DL spectrum]

- Only for device 1 and device 2a

- FFS for CW characteristics for further study in 9.4.2.4

- [FFS: RF energy harvesting]

D1T1-B: indoor BS + indoor AIoT device, CW outside topology (i.e., bistatic backscattering),

- [R2D in at least DL spectrum]

- FFS for CW characteristics for further study in 9.4.2.4

- Only for device 1 and device 2a

- [FFS: RF energy harvesting]

- For D1T1-B:

  - ‘CW’ in CW2D and ‘R’ in D2R are different

  - ‘CW’ in CW2D and ‘R’ in R2D are different

  - ‘R’ in R2D and ‘R’ in D2R are same

D1T1-C: indoor BS + indoor AIoT device with active UL transmission

- Only for device 2b

- R2D in DL spectrum

- D2R in UL spectrum

- [FFS: RF energy harvesting]

FFS for other scenarios

FFS other assumptions for each scenario

In the RAN1 schematic diagram, the letter R in D2R and R2D sometimes refer to the Ambient IoT base station (BS) and other times refer to the intermediate user equipment (UE). To make it simpler to understand, we have made revision to the diagram for D1T1 as in Figure 6.

Figure 6 D1T1 diagram revised from RAN1

|  | diagram from RAN1 | Corresponding diagram for RAN4 |
| --- | --- | --- |
| D1T1-A1 |  |  |
| D1T1-A2 |  |  |
| D1T1-B |  |  |
| D1T1-C: | D1T1-C: indoor BS + indoor device with active UL transmission | D1T1-C: indoor BS + indoor device with active UL transmission |

For D1T1, CW inside the topology indicates that CW is transmitted from the BS. If the CW is outside the topology and carrier-wave is transmitted in the uplink spectrum, it enables the reuse of BS hardware design for the uplink receiver, as it can also receive backscattered signals within the same spectrum.

From the above proposal, we can see that D1T1-A1 and D1T1-A2 are both for CW in inside topology, the carrier-wave is transmitted by an indoor BS.

D1T1-A1 is transmit/receive separation, while D1T1-A2 use one integrated transceiver. The assumptions regarding interference path loss for CW may be different.

D1T1-B is for CW outside topology.

D1T1-C only applies for device 2b.

(Copy from R1-2401874 [7])

For Deployment scenario 2 with topology 2, the following scenarios are used for evaluation of coverage and coexistence,

D2T2-A: outdoor BS + Indoor Intermediate UE + Indoor AIoT device, CW inside topology

- D2T2-A1: different node for CW2D/R2D and D2R

  - ‘CW’ in CW2D and ‘R’ in D2R are different

  - ‘CW’ in CW2D and ‘R’ in R2D are same

  - ‘R’ in R2D and ‘R’ in D2R are different

- D2T2-A2:

  - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D

- R2D in UL spectrum

- Only for device 1 and device 2a

- FFS for CW characteristics for further study in 9.4.2.4

- [FFS: RF energy harvesting]

D2T2-B: outdoor BS + Indoor Intermediate UE + Indoor AIoT device, CW outside topology

- R2D in UL spectrum

- Only for device 1 and device 2a

- FFS for CW characteristics for further study in 9.4.2.4

- [FFS: RF energy harvesting]

D2T2-C: outdoor BS + Indoor Intermediate UE + Indoor AIoT device with active UL transmission

- Only for device 2b

- R2D in UL spectrum

- D2R in UL spectrum

FFS for other scenarios

FFS other assumptions for each scenario

Similar with D1T1，In the RAN1 diagram, the letter R in D2R and R2D sometimes refer to the Ambient IoT base station (BS) and other times refer to the intermediate user equipment (UE). To make it simpler to understand, we have made revision to the diagram.

Figure 7 D2T2 diagram revised from RAN1

|  | diagram from RAN1 | Corresponding diagram for RAN4 |
| --- | --- | --- |
| D2T2-A1 |  |  |
| D2T2-A2 |  |  |
| D2T2-B |  |  |
| D2T2-C: | D2T2-C: outdoor BS + Indoor Intermediate UE + Indoor device with active UL transmission | D2T2-C: outdoor BS + Indoor Intermediate UE + Indoor device with active UL transmission |

For D2T2, CW inside the topology indicates that CW is transmitted from the intermediate UE. From the above proposal, we can see that D2T2-A1 and D2T2-A2 are both for CW in inside topology, the carrier-wave is transmitted by an indoor intermediate UE.

Carrier-wave can be transmitted in the uplink when CW is inside topology. When CW is outside topology, if the CW is assumed to be transmitted in uplink spectrum, it is reasonable and beneficial to reuse the uplink receiver of intermediate UE

D2T2-A1 is transmit/receive separation, while D2T2-A2 use one integrated transceiver. The assumptions regarding interference path loss for CW may be different.

D2T2-B is for CW outside topology.

D2T2-C only applies for device 2b.

- 2.7 Coexistence cases list

According to the above discussion, we propose some general rules (Proposal 3~8) to be considered for ambient IoT coexistence scenarios:

For Device 1 the potential coexistence cases are as follows:

Table 1-1. coexistence cases for device 1

| Coexistence case No. | Diagram | R2D spectrum | CW transmit spectrum | Meet s Proposal 4? | Meet Proposal 5? | Meet Proposal 6? | Whether recommend |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | DL | DL | Yes | NA | Yes | Yes |
| 2 |  | DL | UL | Yes | NA | No | No |
| 3 |  | UL | DL | No | NA | Yes | No |
| 4 |  | UL | UL | No | NA | No | No |
| 5 |  | DL | DL | Yes | NA | Yes | Yes |
| 6 |  | DL | UL | Yes | NA | No | No |
| 7 |  | UL | DL | No | NA | Yes | No |
| 8 |  | UL | UL | No | NA | No | No |
| 9 |  | DL | UL | Yes | NA | Yes | Yes |
| 10 |  | UL | UL | No | NA | Yes | No |
| 11 |  | UL | UL | NA | Yes | Yes | Yes |
| 12 |  | UL | UL | NA | Yes | Yes | Yes |
| 13 |  | UL | DL | NA | Yes | No | No |
| 14 |  | UL | UL | NA | Yes | Yes | Yes |

If we take Device 2a without a frequency shifter as the baseline for Device 2a coexistence analysis (as Proposal 1), then for device 2a, the similar coexistence cases with device 1 are as Table 1-2follows:

Table 1-2. coexistence cases for device 2a

| Coexistence case No. | Diagram | R2D spectrum | CW transmit spectrum | Meet s Proposal 4? | Meet Proposal 5? | Meet Proposal 6? | Whether recommend |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1’ |  | DL | DL | Yes | NA | Yes | Yes |
| 2’ |  | DL | UL | Yes | NA | No | No |
| 3’ |  | UL | DL | No | NA | Yes | No |
| 4’ |  | UL | UL | No | NA | No | No |
| 5’ |  | DL | DL | Yes | NA | Yes | Yes |
| 6’ |  | DL | UL | Yes | NA | No | No |
| 7’ |  | UL | DL | No | NA | Yes | No |
| 8’ |  | UL | UL | No | NA | No | No |
| 9’ |  | DL | UL | Yes | NA | Yes | Yes |
| 10’ |  | UL | UL | No | NA | Yes | No |
| 11’ |  | UL | UL | NA | Yes | Yes | Yes |
| 12’ |  | UL | UL | NA | Yes | Yes | Yes |
| 13’ |  | UL | DL | NA | Yes | No | No |
| 14’ |  | UL | UL | NA | Yes | Yes | Yes |

After excluding the unreasonable scenarios, the remaining coexistence cases for Device 1 and Device 2a are as follows:

Table 1-3. coexistence cases for device 1/2a

| Coexistence case No. | Diagram | Device type | R2D spectrum | CW transmit spectrum |
| --- | --- | --- | --- | --- |
| 1(') |  | Device 1/2a | DL | DL |
| 5(') |  | Device 1/2a | DL | DL |
| 9(') |  | Device 1/2a | DL | UL |
| 11(') |  | Device 1/2a | UL | UL |
| 12(') |  | Device 1/2a | UL | UL |
| 14(') |  | Device 1/2a | UL | UL |

For device 2b, the device’s UL transmission is generated internally by the device. It does not depend on backscatter.

There are two cases so far.

Table 1-4. coexistence cases for device 2b

| Coexistence case No. | Deployment & Topology | Device Type | Description and constraints of the scenario |
| --- | --- | --- | --- |
| 15 | D1T1-C | Device 2b | indoor BS + indoor device with active UL transmission R2D in DL spectrum D2R in UL spectrum (refer to R1-2401735) |
| 16 | D2T2-C | Device 2b | outdoor BS + Indoor Intermediate UE + Indoor device with active UL transmission R2D in UL spectrum D2R in UL spectrum (refer to R1-2401735) |

Proposal 9: To begin with, take Table 1-3 for coexistence cases involving Device 1 and Device 2a. As for Device 2b, take Table 1-4 as start point.

- 2.8 coexistence assumptions and parameters

We propose following parameters & assumptions for Deployment scenario 1 with Topology 1,the reason is list in the note column.

General parameters are proposed to be as follows：

Table 2-1. General parameters

| General Parameter | D1T1&D2T2 | note |
| --- | --- | --- |
| Carrier frequency | 900 MHz (Band 8) |  |
| Waveform | DL: OFDM-based waveform UL: single carrier | RAN1#116 Agreed: A-IoT DL study includes an OFDM-based waveform from A-IoT R2D (reader-to-device) perspective. |
| Channel BW | DL: 180kHz，for 15kHz SCS UL: 15 kHz |  |
| Layout | InF-DH defined in TR38901 Hall size：120×60 m ISD（D）=20m | TR 38.848, the maximum connection density target is 150 devices per 100 m2 for indoor scenarios. |
| Path loss model | D1T1: UL/DL: InF-DH defined in TR38.901；[NLOS/LOS] CW to device: InF-DH LOS D2T2: down-select from: InF-DL; InH-Office model, [NLOS/LOS] | [3 ]R1-2401874，FL summary (final) for Ambient IoT evaluation，Moderator (CMCC)，RAN1#116，Feb. 2024 |
| A-IoT DL power control | No |  |
| A-IoT UL power control | No |  |

For D1T1，A-IoT micro BS parameters are proposed as follows:

Table 2-2. A-IoT micro BS parameters

| A-IoT micro BS parameters | D1T1 | note |
| --- | --- | --- |
| A-IoT micro-BS total Tx power | 33dBm or （optional）38 dBm | The transmit power of the indoor Ambient IoT BS is assumed to be no lower than that of RFID，that is，35 dBm EIRP (e.g., 33 dBm transmit power and 2 dBi antenna gain) |
| A-IoT micro-BS Antenna height | 8m | The indoor factory scenario described in TR 38.901 can be reused for this indoor Ambient IoT deployment, where the height of room and UE are set to 10 m and 1.5 m, respectively. Assuming celling installation for the indoor BS, the BS height can be assumed to be 8m. |
| A-IoT micro-BS receiver Noise Figure（dB） | 5 |  |
| A-IoT micro-BS antenna gain including feeder loss (dBi) | [FFS] |  |
| A-IoT micro-BS ACLR (dB) | ACLR1:40dB，ACLR2:50dB | Reference to TS 36.104 NB-IoT standalone |

For D2T2，A-IoT intermediate UE parameters are proposed as follows:

Table 2-3. intermediate UE parameters

| intermediate UE parameters | D2T2 | note |
| --- | --- | --- |
| intermediate UE total Tx power（dBm） | 23 |  |
| intermediate UE Antenna height(m) | 1.5 |  |
| gain of antenna intermediate UE including feeder loss (dBi) | 0 |  |
| intermediate UE receiver Noise Figure（dB） | 7 |  |

NR UE parameters are proposed as follows:

Table 2-4. NR UE parameters

| NR UE Parameter | Recommended value | note |
| --- | --- | --- |
| NR UE Tx power (dBm) | 23 (maximum) |  |
| NR UE Antenna gain (dBi) | 0 |  |
| NR UE noise floor (dBm) | -112.4 @ 180kHz | noise floor=-174+10*log (180*1000)+9 |
| NR UE ACLR（dB） | 30 | For power class 3 NR UE |
| NR UE Maximum input level（dBm） | -25 | TS38101-1 7.4 Maximum input level |
| NR UE Noise Figure（dB） | 9 |  |

NR outdoor macro-BS parameters are proposed as follows:

Table 2-5. NR macro-BS parameters

| NR macro-BS Parameter | Recommended value | note |
| --- | --- | --- |
| Macro-BS Tx power (dBm) | 46 |  |
| Height of macro NR BS (m) | 25 |  |
| NR Macro-BS Noise Figure(dB) | 5 |  |

Both of Device 1 and Device 2a base on backscattered UL transmissions. Device 2a has DL and/or UL amplification to enhance receiver sensitivity and increase backscattered signal power, which sets it apart from Device 1 that lacks an amplifier.

For device 1 and 2a, the power of backscattered signals depends on the power of the received external carrier-wave and the distance between carrier-wave node and Ambient IoT devices. It is reasonable to assume backscattered signals of both of device 1 and 2a are less than -10dBm.

Device 2b generates its UL transmission internally, independent of backscatter. Considering the peak power consumption of few 100 µW and the power added efficiency (PAE) of power amplifier, the maximum transmit power is expected to be no higher than -10 dBm. We can use [-10/-20] dBm to analysis.

Device parameters are proposed as follows:

Table 2-6. Devices parameters

| A-IoT device parameters | Device 1 | Device 2a | Device 2b | Note |
| --- | --- | --- | --- | --- |
| Height of Device (m) | 1.5 | 1.5 | 1.5 |  |
| A-IoT device Tx power (dBm) | <-10 | <-10 | [-10/-20] |  |
| Device antenna height（m） | 1.5 | 1.5 | 1.5 |  |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | -3 | -3 | -3 |  |
| A-IoT device reflection （backscatter）loss (dB) Note: due to, e.g., impedance mismatch | -6 dB and 0 dB for OOK / FSK and BPSK | N/A | N/A |  |
| A-IoT device power gain of reflection amplifier (dB) | N/A | [10 / 20] | N/A |  |
| A-IoT Device receiver sensitivity (dBm) | -36 | -46 | [FFS] |  |

Proposal 10: For Ambient IoT, adopt the assumptions and parameters for co-existence analysis as Table 2-1~ 2-6.

Conclusion

Based on the analysis in this contribution, we have the following observations and proposals:

Observation 1: Device 1 and Device 2a share similar coexistence scenarios, as their UL transmissions are backscattered on an externally provided carrier wave.

Proposal 1: Take Device 2a without a frequency shifter as the baseline for Device 2a coexistence analysis, because Device 2a without a frequency shifter brings more severe carrier-wave interference to the uplink receiver compared to the one with a frequency shifter.

Proposal 2: Device 2b behaves differently from Device 1 and Device 2a, and requires separate treatment on co-existence scenarios.

Proposal 3: To minimize the chance of interference, it is recommended to use D1T1 co-site case (1) as a coexistence start point, i.e. deploying different frequency bands for NR and Ambient IoT on the same site. D1T1 co-site case (2) can be considered later.

Proposal 4: To meet regulations, in D1T1, the Ambient IoT indoor micro base station transmits on the downlink spectrum and receives on the uplink spectrum since it operates on FR1 licensed spectrum in FDD.

Proposal 5: To meet regulations, in D2T2, the Ambient IoT intermediate UE transmits on the uplink spectrum and receives on the downlink spectrum, just like a typical NR FDD UE.

Proposal 6: From regulation perspective, choose the following CW spectrum usage:

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied.

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied.

- Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

- Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Proposal 7: For the Ambient IoT spectrum deployment in guard-band to LTE/NR, only analysis in guard-band to NR.

Proposal 8: Deploying NR Ambient IoT spectrum in guard-band to NR can be a top priority for coexistence analysis.

Proposal 9: To begin with, take Table 1-3 for coexistence cases involving Device 1 and Device 2a. As for Device 2b, take Table 1-4 as start point.

Proposal 10: For Ambient IoT, adopt the assumptions and parameters for co-existence analysis as Table 2-1~ 2-6.

Reference

[1] RP-234058，New SID: Study on solutions for Ambient IoT (Internet of Things) in NR，Huawei，RAN#102, Dec., 2023

[2] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

[3] R1-2401682, FL Summary for 9.4.1.2 Ambient IoT Device Architecture, Moderator (Qualcomm), RNA1#116, Feb. ,2024

[4] 3GPP TR 38.848 V18.0.0 (2023-09), Study on Ambient IoT (Internet of Things) in RAN

[5] R1-2402011, Evaluation methodology and assumptions for Ambient IoT, RAN1#116bis, Huawei, HiSilicon, Apr.2024

[6] R1-2401767, Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things) in NR, Ad-Hoc Chair (Huawei), RNA1#116, Feb. ,2024

[7 ]R1-2401874，FL summary (final) for Ambient IoT evaluation，Moderator (CMCC)，RAN1#116，Feb. 2024