3GPP TSG-RAN WG4 Meeting #110bis	R4-231XXXX

# Changsha, China, Apr. 15 – Apr. 19, 2024

Source: 	Huawei, HiSilicon

Title: 					R19 ATG BS RF requirements for CA

Agenda Item:	9.8.3

Document for:	Discussion

Introduction

The new WID on Enhancements for Air-to-ground network for NR was approved at the RAN#103 meeting [1] .The RAN4 BS RF relative objectives are copied as below:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: n79

  - FR1+FR1 inter-band CA

    - Example band combination: n3+n39

…

In this paper we provide our opinions on R19 ATG BS RF requirements for CA.

Discussion

According to TS38.104, TAE requirements are defined as follows [2]：

- 6.5.3.2	Minimum requirement for BS type 1-C and BS type 1-H

For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

The time alignment error requirements for NB-IoT are specified in TS 36.104 [13] clause 6.5.3.

- 9.6.3.2	Minimum requirement for BS type 1-O

For MIMO transmission, at each carrier frequency, OTA TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 260 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

To support intra-band co-located and inter-band co-located DL CA for ATG BS, it makes sense to reuse the existing TAE requirements in TS38.104.

Proposal: Reuse the current TAE requirements in TS38.104 for ATG BS supporting intra-band co-located and inter-band co-located DL CA.

Conclusion

In the contribution, we provide our opinions on R19 ATG BS RF requirements for CA and have the following proposal:

Proposal: Reuse the current TAE requirements in TS38.104 for ATG BS supporting intra-band co-located and inter-band co-located DL CA.

Reference

[1] RP-240839, New WID on Enhancements for Air-to-ground network for NR, Moderator (Apple), RAN#103, Mar., 2024

[2] 3GPP TS 38.104, V18.4.0 (2023-12), NR; Base Station (BS) radio transmission and reception