3GPP TSG-RAN WG4 Meeting #110bis 	R4-2405364

# Changsha, China, 15th – 19th April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for R18 LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | Introduce test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for R18 LTM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for R18 LTM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

A.6.5.x.2	PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM

A.6.5.x.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM. The Te requirements are specified in 7.1. The interruption requirements specified in clause 8.2.2.2.20. This test is for UE supporting PDCCH-ordered RACH to an inter-frequency candidate cell, whose SSB is outside active BWPs of the UE.

A.6.5.x.2.2	Test Parameters

Supported test configurations are shown in table A.6.5.x.2.2-1. Both interruption length and Te requirements for the first uplink transmission are tested by using the parameters in table A.6.5.x.2.2-2, A.6.5.x.2.2-3, and A.6.5.x.2.2-4.

The test consists of a preparation period T0 for the repeated test, and 3 successive time periods, with time durations of T1, T2 and T3 respectively.

Before the start of time duration T1, UE has received Preconfiguration of candidate cells, L3 measurement configuration, L1 measurement configuration of Cell 1. Cell 2 is configured as one candidate SpCell in the LTM-config of Cell 1. Early UL sync to the candidat cell is also configured. The UE has sent one A3-event based L3 report to Cell 1 which triggeres the L1 measurement on Cell 2. A L1 report from UE implies the start of T1.

During T1, UE shall continuously send L1 reports to TE. For UE supporting TCI activation before cell switch, the TE also sends the TCI activation command to the UE. Based on L1 reports, TE sends the PDCCH-oder to the UE, indicating UE to transmit PRACH on the PRACH occasion as specified in TS 38.213. This implies the start of T2. For UE supporting TCI activation before cell switch, the PDCCH-order shall not be sent earlier than THARQ + 3Nslotsubframe,µ+ (Tfirst-SSB + TSSB-proc) / NR slot length ms after the TCI activation command is sent by TE.

During T2, UE shall prepare the corresponding RF chains for the PDCCH ordered RACH transmission. The end of T2 is the time UE sends PRACH to the candidate cell. Afther UE finishes the PRACH transmission, UE tunes the RF chain back to the source cell. The duration of T2 shall cover the periods for UE tuning away, sending PRACH, and tuning back. At the end of T2, configuration for LTM L1 measurement on cell 2 is released/de-activated

During T3, UE sends A3-event based L3 report to Cell 1 which triggeres the configuration/activation of L1 measurement on Cell 2. A L1 report from UE implies the start of T1.

Table A.6.5.x.2.2-1: Inter-frequency PDCCH-ordered RACH from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.x.2.2-2: General test parameters for Inter-frequency PDCCH-ordered RACH test from FR1 to FR1 from FR1 to FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| A3-Offset | dB | 0 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| Measurement gap pattern Id |  | #0 | Gaps are configured before T1. |
| Access Barring Information | - | Not Sent | No additional delays in random access procedure. |
| ltm-CSI-SSB-ResourceList |  | SSB.0 of Cell 1 SSB.0 of Cell 2 |  |
| ltm-ReportConfigType |  | periodic |  |
| ltm-ReportContent |  | Number of reported cells: 1 Number of reported RSs per Cell: 1 |  |
| Time offset between cells |  | < 3 s | Synchronous cells |
| T0 | ms | 5s |  |
| T1 | ms | < 200ms |  |
| T2 | ms | 100 ms |  |
| T3 | ms | < 200ms |  |

Table A.6.5.x.2.2-3: Cell specific test parameters for NR FR1-FR1 Inter-frequency PDCCH-ordered RACH test

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |
| LTM-TCI-Info |  | Config 1 |  | DLorJointTCI:TBD; ULTCI: TBD |  |  |  |  |  |
|  |  | Config 2 |  | DLorJointTCI:TBD; ULTCI: TBD |  |  |  |  |  |
|  |  | Config 3 |  | DLorJointTCI:TBD; ULTCI: TBD |  |  |  |  |  |
| TRS configuration for serving cell |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| [TRS configuration for candidate cell] |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration for candidate cell |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |
|  |  |  | dB | 4 | 8 | 4 | 8 | 4 | 8 |
|  |  |  | dB | 4 | 8 | 4 | 8 | 4 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -87 | -90 | -87 | -90 | -87 |
|  | Config 3 |  | dBm/SCS | -87 | -84 | -87 | -84 | -87 | -84 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.06 | -61.41 | -57.06 | -61.41 | -57.06 |
|  | Config 3 |  | dBm/ 9.36MHz | -55.31 | -50.96 | -55.31 | -50.96 | -55.31 | -50.96 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

A.6.5.x.2.3 Test Requirements

The UE shall transmit PRACH to the candidate cell with the Te requirements as specified in 7.1.

The UE shall be scheduled on PCell continuously throughout the test. During the time duration T2, the interruption on PCell shall not be more than the values specified for SA in clause 8.2.2.2.10.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change #1>