3GPP TSG-RAN WG4 Meeting #110bis    	R4-2405408

Changsha, China, 15th – 19th March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | <CR#> | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR 36.101 Addition of UL CA_28C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R18_xBDL_yBUL-Core |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of UL CA_28C |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant tables updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL CA_28C not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6A.1, 6.2.2A, 6.2.4A, 6.2.4A.11, 6.6.2.2A.5, 6.6.3.2A, 7.3.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of changes ************************************

### 5.6A.1	Channel bandwidths per operating band for CA

The requirements for carrier aggregation in this specification are defined for carrier aggregation configurations with associated bandwidth combination sets. For inter-band carrier aggregation, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class. For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class.

For each carrier aggregation configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

Requirements for intra-band contiguous carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-1. Requirements for inter-band carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-2, Table 5.6A.1-2a, Table 5.6A.1-2b and Table 5.6A.1-2c. Requirements for intra-band non-contiguous carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-3.

The DL component carrier combinations for a given CA configuration shall be symmetrical in relation to channel centre unless stated otherwise in Table 5.6A.1-1, Table 5.6A.1-2, Table 5.6A.1-2a, Table 5.6A.1-2b and Table 5.6A.1-2c.

Table 5.6A.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_1C | CA_1C | 15 | 15 |  |  |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |  |  |
|  |  | 5, 10, 15 | 20 |  |  |  | 40 | 1 |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_2C |  | 5 | 20 |  |  |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_3B |  | 5 | 3 |  |  |  | 10 | 0 |
|  |  | 3, 5 | 5 |  |  |  |  |  |
| CA_3C | CA_3C | 5, 10, 15 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_5B | CA_5B | 5, 10 | 10 |  |  |  | 20 | 0 |
|  |  | 10 | 5 |  |  |  |  |  |
| CA_7B |  | 15 | 5 |  |  |  | 20 | 0 |
| CA_7C | CA_7C | 15 | 15 |  |  |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15 |  |  |  | 40 | 2 |
|  |  | 20 | 15, 20 |  |  |  |  |  |
| CA_8B | CA_8B | 5,10 | 10 |  |  |  | 20 | 0 |
|  |  | 10 | 5 |  |  |  |  |  |
|  |  | 10 | 3, 5 |  |  |  | 15 | 1 |
|  |  | 3, 5 | 10 |  |  |  |  |  |
| CA_12B | - | 5 | 5, 10 |  |  |  | 15 | 0 |
| CA_23B | - | 10 | 10 |  |  |  | 20 | 0 |
|  |  | 5 | 15 |  |  |  |  |  |
| CA_27B | - | 1.4, 3, 5 | 5 |  |  |  | 13 | 0 |
|  |  | 1.4, 3 | 10 |  |  |  |  |  |
| CA_28C | CA_28C | 5 | 20 |  |  |  | 30 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15 |  |  |  |  |  |
|  |  | 20 | 5, 10 |  |  |  |  |  |
| CA_38C | CA_38C | 15 | 15 |  |  |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |  |  |
| CA_39C | CA_39C | 5,10,15 | 20 |  |  |  | 35 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |  |  |
| CA_40C | CA_40C | 10 | 20 |  |  |  | 40 | 0 |
|  |  | 15 | 15 |  |  |  |  |  |
|  |  | 20 | 10, 20 |  |  |  |  |  |
|  |  | 10, 15 | 20 |  |  |  | 40 | 1 |
|  |  | 15 | 15 |  |  |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |  |  |
| CA_40D | CA_40C, CA_40D | 10, 15, 20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 10, 15 | 20 |  |  |  |  |
|  |  | 20 | 20 | 10, 15 |  |  |  |  |
|  |  | 15, 20 | 15, 20 | 15, 20 |  |  | 60 | 1 |
| CA_40E | CA_40C | 15, 20 | 15, 20 | 15, 20 | 20 |  | 80 | 0 |
| CA_40F | CA_40C | 15, 20 | 15, 20 | 15, 20 | 20 | 20 | 100 | 0 |
| CA_41C5 | CA_41C | 10 | 20 |  |  |  | 40 | 0 |
|  |  | 15 | 15, 20 |  |  |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |  |  |
|  |  | 5, 10 | 20 |  |  |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
|  |  | 10 | 15, 20 |  |  |  | 40 | 2 |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 40 | 3 |
|  |  | 20 | 20 |  |  |  |  |  |
| CA_41D | CA_41C, CA_41D | 10 | 20 | 15 |  |  | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |  |  |
| CA_41E | CA_41C, CA_41D | 15, 20 | 15, 20 | 15, 20 | 20 |  | 80 | 0 |
| CA_41F | CA_41C, CA_41D | 10,15, 20 | 15, 20 | 20 | 20 | 20 | 100 | 0 |
| CA_42C5 | CA_42C | 5, 10, 15, 20 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |  |  |
|  |  | 10, 15, 20 | 20 |  |  |  | 40 | 1 |
|  |  | 20 | 10, 15 |  |  |  |  |  |
| CA_42D | CA_42C | 5,10,15,20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 20 | 5,10,15 |  |  |  |  |
|  |  | 10, 15, 20 | 20 | 20 |  |  | 60 | 1 |
|  |  | 20 | 20 | 10, 15 |  |  |  |  |
| CA_42E | CA_42C | 5,10,15,20 | 20 | 20 | 20 |  | 80 | 0 |
|  |  | 20 | 20 | 20 | 5,10,15 |  |  |  |
| CA_42F | CA_42C | 5, 10, 15, 20 | 20 | 20 | 20 | 20 | 100 | 0 |
|  |  | 20 | 20 | 20 | 20 | 5, 10, 15, 20 |  |  |
| CA_43C | - | 5 | 20 |  |  |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_46C4 | - | 20 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 10, 20 |  |  |  | 40 | 1 |
|  |  | 10, 20 | 20 |  |  |  |  |  |
| CA_46D 4 | - | 20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 20 | 10, 20 |  |  | 60 | 1 |
|  |  | 10, 20 | 20 | 20 |  |  |  |  |
| CA_46E 4 | - | 20 | 20 | 20 | 20 |  | 80 | 0 |
|  |  | 20 | 20 | 20 | 10, 20 |  | 80 | 1 |
|  |  | 10 | 20 | 20 | 20 |  |  |  |
| CA_48B | CA_48B | 10 | 10 |  |  |  | 20 | 0 |
| CA_48C | CA_48C | 5, 10, 15, 20 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |  |  |
| CA_48D | CA_48C | 5,10,15,20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 20 | 5,10,15 |  |  |  |  |
| CA_48E | CA_48C | 5,10,15,20 | 20 | 20 | 20 |  | 80 | 0 |
|  |  | 20 | 20 | 20 | 5,10,15 |  |  |  |
| CA_48F | - | 5, 10, 15, 20 | 20 | 20 | 20 | 20 | 100 | 0 |
|  |  | 20 | 20 | 20 | 20 | 5, 10, 15, 20 |  |  |
| CA_66B | CA_66B | 5 | 5, 10, 15 |  |  |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |  |  |
|  |  | 15 | 5 |  |  |  |  |  |
| CA_66C | CA_66C | 5 | 20 |  |  |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_66D | - | 5 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 5 | 20 |  |  |  |  |
|  |  | 20 | 20 | 5 |  |  |  |  |
|  |  | 10 | 20 | 15 |  |  |  |  |
|  |  | 15 | 20 | 10 |  |  |  |  |
|  |  | 10, 15, 20 | 15, 20 | 20 |  |  |  |  |
|  |  | 15, 20 | 10 | 20 |  |  |  |  |
|  |  | 15 | 15, 20 | 15 |  |  |  |  |
|  |  | 20 | 15, 20 | 10, 15 |  |  |  |  |
|  |  | 20 | 10 | 15 |  |  |  |  |
| CA_70C | - | 5 | 20 |  |  |  | 25 | 0 |
|  |  | 10 | 15 |  |  |  |  |  |
|  |  | 15 | 10 |  |  |  |  |  |
| NOTE 1: The CA configuration refers to an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 3: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 4: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 5: 8Rx Requirements are applicable for this band configuration if UE supports 8Rx. |  |  |  |  |  |  |  |  |

***************************** No changes ************************************

Table 6.2.2A-1: CA UE Power Class for intraband contiguous CA

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_3C |  |  |  |  | 23 | +2/-22 |  |  |
| CA_5B |  |  |  |  | 23 | +2/-22 |  |  |
| CA_7C |  |  |  |  | 23 | +2/-22 |  |  |
| CA_8B |  |  |  |  | 23 | +2/-22 |  |  |
| CA_28C |  |  |  |  | 23 | +2/-22 |  |  |
| CA_38C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_39C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_40C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_40D |  |  |  |  | 23 | +2/-2 |  |  |
| CA_41C |  |  | 26 | +2/-22 | 23 | +2/-22 |  |  |
| CA_41D |  |  |  |  | 23 | +2/-22 |  |  |
| CA_42C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_48B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_48C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_66B |  |  |  |  | 23 | +2/-2 |  |  |
| CA_66C |  |  |  |  | 23 | +2/-2 |  |  |
| NOTE 1: Void NOTE 2: If all transmitted resource blocks (Figure 5.6A-1) over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For intra-band contiguous carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

***************************** No changes ************************************

Table 6.2.4A-1: Additional Maximum Power Reduction (A-MPR) for intra-band contiguous CA

| CA Network Signalling value | Requirements (subclause) | Uplink CA Configuration | A-MPR [dB] (subclause) |
| --- | --- | --- | --- |
| CA_NS_01 | 6.6.3.3A.1 | CA_1C | 6.2.4A.1 |
| CA_NS_02 | 6.6.3.3A.2 | CA_1C | 6.2.4A.2 |
| CA_NS_03 | 6.6.3.3A.3 | CA_1C | 6.2.4A.3 |
| CA_NS_04 | 6.6.2.2A.1, 6.6.3.3A.8 | CA_41C, CA_41D | 6.2.4A.4 |
| CA_NS_05 | 6.6.3.3A.4 | CA_38C | 6.2.4A.5 |
| CA_NS_06 | 6.6.3.3A.5 | CA_7C | 6.2.4A.6 |
| CA_NS_07 | 6.6.3.3A.6 | CA_39C | 6.2.4A.7 |
| CA_NS_08 | 6.6.3.3A.7 | CA_42C | 6.2.4A.8 |
| CA_NS_09 | 6.6.2.2A.2 | CA_66B | N/A |
|  | 6.6.2.2A.3 | CA_66C | N/A |
| CA_NS_10 | 6.6.2.2A.4 6.6.3.3A.10 | CA_48B, CA_48C | 6.2.4A.10 |
| CA_NS_12 | 6.6.3.3A.11 | CA_28C | 6.2.4A.12 |
| CA_NS_13 | 6.6.3.3A.12 | CA_28C | 6.2.4A.13 |
| … |  |  |  |
| CA_NS_31 | NOTE 1 | Table 5.6A.1-1 (NOTE 1) | N/A |
| CA_NS_32 | Reserved |  |  |
| NOTE 1: Applicable for uplink CA configurations listed in Table 5.6A.1-1 for which none of the additional requirements in subclauses 6.6.2.2A or 6.6.3.3A apply. NOTE 2: The index of the sequence CA_NS corresponds to the value of additionalSpectrumEmissionSCell-r10. |  |  |  |

***************************** No changes ************************************

#### 6.2.4A.12	A-MPR for CA_NS_12

If the UE is configured to CA_28C and it receives IE CA_NS_12 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.12-1.

Table 6.2.4A.12-1: Contiguous Allocation A-MPR for CA_NS_12

| CA Bandwidth Class C | Upper edge frequency [MHz] | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR per modulation [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 25 RB / 100 RB and 100 RB / 25 RB | >=743 and < 748 | 0-25 | 100-125 |  | 4 | 3 | 3 | 1 |
|  |  | 0-17 | 25-99 and span across two carriers |  | 4 | 3 | 3 | 1 |
|  |  | 0-26 | 1-8 | <=32 | 3 | 3 | 1 | N/A |
|  |  | 19-24 | 74-99 |  | 3 | 2 | 2 |  |
|  |  | 19-52 | 70-99 and span across two carriers |  | 3 | 2 | 1 |  |
|  |  | 0-22 | 72–99 and within one carrier |  | 3 | 2 | 2 |  |
|  |  | 0-13 | 32-64 and within one carrier | 32-77 | 3 | 2 | 2 |  |
|  |  | >25 | 90-99 and across two carriers |  | 2 | 1 | 1 |  |
|  |  | 0-22 | 9-25 | <=41 | 2 | 1 | N/A |  |
|  | 748 | 0-25 | 100-125 |  | 2.5 | 2 | 2 | N/A |
|  |  | 0-10 | 51-99 and span across two carriers |  | 2 | 2 | 2 |  |
|  |  | 0-26 | 1-8 | <=32 | 2.5 | 2 | 1 |  |
|  |  | 19-52 | 87-99 and span across two carriers |  | 2 | 1 | 1 |  |
|  |  | 0-10 | 90-99 and within one carrier |  | 2 | 1 | 1 |  |
|  |  | 0-22 | 9-25 | <=41 | 2 | 1 | N/A |  |
| 50 RB / 75 RB and 75 RB / 50 RB | >=743 and < 748 | 0-25 | 100-125 |  | 4 | 3 | 3 | 1 |
|  |  | 0-15 | 45-99 and span across two carriers |  | 4 | 3 | 3 | 1 |
|  |  | 18-26 | 38-99 and span across two carriers |  | 3 | 2 | 2 | N/A |
|  |  | 0-11 | 60-75 and within one carrier |  | 3 | 2 | 2 |  |
|  |  | 0-26 | 1-12 | <=35 | 3 | 2 | 1 |  |
|  |  | 30-45 | 60-99 and across two carriers |  | 2 | 2 | 1 |  |
|  |  | 0-20 | 13-54 and within one carrier |  | 2 | 1 | 1 |  |
|  | 748 | 0-25 | 100-125 |  | 3 | 2 | 2 | N/A |
|  |  | 0-15 | 55-99 and span across two carriers |  | 3 | 2 | 2 |  |
|  |  | 0-26 | 1-12 | <=35 | 3 | 2 | 1 |  |
|  |  | 18-26 | 72-99 and span across two carriers |  | 2 | 1 | 1 |  |
|  |  | 0-8 | 13-50 and within one carrier |  | 1 | 1 | N/A |  |
| 50 RB / 100 RB and 100 RB / 50 RB | 748 | 0-26 | 125-150 |  | 4 | 3 | 3 | 1 |
|  |  | 0-14 | 41-124 and span across two carriers |  | 4 | 3 | 3 |  |
|  |  | 18-28 | 34-124 and span across two carriers |  | 3 | 2 | 2 |  |
|  |  | 0-22 | 72-124 and within one carrier |  | 3 | 3 | 2 | N/A |
|  |  | 0-35 | 1-12 | <=35 | 3 | 2 | 1 |  |
|  |  | 30-40 | 70-124 and across two carriers |  | 3 | 2 | 2 |  |
|  |  | 0-20 | 13-64 and within one carrier |  | 2 | 2 | 1 |  |
|  |  | 46-60 | 75-124 and across two carriers |  | 2 | 1 | 1 |  |
| 75 RB / 75 RB | 748 | 0-26 | 125-150 |  | 4.5 | 3 | 3 | 1 |
|  |  | 0-3 | 72-124 and span across two carriers |  | 4.5 | 3 | 3 |  |
|  |  | 11-30 | 45-124 and span across two carriers |  | 4 | 3 | 3 |  |
|  |  | 0-15 | 60-75 and within one carrier |  | 4 | 3 | 2 | N/A |
|  |  | 35-39 | 45-124 and across two carriers |  | 3 | 2 | 2 |  |
|  |  | 0-36 | 1-16 | <=48 | 3 | 2 | 1 |  |
|  |  | 43-51 | 45-124 and across two carriers |  | 2 | 1 | 1 |  |
|  |  | 0-25 | 40-50 and within one carrier |  | 2 | 2 | 1 |  |
|  |  | 0-28 | 18-36 and within one carrier |  | 1 | 1 | N/A |  |

***************************** No changes ************************************

#### 6.2.4A.13	A-MPR for CA_NS_13

If the UE is configured to CA_28C and it receives IE CA_NS_13 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.13-1.

Table 6.2.4A.13-1: Contiguous Allocation A-MPR for CA_NS_13

| CA Bandwidth Class C | Lower edge frequency [MHz] |  | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR per modulation [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 25 RB / 100 RB and 100 RB / 25 RB |  | >=703 and <713 |  | 0-25 |  | 100-125 |  |  |  | 5 |  | 3.5 |  | 3.5 |  | 1.5 |  |
|  |  |  |  | 0-1 |  | 24-99 and span across two carriers |  |  |  | 5 |  | 3.5 |  | 3.5 |  | 1.5 |  |
|  |  |  |  | 5-13 |  | 45-99 and span across two carriers |  |  |  | 4 |  | 3 |  | 3 |  | 1 |  |
|  |  |  |  | 15-25 |  | 81-99 |  |  |  | 4 |  | 3 |  | 3 |  | 1 |  |
|  |  |  |  | 0-13 |  | 72-99 and within one carrier |  |  |  | 4 |  | 3 |  | 3 |  | 1 |  |
|  |  |  |  | 0-29 |  | 40-64 and within one carrier |  |  |  | 4 |  | 3 |  | 3 |  | 1 |  |
|  |  |  |  | 20-29 |  | 1-8 |  | <=35 |  | 4 |  | 3 |  | 2 |  | N/A |  |
|  |  |  |  | 0-20 |  | 1-8 |  |  |  | 4 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 5-13 |  | 20-44 and span across two carriers |  |  |  | 3 |  | 3 |  | 3 |  |  |  |
|  |  |  |  | 15-25 |  | 9-80 and span across two carriers |  |  |  | 3 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 13-40 |  | 72-99 and within one carrier |  |  |  | 3 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 28-55 |  | 62-99 and across two carriers |  |  |  | 3 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 0-25 |  | 26-39 and within one carrier |  |  |  | 2 |  | 1 |  | N/A |  |  |  |
|  |  |  |  | 0-25 |  | 9-25 |  | <=47 |  | 1 |  | 1 |  | 1 |  |  |  |
|  |  | >=713 |  | 0-10 |  | 100-125 |  |  |  | 2.5 |  | 2 |  | 1.5 |  | 1 |  |
|  |  |  |  | 0-13 |  | 1-8 |  | <=20 |  | 2.5 |  | 2 |  | 1 |  | 1 |  |
|  |  |  |  | 0-1 |  | 65-99 and span across two carriers |  |  |  | 2.5 |  | 1 |  | 1 |  | N/A |  |
|  |  |  |  | 0-25 |  | 9-25 |  | <=37 |  | 2 |  | 1 |  | N/A |  |  |  |
|  |  |  |  | 19-28 |  | 91-99 |  |  |  | 2 |  | N/A |  | N/A |  |  |  |
| 50 RB / 75 RB and 75 RB / 50 RB |  | >=703 and <713 |  | 0-5 |  | 45-125 and span across two carriers |  |  |  | 6 |  | 4.5 |  | 4 |  | 2.5 |  |
|  |  |  |  | 10-20 |  | 30-115 and span across two carriers |  |  |  | 5 |  | 4 |  | 4 |  | 1 |  |
|  |  |  |  | 0-5 |  | 36-75 and within one carrier |  |  |  | 5 |  | 4 |  | 3 |  | 1 |  |
|  |  |  |  | 21-27 |  | 23-104 and span across two carriers |  |  |  | 4 |  | 3 |  | 3 |  | 1 |  |
|  |  |  |  | 5-10 |  | 36-75 and within one carrier |  |  |  | 4 |  | 3 |  | 2 |  | N/A |  |
|  |  |  |  | 10-30 |  | 36-75 and within one carrier |  |  |  | 3 |  | 3 |  | 1 |  |  |  |
|  |  |  |  | 30-35 |  | 30-95 and span across two carriers |  |  |  | 3 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 0-32 |  | 1-13 |  | <=41 |  | 3 |  | 2 |  | 1 |  |  |  |
|  |  |  |  | 38-51 |  | 45-87 and span across two carriers |  |  |  | 2 |  | 1 |  | 1 |  |  |  |
|  |  |  |  | 0-26 |  | 15-32 |  | <=54 |  | 2 |  | 1 |  | 1 |  |  |  |
|  |  | >713 |  | 0-5 |  | 45-125 and span across two carriers |  |  |  | 2.5 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 0-13 |  | <13 |  | <=22 |  | 2.5 |  | 2 |  | 1 |  |  |  |
|  |  |  |  | 10-20 |  | 70-110 and span across two carriers |  |  |  | 2 |  | 1 |  | 1 |  |  |  |
|  |  |  |  | 0-26 |  | 15-32 |  | <=54 |  | 1 |  | 1 |  | N/A |  |  |  |
|  |  |  |  | 21-27 |  | 23-104 and span across two carriers |  |  |  | 1 |  | N/A |  | N/A |  |  |  |
| 50 RB / 100 RB and 100 RB / 50 RB |  | >703 and <713 |  | 0-5 |  | 45-150 and span across two carriers |  |  |  | 6 |  | 4.5 |  | 4.5 |  | 2.5 |  |
|  |  |  |  | 10-20 |  | 30-140 and span across two carriers |  |  |  | 5 |  | 3 |  | 3 |  | 1 |  |
|  |  |  |  | 0-10 |  | 36-100 and within one carrier |  |  |  | 4 |  | 3 |  | 3 |  | 1 |  |
|  |  |  |  | 21-28 |  | 22-129 and span across two carriers |  |  |  | 4 |  | 3 |  | 3 |  | 1 |  |
|  |  |  |  | 10-28 |  | 36-100 and within one carrier |  |  |  | 3 |  | 2 |  | 2 |  | N/A |  |
|  |  |  |  | 30-38 |  | 20-120 and span across two carriers |  |  |  | 3 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 0-40 |  | 1-13 |  | <=49 |  | 3 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 38-68 |  | 60-112 and span across two carriers |  |  |  | 3 |  | 2 |  | 2 |  |  |  |
|  |  |  |  | 0-26 |  | 15-35 |  | <=55 |  | 3 |  | 1 |  | N/A |  |  |  |
|  |  |  |  | 28-30 |  | 36-75 and within one carrier |  |  |  | 1 |  | 1 |  | 1 |  |  |  |
|  |  | >=713 |  | 0-5 |  | 60-150 and span across two carriers |  |  |  | 3 |  | 2 |  | 2 |  | N/A |  |
|  |  |  |  | 0-22 |  | 1-13 |  | <=13 |  | 3 |  | 2 |  | 1 |  |  |  |
|  |  |  |  | 10-20 |  | 64-140 and span across two carriers |  |  |  | 2 |  | 1 |  | 1 |  |  |  |
|  |  |  |  | 0-16 |  | 15-35 |  | <=40 |  | 1 |  | 1 |  | N/A |  |  |  |
|  |  |  |  | 0-10 |  | 36-50 and within one carrier |  |  |  | 1 |  | N/A |  | N/A |  |  |  |
|  |  |  |  | 21-35 |  | 90-129 and span across two carriers |  |  |  | 1 |  | N/A |  | N/A |  |  |  |
| 75 RB / 75 RB | >=703 and <713 |  | 0-3 | 72-150 and span across two carriers |  | 6 | 4.5 | 4 | 2 |
|  |  |  | 11-15 | 60-150 and span across two carriers |  | 5 | 4 | 4 | 2 |
|  |  |  | 21-30 | 45-129 and span across two carriers |  | 5 | 4 | 3 | 1 |
|  |  |  | 0-10 | 36-75 and within one carrier |  | 5 | 4 | 3 | 1 |
|  |  |  | 35-39 | 42-115 and span across two carriers |  | 4 | 3 | 2 | 1 |
|  |  |  | 10-15 | 36-75 and within one carrier |  | 4 | 3 | 2 | N/A |
|  |  |  | 43-51 | 48-107 and span across two carriers |  | 3 | 2 | 2 |  |
|  |  |  | 0-41 | 1-17 | <=53 | 3 | 2 | 1 |  |
|  |  |  | 15-31 | 36-75 and within one carrier |  | 3 | 2 | 1 |  |
|  |  |  | 55-60 | 40-95 and span across two carriers |  | 3 | 1 | N/A |  |
|  |  |  | 0-26 | 18-36 | <=62 | 2 | 1 |  |  |
|  | >=713 |  | 0-3 | 72-150 and span across two carriers |  | 3 | 2 | 2 | N/A |
|  |  |  | 11-15 | 60-139 and span across two carriers |  | 3 | 2 | 2 |  |
|  |  |  | 21-30 | 68-129 and span across two carriers |  | 2 | 1 | 1 |  |
|  |  |  | 0-23 | 1-17 | <=35 | 3 | 2 | 1 |  |
|  |  |  | 0-18 | 18-36 | <=41 | 3 | 1 | N/A |  |
|  |  |  | 0-10 | 36-75 and within one carrier |  | 1 | N/A | N/A |  |
|  |  |  | 35-39 | 42-115 and span across two carriers |  | 1 | N/A | N/A |  |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation |  |  |  |  |  |  |  |  |  |

***************************** No changes ************************************

##### 6.6.3.3A.11	Minimum requirement for CA_28C (network signalled value "CA_NS_12")

When "CA_NS_12" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.11-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.11-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 3, 5, 10 MHz |  |  |
| 470 ≤ f ≤ 710 | -26.2 | 6 MHz | 1 |
| NOTE 1: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Applicable when the assigned E-UTRA carrier is confined within 715 MHz and 718 MHz and when the channel bandwidth used is 3 MHz. |  |  |  |

***************************** No changes ************************************

##### 6.6.3.3A.12	Minimum requirement for CA_28C (network signalled value "CA_NS_13")

When "CA_NS_13" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.12-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.12-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |  |
| 692-698 | -26.2 | 6 MHz |  |

***************************** No changes ************************************

Table 6.6.3.2A-1: Requirements for intraband carrier aggregation

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1 | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 50, 51, 52, 65, 67, 72, 73, 74, 75, 76 NR Band n78, n79, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| CA_3 |  | E-UTRA Band 1, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79, n100, n101 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 52, 53, 54 NR Band n77, n78,n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_7 | E-UTRA Band 1, 3, 7, 8, 20, 22, 27, 28, 29, 30. 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 50, 54, 51, 72, 73, 74, 75, 76 NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  |  | E-UTRA Band 22, 41, 42, 43, 52 NR Band n77, n78,n79 |  | FDL_low |  |  |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| CA_28 |  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n100, n101 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 16, 25 |  |
|  |  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 16, 17 |  |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 10, 18 |  |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 19 |  |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 10 |  |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 10 |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 15, 16 |  |
| CA_38 |  | E-UTRA Band 1,3, 8, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n100, n101 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| CA_39 | E-UTRA Band 22, 34, 40, 41, 42, 44, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 50, 51, 52, 65, 67, 72, 73, 74, 75, 76 NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 15 |
| CA_41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 50, 51, 65, 66, 67, 72, 73, 74, 75, 76 NR Band n79, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| CA_48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: To meet these requirements some restriction will be needed for either the operating band or protected band NOTE 4: N/A NOTE 5: N/A NOTE 6: N/A NOTE 7: N/A NOTE 8: N/A NOTE 9: N/A NOTE 10: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 11: N/A NOTE 12: N/A NOTE 13: N/A NOTE 14: N/A NOTE 15: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 16: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Applicable when the assigned E-UTRA carrier is confined within 715 MHz and 718 MHz and when the channel bandwidth used is 3 MHz. NOTE 17: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 18: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 19: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Applicable when the assigned E-UTRA carrier is confined within 715 MHz and 718 MHz and when the channel bandwidth used is 3 MHz. |  |  |  |  |  |  |  |

***************************** No changes ************************************

Table 7.3.1A-1: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class C

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink CA configuration | 100RB+25RB |  | 100RB+50RB |  | 75RB+75RB |  | 75RB+50RB |  | 100RB+75RB |  | 100RB+100RB |  | Duplex Mode |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |
| CA_1C | N/A | N/A | N/A | N/A | 75 | 54 | N/A | N/A | N/A | N/A | 100 | 30 | FDD |
| CA_3C | 50 | 0 | 50 | 0 | N/A | N/A | N/A | N/A | 50 | 0 | 50 | 0 | FDD |
| CA_7C | N/A | N/A | 75 | 0 | 75 | 0 | 75 | 0 | 75 | 0 | 75 | 0 | FDD |
| CA_28C | 25 | 0 | 25 | 0 | 25 | 0 | 25 | 0 | N/A | N/A | N/A | N/A | FDD |
| CA_38C | N/A | N/A | N/A | N/A | 75 | 75 | N/A | N/A | N/A | N/A | 100 | 100 | TDD |
| CA_39C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | N/A | N/A | TDD |
| CA_40C | N/A | N/A | 100 | 50 | 75 | 75 | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_41C | 100 | 25 | 100 | 50 | 75 | 75 | 75 | 50 | 100 | 75 | 100 | 100 | TDD |
| CA_42C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_48C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_66C | 100 | 25 | 100 | 50 | 75 | 75 | 75 | 50 | 100 | 75 | 100 | 100 | FDD |
| NOTE 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. NOTE 5: In case a CA configuration consists of CC channel bandwidths which are unequal in bandwidth the PCC channel bandwidth shall be the larger one for reference sensitivity test. NOTE 6: Void. NOTE 7: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |

***************************** End of changes ************************************