3GPP TSG-RAN WG4 Meeting # 110bis 	R4-2405419

Changsha, China, April 15 – 19, 2024

Source: 	Huawei, HiSilicon

Title: 	UE capability for Redcap UE

Agenda Item:	4.2.1.3

Document for:	Approval

1 Introduction

In this contribution, we provide our view on the remaining issue of UE capability for enhanced channel raster.

2 Discussion

As discussed in last meeting the following shows the agreement made in R4-2403842

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_channel_raster_enh | 28-1 | Enhanced channel raster | The UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1, TS 38.101-5 |  | Yes |  | UE may not support requirements for UE specific channel bandwidths located on enhanced channel raster; configuring a narrower UE-specific channel bandwidth inside a wider gNB channel bandwidth may not be possible. | Per Band | No | FR1 only | The feature is supported for applicable bands in FDD-TDD and FR1/FR2 combinations | Applies only for bands with a 100 kHz channel raster for both TN and NTN. Should be early implementable from Rel-16. | Mandatory with capability signaling for all Rel-18 UEs for certain bands as defined in 38.101-1 and 38.101-5 Optional otherwise FFS for RedCap |

It is noted that the only remaining issue is FFS for RedCap. In last meeting interested operators provided inputs on the request of mandatory bands for enhanced channel raster [2-4]. And Redcap is the major justification for the mandatory feature proposal. The following agreement was made and in the feature list it was agreed as “Mandatory with capability signaling for all Rel-18 UEs for certain bands as defined in 38.101-1 and 38.101-5”. In our view here “all Rel-18 UEs” should also include Rel-18 Redcap UEs.

Agreement:

- Based on discussion papers from interested operators, RAN4 can support the enhanced channel raster as mandatory feature in NR Band n1, n2, n3, n5, n25, n28, n66, n71 and n85 from Rel-18.

And for early release, there are a large number of Rel-17 commercial Redcap UEs in the field sine Rel-17 are frozen in 2022. Hence it is not possible to assume this new feature can be mandate for the Rel-17 commercial Redcap UEs. Hence it is unfortunately that it has to be an optional feature for Rel-17. Hence we propose that the channel raster enhancement feature is optional for Rel-17 RedCap UE, .i.e. the same as normal UEs.

Proposal 1: it is proposed to remove FFS for RedCap UE in the RAN4 feature list.

3 Conclusion

Proposal 1: it is proposed to remove FFS for RedCap UE in the RAN4 feature list.

References

[1] R4-2400647,	Frequency bands with mandatory support of the new channel raster, Orange UK

[2] R4-2400723,	Mandatory enhanced raster for NR bands n2, n5, and n66, AT&T

[3] R4-2402472,	Request for mandatory enhanced raster for n25, n66, n71 and n85, T-Mobile USA

[4] R4-2403842, Rel-18 RAN4 UE feature list for NR (version 3), CMCC