3GPP TSG-RAN WG4 Meeting # 110bis 	R4-2405428

Changsha, China, April 15 – 19, 2024

Source: 	Huawei, HiSilicon

Title: 	On pre-set sub array tilt

Agenda Item:	9.3.5

Document for:	Approval

1 Introduction

Working Party 5D is seeking information on IMT parameters for the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz [1]. In the last plenary (RAN#103) the Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [2]. In the objective of SI, there are questions on the antenna parameters asked by WP5D.

In this contribution we provide our view on the pre-set sub array tilt parameter.

2 Discussion

In the WP5D LS, there are serval comment/questions on the AAS antenna parameter pre-set sub array:

“i)	The pre-set sub array tilt is understood as a parameter (a pre-tilt within the elements combined to form a sub array) that is used to focus the main beam below the boresight where a base station is intended to serve its UEs, similarly to the mechanical downtilt. Noticing that the mechanical downtilt of the BS antenna is environment (urban/suburban/rural) dependent, WP5D welcomes further confirmation whether the preset sub-array downtilt varies with the environment or not, what would be the range of values for this parameter and the associated environments and if multiple set of configurable preset sub-array downtilt per BS is foreseen in the future.”

Table 1-1: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation (Note 1) | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern (Note 1) | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan Where M and N is corresponding to (Row × Column) in Table 1-2, row 1.6. |
| Composite array radiation pattern | For sub-array structures: AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 For element structures: AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ Where M and N is corresponding to (Row × Column) in Table 1-2, row 1.6. |
| Note 1: it is only applicable for sub-array structures |  |

Pre-set sub-array down-tilt (θsubtilt) is a parameter used for setting the Sub-array excitation in the extended AAS model as in above Table.

In the current product design, pre-set sub-array down-tilt is a fixed parameter for an AAS base station. It is designed to tilt main beam of the sub-array to the intended coverage. Normally it uses passive circuits (e.g. transmission line) to design the needed phase of each radiated element within a sub-array. Hence the pre-set sub-array down-tilt does not vary with the environment. Meanwhile deployment configuration such as mechanical down-tilt is environment dependent, i.e. the performance can be further optimized for a specific environment by adjusting the mechanical down-tilt. Hence there is no need to design configurable sub-array down-tilt, which need to introduce additional RF component such as high power phase shifter, meaning higher cost and large size.

3	Conclusion

In this contribution we provide our proposal on the pre-set sub array tilt parameter for LS reply.

Proposal 1: It is proposed to rely WP5D with following text on pre-set sub-array

In the current product design, pre-set sub-array down-tilt is a fixed parameter for an AAS base station. It is designed to tilt main beam of the sub-array to the intended coverage. Normally it uses passive circuits (e.g. transmission line) to design the needed phase of each radiated element within a sub-array. Hence the pre-set sub-array down-tilt does not vary with the environment. Meanwhile deployment configuration such as mechanical down-tilt is environment dependent, i.e. the performance can be further optimized for a specific environment by adjusting the mechanical down-tilt. Hence there is no need to design configurable sub-array down-tilt, which need to introduce additional RF component such as high power phase shifter, meaning higher cost and large size.

References

[1] R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz” ITU-R WP 5D

[2] R4-2403653, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, RAN WG4