3GPP TSG-RAN WG4#110bis	R4-2405445

Changsha, China, April 15th – 19th, 2024

Agenda item:	5.1.1.1

Source: 	Qualcomm France

Title: 	Considerations on CA_n41C-n79A

Document for:	Approval

# Introduction

Considerations on CA_n41C-n79A are provided in this contribution.

- Background

TP for CA_n41C-n79A was noted in RAN4#110 [1].

The following MSD test point was discussed under Issues Arising-topic.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n79 | N/A | 40 | N/A | 4872.5 | 3.115 | TDD | IMD4 |
| NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

- Discussion

We analysed n41C IMD4 into n79 DL with the following results.

Resulting MSD assuming separate antennas or diplexer between aggressor TX path and victim n79 paths is 17.4dB.

Proposal 1: Use the following MSD among the inputs for specifying IMD4 MSD for CA_n41C-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n79 | N/A | 40 | N/A | 4872.5 | 17.415 | TDD | IMD4 |
| NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

- Conclusion

Considerations on CA_n41C-n79A were provided with the following proposal

Proposal 1: Use the following MSD among the inputs for specifying IMD4 MSD for CA_n41C-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n79 | N/A | 40 | N/A | 4872.5 | 17.415 | TDD | IMD4 |
| NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

- Reference

[1] R4-2401274, “TP for TR38.718-02-01_CA_n41A-n79C and CA_n41C-n79A”, ZTE Corporation, Mediatek