3GPP TSG-RAN WG4 Meeting #110bis                                                       R4-2405484

Changsha, China, 15th – 19th Apr. 2024

Agenda Item:	6.23.5

Source:	Ericsson

Title:	On demodulation and CSI reporting requirement for mIAB-MT

Document for:	Discussion

# 1	Introduction

The testing scope for mIAB-MT has been agreed to include PBCH, PDSCH, PDCCH and CQI reporting with the testing methodology of reusing all IAB-MT demodulation requirements except the newly selected cases from TS38.101-4 with similar configurations except channel model.

To select the legacy test cases properly, following parameter assumptions have been agreed [1] to better fit the Rel-18 mIAB-MT implementation scenario:

Issue 1-1-2: Frequency range Agreement: Both FR1 and FR2-1will be considered for mIAB-MT demodulation requirements. Issue 1-1-3: Duplex mode for FR1 requirements Agreement: Only TDD is considered for FR1 mIAB-MT requirements. Issue 1-2-2: FR1 PDSCH demodulation requirements selection from 38.101-4: Rank. Agreement: Only rank 1 is considered. Issue 1-2-3: FR1 PDSCH demodulation requirements selection from 38.101-4: Antenna configuration. Agreement: 2x4 for conducted and 2x2 for radiated requirements. Issue 1-3-1: FR1 PDCCH demodulation requirements selection from 38.101-4: Channel model Agreement: TDLC300-100.

In this contribution, we discussed the left parameter assumptions and provided our proposals on selecting the legacy test cases from 38.101-4.

# 2	Discussion

## 2.1 FR1 requirements

In this section, we discussed the selection of FR1 PDSCH, PDCCH and CQI requirement from 38.101-4 to fit Rel-18 mIAB testing purpose.

### 2.1.1 PDSCH

For modulation order and channel model, the following has been captured in the WF [1]:

Issue 1-2-1: FR1 PDSCH demodulation requirements selection from 38.101-4: Modulation order and Channel model. - Option 1: QPSK with TDLB100-400 - Option 2: 16QAM with TDLC300-100. Way forward: Both option 1 and option 2 are introduced, but option 2 replaces a corresponding existing test case.

For conducted requirement, based on the guidance from the Way forward above, we propose to replace the following existing test case 1-2 in Table 8.2.2.1.2-1 of TS38.174 with test case 1-4 in Table 5.2.3.2.1-3 of TS38.101-4. Test cases 1-1 in Table 5.2.3.2.1-3 can be added as an additional case.

Table 8.2.2.1.2-1: Minimum requirements for PDSCH Type A with Rank 1 Test number; FRC (Annex A); Bandwidth (MHz) / Subcarrier spacing (kHz); Propagation conditions (Annex I); Antenna configuration; Fraction of maximum throughput (%); SNR (dB). 1-1; M-FR1-A.3.3-1; 40/30; TDLA30-10; 2x4, ULA Low; 70; 21.6. 1-2; M-FR1-A.3.1-1; 40/30; TDLA30-10; 2x4, ULA Low; 30; -1.1. Table 5.2.3.2.1-3: Minimum performance for Rank 1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.2-1.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-1A; TDLB100-400; 2x4, ULA Low; 70; -4.1. 1-2; R.PDSCH.2-1.2 TDD; 40 / 30; QPSK, 0.30; FR1.30-1; TDLC300-100; 2x4, ULA Low; 70; -2.7. 1-3; R.PDSCH.2-4.1 TDD; 40 / 30; 256QAM, 0.82; FR1.30-1; TDLA30-10; 2x4, ULA Low; 70; 21.6. 1-4; R.PDSCH.2-2.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-100; 2x4, ULA Low; 30; -1.2. 1-5; R.PDSCH.2-5.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-2; TDLA30-10; 2x4, ULA Low; 70; -3.8. 1-6; R.PDSCH.2-6.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-3; TDLA30-10; 2x4, ULA Low; 70; -3.6. 1-7; R.PDSCH.2-10.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; HST-1000; 1x4; 70; 3.4. 1-8; R.PDSCH.2-11.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-5; TDLB100-400; 2x4, ULA Low; 70; -4.0. 1-9; R.PDSCH.2-12.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-6; TDLB100-400; 2x4, ULA Low; 70; -4.0. 1-10; R.PDSCH.2-10.2 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-1200; 2x4; 70; 5.8. 1-11; R.PDSCH.2-10.3 TDD; 40 / 30; 64QAM, 0.43; FR1.30-1; HST-1667; 1x4; 70; 6.8. 1-12; R.PDSCH.2-25.1 TDD; 40 / 30; 1024QAM, 0.79; FR1.30-1; TDLD30-5; 2x4, ULA Low; 70; 26.3.

According to the following Note 1 in Table 8.2.2.1.1-1, it is enough to only consider the UL-DL pattern: 7D1S2U, S=6D:4G:4U for Rel-18 mIAB demodulation requirement when selecting legacy requirement in TS38.101-4. Therefore, we propose to consider the TDD pattern: FR1.30-1 and FR1.30-1A.

Table: 8.2.2.1.1-1 Test parameters for testing PDSCH Parameter; Value. Cyclic prefix; Normal. Default TDD UL-DL pattern (Note 1); 7D1S2U, S=6D:4G:4U. HARQ; Maximum number of HARQ transmissions; 4. RV sequence; 0, 2, 3, 1. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. DM-RS position (l0); 2. Additional DM-RS position; pos1. Number of DM-RS CDM group(s) without data; 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests. DM-RS port(s); {1000} for Rank 1 tests {1000-1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests. DM-RS sequence generation; NID0=0. Time domain resource assignment; PDSCH mapping type; A. Start symbol; 2. Allocation length; 12. Frequency domain resource assignment; RB assignment; Full applicable test bandwidth. PT-RS configuration; Not configured. PRB bundling size; 2. VRB-to-PRB mapping type; Not interleaved. PDSCH & PDSCH DMRS Precoding configuration; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Note 1: The same requirements are applicable to TDD with different UL-DL patterns..

Proposal 1: Replace the test case 1-2 in Table 8.2.2.1.2-1 of TS38.174 with test case 1-4 in Table 5.2.3.2.1-3 of TS38.101-4.

Proposal 2: Select additional test case 1-1 from Table 5.2.3.2.1-3 of 38.101-4 to fulfil the test need for TDLB100-400 with QPSK

Proposal 3: Consider the TDD pattern: FR1.30-1 and FR1.30-1A

For radiated requirement, we propose to replace the following blue highlighted test case 2 in Table 11.2.2.1.1.2-1 of TS38.174 with yellow highlighted test case 1-4 in Table 5.2.2.2.1-3 of TS38.101-4. Test cases 1-1 in Table 5.2.2.2.1-3 can be added as an additional case.

Table 11.2.2.1.1.2-1: Minimum requirements for PDSCH with Rank 1, 40 MHz Channel Bandwidth, 30 kHz SCS Number of TX antennas; Number of demodulation branches; Propagation conditions and correlation matrix (Annex I); FRC (Annex A); Fraction of maximum throughput (%); SNR (dB). 2; 2; TDLA30-10, ULA Low; M-FR1-A.3.3-1; 70; 25.3. TDLA30-10, ULA Low; M-FR1-A.3.1-1; 30; 2.2. Table 5.2.2.2.1-3: Minimum performance for Rank 1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.2-1.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-1A; TDLB100-400; 2x2, ULA Low; 70; -1.1. 1-2; R.PDSCH.2-1.2 TDD; 40 / 30; QPSK, 0.30; FR1.30-1; TDLC300-100; 2x2, ULA Low; 70; 0.2. 1-3; R.PDSCH.2-4.1 TDD; 40 / 30; 256QAM, 0.82; FR1.30-1; TDLA30-10; 2x2, ULA Low; 70; 25.3. 1-4; R.PDSCH.2-2.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-100; 2x2, ULA Low; 30; 1.6. 1-5; R.PDSCH.2-5.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-2; TDLA30-10; 2x2, ULA Low; 70; -0.9. 1-6; R.PDSCH.2-6.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-3; TDLA30-10; 2x2, ULA Low; 70; -0.8. 1-7; R.PDSCH.2-10.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; HST-1000; 1x2; 70; 6.4. 1-8; R.PDSCH.2-11.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-5; TDLB100-400; 2x2, ULA Low; 70; -1.0. 1-9; R.PDSCH.2-12.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-6; TDLB100-400; 2x2, ULA Low; 70; -1.1. 1-10; R.PDSCH.2-10.2 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-1200; 2x2; 70; 9.5. 1-11; R.PDSCH.2-10.3 TDD; 40 / 30; 64QAM, 0.43; FR1.30-1; HST-1667; 1x2; 70; 9.6. 1-12; R.PDSCH.2-25.1 TDD; 40 / 30; 1024QAM, 0.79; FR1.30-1; TDLD30-5; 2x2, ULA Low; 70; 29.4.

Proposal 4: Replace the test case 2 in Table 11.2.2.1.1.2-1 of TS38.174 with test case 1-4 in Table 5.2.2.2.1-3 of TS38.101-4.

Proposal 5: Select additional test case 1-1 from Table 5.2.2.2.1-3 of 38.101-4 to fulfil the test need for TDLB100-400 with QPSK

### 2.1.2 PDCCH

As for the PDCCH requirement, similar approach should be also applied so that the legacy requirement for IAB-MT in TS38.174 should be reused. Some particular test cases are expected to be selected from TS38.101-4 to fulfil the need of testing higher Doppler. Those are suggested to replace the test cases in 38.174 that have similar parameter configurations.

For FR1 conducted requirements, 1T4R has been agreed for antenna configuration, FFS on 2T4R.

The following highlighted 1T4R test case is preferred to be selected from TS38.101-4:

Table 5.3.3.2.1-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 102; 1; 2; R.PDCCH. 2-1.1 TDD; TDLA30-10; 1x4 Low; 1; 2.1. 2; 40; 102; 1; 4; R.PDCCH. 2-1.2 TDD; TDLC300-100; 1x4 Low; 1; -0.9. 3; 40; 48; 2; 16; R.PDCCH. 2-2.1 TDD; TDLA30-10; 1x4 Medium A; 1; -3.6.

The test cases above should replace the following highlighted 1T4R test case in TS38.174:

Table 8.2.2.2.2-1: Minimum requirements for PDCCH Test number; Bandwidth (MHz) / Subcarrier spacing (kHz); CORESET RB; CORESET duration; Aggregation level; FRC (Annex A); Propagation conditions (Annex I); Antenna configuration; Pm-dsg (%); SNR (dB). 1; 40/30; 102; 1; 2; M-FR1-A.3.4-1; TDLA30-10; 1x4, ULA Low; 1; 2.1. 2; 40/30; 102; 1; 4; M-FR1-A.3.4-2; TDLA30-10; 1x4, ULA Low; 1; 0.7. 3; 40/30; 90; 1; 8; M-FR1-A.3.4-3; TDLA30-10; 2x4, ULA Low; 1; -4.1.

In the meantime, we propose to consider 2T4R requirements as well.

The following highlighted 2T4R test case is preferred to be selected from TS38.101-4:

Table 5.3.3.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x4 Low; 1; -4.3.

The test cases above should replace the following highlighted 2T4R test case in TS38.174:

Table 8.2.2.2.2-1: Minimum requirements for PDCCH Test number; Bandwidth (MHz) / Subcarrier spacing (kHz); CORESET RB; CORESET duration; Aggregation level; FRC (Annex A); Propagation conditions (Annex I); Antenna configuration; Pm-dsg (%); SNR (dB). 1; 40/30; 102; 1; 2; M-FR1-A.3.4-1; TDLA30-10; 1x4, ULA Low; 1; 2.1. 2; 40/30; 102; 1; 4; M-FR1-A.3.4-2; TDLA30-10; 1x4, ULA Low; 1; 0.7. 3; 40/30; 90; 1; 8; M-FR1-A.3.4-3; TDLA30-10; 2x4, ULA Low; 1; -4.1.

For FR1 radiated requirements, 1T2R has been agreed for antenna configuration, and we propose to consider 2T2R as well.

The following highlighted test cases are preferred to be selected from TS38.101-4:

Table 5.3.2.2.1-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 102; 1; 2; R.PDCCH. 2-1.1 TDD; TDLA30-10; 1x2 Low; 1; 7.0. 2; 40; 102; 1; 4; R.PDCCH. 2-1.2 TDD; TDLC300- 100; 1x2 Low; 1; 3.0. 3; 40; 48; 2; 16; R.PDCCH. 2-2.1 TDD; TDLC300- 100; 1x2 Low; 1; -3.8. ##### 5.3.2.2.2 2 Tx Antenna performances For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.2-1. The downlink physical setup is in accordance with Annex C.3.1. Table 5.3.2.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x2 Low; 1; -1.2.

The test cases above should replace the following highlighted 1T2R, 2T2R test case in TS38.174:

Table 11.2.2.1.2.2-1: Minimum requirements for PDCCH, 40 MHz Channel Bandwidth, 30 kHz SCS Antenna configuration; CORESET RB; CORESET duration; Aggregation level; FRC (Annex A); Propagation conditions and correlation matrix (Annex I); Pm-dsg (%); SNR (dB). 1x2; 102; 1; 2; M-FR1-A.3.4-1; TDLA30-10, Low; 1; 7.0. 1x2; 102; 1; 4; M-FR1-A.3.4-1; TDLA30-10, Low; 1; 4.9. 2x2; 90; 1; 8; M-FR1-A.3.4-1; TDLA30-10, Low; 1; -0.7.

Proposal 6: Consider the following PDCCH test case design:

|  | Antenna | Test case in 38.174 | Replaced by | Test case in 38.101-4 |
| --- | --- | --- | --- | --- |
|  | 1x4 | Test num.2 in Table 8.2.2.2.2-1 |  | Test num.2 in Table 5.3.3.2.1-1 |
|  | 2x4 | Test num.3 in Table 8.2.2.2.2-1 |  | Test num.1 in Table 5.3.3.2.2-1 |
|  | 1x2 | 1x2 test case in Table 11.2.2.1.2.2-1 |  | Test num.2 in Table 5.3.2.2.1-1 |
|  | 2x2 | 2x2 test case in Table 11.2.2.1.2.2-1 |  | Test num.1 in Table 5.3.2.2.2-1 |

Based on the selected cases above, the corresponding antenna configuration, CORESET duration, and aggregation level is proposed as the following respectively:

Proposal 7: Consider the following FR1 PDCCH parameter configurations:

- Antenna configuration

  - Both 1T4R and 2T4R for conducted requirements

  - Both 1T2R and 2T2R for radiated requirements

- CORESET duration

  - 1

- Aggregation level

  - Both 4 and 8

## 2.2 FR2 requirements

In this section, we discussed the selection of FR2 PDSCH, PDCCH and CQI requirement from 38.101-4 to fit Rel-18 mIAB testing purpose.

### 2.2.1 PDSCH

For PDSCH FR2 requirements, following yellow highlighted test cases from TS38.101-4 with higher Doppler value are proposed to be selected, and one of which can replace the blue highlighted test case in TS38.174:

The following yellow highlighted test cases are preferred to be selected from TS38.101-4:

Table 7.2.2.2.1-3: Minimum performance for Rank 1 (FRC) for FR2-1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNRBB (dB). 1-1; R.PDSCH.5-1.1 TDD; 100 / 120; QPSK, 0.30; FR2.120-1A; TDLC60-300; 2x2 ULA Low; 70; -0.4. 1-2; R.PDSCH.5-2.1 TDD; 100 / 120; 16QAM, 0.48; FR2.120-1; TDLA30-300; 2x2 ULA Low; 30; 1.7. 1-3; R.PDSCH.5-3.1 TDD; 100 / 120; 64QAM, 0.46; FR2.120-1; TDLA30-300; 2x2 XPL Medium; 70; 12.4. 1-4; R.PDSCH.5-10.1 TDD; 50 / 120; 256QAM 0.67; FR2.120-1; TDLD30-75; 2x2 ULA Low; 70; 20.2.

In the above, the test num. 1-2 can be used to replace the following existing test case in TS38.174, for which is using the same modulation order (16QAM)

Table 11.2.2.2.1.2-1: Minimum requirements for PDSCH with Rank 1, 100 MHz Channel Bandwidth, 120 kHz SCS Number of TX antennas; Number of demodulation branches; Propagation conditions and correlation matrix (Annex I); FRC (Annex A); Fraction of maximum throughput (%); SNR (dB). 2; 2; TDLA30-75, ULA Low; M-FR2-A.3.1-2; 30; 2.3. 2; 2; TDLA30-75, ULA Low; M-FR2-A.3.2-1; 70; 11.7.

Proposal 8: Replace the first test case in Table 11.2.2.2.1.2-1 of TS38.174 with test case 1-2 in Table 7.2.2.2.1-3 of TS38.101-4.

Proposal 9: Select additional test case 1-3 from Table 7.2.2.2.1-3 of 38.101-4 to cover 64QAM.

Based on the selected cases above, the corresponding rank, modulation order and the TDD pattern is proposed respectively as follows.

Proposal 10: Consider the following FR2 PDSCH parameter configurations.

- Rank

  - Rank 1 only

- Modulation order

  - 16QAM and 64QAM

- TDD pattern

  - FR2 120-1, DDDSU, S=10D2G2U

### 2.2.2 PDCCH

For PDCCH FR2 requirements, the following yellow highlighted test case from TS38.101-4 with higher Doppler value are proposed to replace the blue highlighted test case in TS38.174.

The following yellow highlighted test case is preferred to be selected from table 7.3.2.2.1-1 in TS38.101-4 to replace the blue highlighted test case from table 11.2.2.2.2.2-1 in TS38.174:

Table 7.3.2.2.1-1: Minimum performance requirements with 120 kHz SCS for FR2-1 Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNRBB (dB). 1-1; 100; 60; 1; 2; R.PDCCH. 5-1.1 TDD; TDLA30-75; 1x2 Low; 1; 6.4. 1-2; 100; 60; 1; 4; R.PDCCH. 5-1.2 TDD; TDLA30-300; 1x2 Low; 1; 3.0. Table 11.2.2.2.2.2-1: Minimum requirements for PDCCH, 100 MHz Channel Bandwidth, 120 kHz SCS Antenna configuration; CORESET RB; CORESET duration; Aggregation level; FRC (Annex A); Propagation conditions and correlation matrix (Annex I); Pm-dsg (%); SNR (dB). 1x2; 60; 1; 2; M-FR2-A.3.4-1; TDLA30-75, ULA Low; 1; 6.4. 1x2; 60; 1; 4; M-FR2-A.3.4-2; TDLA30-75, ULA Low; 1; 2.9. 2x2; 60; 1; 8; M-FR2-A.3.4-3; TDLA30-75, ULA Low; 1; 0.1.

Based on the above selected test case in TS38.101-4, we propose the following FR2 PDCCH parameter configurations:

Proposal 11: Consider the following FR2 PDCCH parameter configurations:

- Antenna configuration

  - 1T2R

- CORESET duration

  - 1

- Aggregation level

  - 4

## 2.3 Legacy CQI reporting requirement

According to the agreed WF [1], the following agreement has been reached:

Issue 1-4-1: Requirement selection of CSI reporting from 38.101-4 Agreement: Only select CQI report requirement with fading channel for mIAB-MT.

Based on the above, we propose to select following legacy CQI reporting requirement from TS38.101-4:

Proposal 12: Select the following legacy CQI reporting requirements:

- Wideband CQI reporting

  - Conducted:

    - All tests in section 6.2.3.2.2.1.

  - Radiated:

    - All tests in section 8.2.2.2.2.1

- Sub-band CQI reporting

  - Conducted:

    - All tests in section 6.2.3.2.2.2.

## 2.4 Legacy PBCH requirement

According to the agreed WF [1], the following agreement has been reached:

Issue 1-1-1: Test case scope Agreement: Select PBCH demodulation requirements from 38.101-4 for mIAB-MT.

Based on the above, we propose to select following legacy PBCH requirement from TS38.101-4:

Proposal 13: Select the following legacy PBCH requirements:

- Conducted

  - All tests in section 5.4.3.2

- Radiated

  - All tests in section 7.4.2.2

# 3	Summary

In this contribution, we gave our proposals on how to select the legacy test cases including FR1 and FR2 for PDSCH, PDCCH, PBCH and CQI reporting.

In the end, we summarized our proposals as follows:

Proposal 1: Replace the test case 1-2 in Table 8.2.2.1.2-1 of TS38.174 with test case 1-4 in Table 5.2.3.2.1-3 of TS38.101-4.

Proposal 2: Select additional test case 1-1 from Table 5.2.3.2.1-3 of 38.101-4 to fulfil the test need for TDLB100-400 with QPSK

Proposal 3: Consider the TDD pattern: FR1.30-1 and FR1.30-1A

Proposal 4: Replace the test case 2 in Table 11.2.2.1.1.2-1 of TS38.174 with test case 1-4 in Table 5.2.2.2.1-3 of TS38.101-4.

Proposal 5: Select additional test case 1-1 from Table 5.2.2.2.1-3 of 38.101-4 to fulfil the test need for TDLB100-400 with QPSK

Proposal 6: Consider the following PDCCH test case design:

|  | Antenna | Test case in 38.174 | Replaced by | Test case in 38.101-4 |
| --- | --- | --- | --- | --- |
|  | 1x4 | Test num.2 in Table 8.2.2.2.2-1 |  | Test num.2 in Table 5.3.3.2.1-1 |
|  | 2x4 | Test num.3 in Table 8.2.2.2.2-1 |  | Test num.1 in Table 5.3.3.2.2-1 |
|  | 1x2 | 1x2 test case in Table 11.2.2.1.2.2-1 |  | Test num.2 in Table 5.3.2.2.1-1 |
|  | 2x2 | 2x2 test case in Table 11.2.2.1.2.2-1 |  | Test num.1 in Table 5.3.2.2.2-1 |

Proposal 7: Consider the following FR1 PDCCH parameter configurations:

- Antenna configuration

  - Both 1T4R and 2T4R for conducted requirements

  - Both 1T2R and 2T2R for radiated requirements

- CORESET duration

  - 1

- Aggregation level

  - Both 4 and 8

Proposal 8: Replace the first test case in Table 11.2.2.2.1.2-1 of TS38.174 with test case 1-2 in Table 7.2.2.2.1-3 of TS38.101-4.

Proposal 9: Select additional test case 1-3 from Table 7.2.2.2.1-3 of 38.101-4 to cover 64QAM.

Proposal 10: Consider the following FR2 PDSCH parameter configurations.

- Rank

  - Rank 1 only

- Modulation order

  - 16QAM and 64QAM

- TDD pattern

  - FR2 120-1, DDDSU, S=10D2G2U

Proposal 11: Consider the following FR2 PDCCH parameter configurations:

- Antenna configuration

  - 1T2R

- CORESET duration

  - 1

- Aggregation level

  - 4

Proposal 12: Select the following legacy CQI reporting requirements:

- Wideband CQI reporting

  - Conducted:

    - All tests in section 6.2.3.2.2.1.

  - Radiated:

    - All tests in section 8.2.2.2.2.1

- Sub-band CQI reporting

  - Conducted:

    - All tests in section 6.2.3.2.2.2.

Proposal 13: Select the following legacy PBCH requirements:

- Conducted

  - All tests in section 5.4.3.2

- Radiated

  - All tests in section 7.4.2.2

# References

[1]	R4-2402871: WF on [110][326] NR_mobile_IAB_demod, Ericsson, RAN4 #110, Athens, Greece