3GPP TSG-RAN WG4 Meeting #110                                          R4-2405489

Athens, Greece, 26 Feb – 01 Mar, 2024

Source: 	Huawei, HiSilicon

Title: 	On basket WI TR and WID improvement

Agenda Item:	11.3

Document for:	Approval

Introduction

In order to study the band combination specific requirements more efficiently, basket procedure was introduced from Rel-12 for LTE CA. After the practice of several releases, the basic TR skeleton is almost the same as those adopted in Rel-12. For years, the TRs for CA/DC have supported numerous band combinations in RAN4 and they are the basis for deriving the corresponding requirements in the specification. However, one obvious drawback is that the detailed analysis including the reference architectures, assumed parameters in the calculation for the MSD values, specific consideration for a particular combination, etc. are not recorded in the TR, which makes the TRs less useful but tedious repetition in the end during the study of basket WIs.

There are a lot of efforts to improve the block approval process, and some efforts are also extended to the improvement of the TP templates for Rel-19 basket WIs [1][2]. In order to make the TRs for basket WIs more technically useful, further adjustment and improvement of the TR structure as well as ways to improve the TR quality for basket WI study are necessary.

Since the Rel-18 basket WIs are supposed to be extended to June, more discussion would be helpful to have a more meaningful and better TR for the upcoming new release. Specifically, in this contribution we discuss how to improve the TR quality of MSD analysis and also the consideration for Rel-19 basket WID based on the long debating release independent spec impact during the RAN meeting.

Discussion

How to derive the MSD requirements

We use the Rx requirements in TS 38.101-1 as example for the following discussion. Three categories of MSD requirements are considered in the specification, i.e. harmonic MSD (including harmonic and harmonic mixing), IMD MSD (including 2 bands/3 bands and triple beat) and cross band isolation MSD. MSD values for some of the band combinations in the spec are copied as below.

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n1 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |

able 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n1 | n3 | 1922.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n1 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 19.7 | ACLR1 |

Recently, lots of MSD evaluations are provided under Rel-18 UE RF FR1 WI [3]. The TR for lower MSD recorded analysis of different MSD types for the selected band combinations among companies. From the analysis, it can be seen that two aspects are necessary for the MSD calculation, though for different MSD types the utilized methodology could be slightly different. These two aspects are:

- Reference UE architecture

- Assumptions for the passive/active RF components

Regarding UE architectures, for a same band combination, companies may select different architectures for the analysis, e.g. some may use duplexer + diplexer, but others may use triplexer instead. In that case, the usual way adopted in RAN4 is to average the inputs from companies if the analysis itself is deemed correct.

Observation 1: For MSD analysis, the reference UE architecture, assumptions for RF components are important for the final requirements.

Though the above two aspects are important for the MSD analysis, however, they are not captured anywhere, not to mention the way adopted for the final values if several inputs/different reference architectures are provided for the band combination.

Observation 2: The analysis procedure is not recorded in the technical report though they are the most valuable part for the MSD requirements from technical point of view.

Sometimes, due to the develop of the real implementation, it could also identify that the requirements defined in the spec may not be appropriate with incorrect assumptions for some specific band combinations, the group may revisit the previously specified requirements. But at that time, the retrospect is very difficult as digging the previous discussion is really a time-consuming physical work.

Observation 3: Retrospect the MSD requirements sometimes in the group due to identified issues with development of the specific band combination is difficult since the analysis procedure is missing in the TR.

Existing TR structure for combinations with MSD

TR 38.718-02-01 for Inter-band Carrier Aggregation/Dual Connectivity for2 bands DL with x bands UL (x=1,2) and TR 37.718-11-11 for Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) are taken for instance to see the typical TR structure for the existing basket WIs.

Inter-band CA

The TOC structure below is the one used in TR 38.718-02-01. Certainly, the band and CBW info are necessary for a band combination. Apart from that, in the sections related to the final requirements, usually the content in the TR is just the mirror part as that in the spec.

It is noted that in Rel-16 when companies initially discussed to optimize the TR content, it was agreed that the co-existence table may not be necessary to be captured as the manner we still use in the current TR, as it could not provide more useful information for specifying the requirements. Most importantly is to identify what are the possible issues which could have impact on the final requirements, e.g. to make it clear whether MSD exists for the band combination, and if it has, what are the possible MSD types to by analyzed.

Regarding delta Tib and Rib values, after completion of hundreds of thousands of band combinations from Rel-15, nowadays, usually for most of new proposed combinations, reference values can be found in the specification. The issue is, can we find some general rules to specify the values without analysis in reality that so many combinations have been specified? Though for some specific combinations, the rule based automatic applied delta Tib and Rib values may have some issues for the implementation, can we treat them as exceptions and correct the values in maintenance stage?

As for the REFSENS requirements part, we can only see the final values captured in the TR, while how to derive the values, there is no hint to track the analysis.

On the other hand, though the preparation of the TP is stereotyped, following the same pattern, and time consuming, the TR itself provides less useful information from the technical point. It is also a heavy workload for the rapporteur to combine the TPs in the TR especially if there are many TPs approved in the same meeting. Since so much energy has been spent for the TR, can we do it better and make it more useful for the following study and make the TR a real technical report? We think it’s the right time to make some changes for the TRs during the study of the basket WIs.

Observation 4: The TRs for existing basket WIs do not provide sufficient information with technical analysis for the suggested values if MSD is identified for the band combination.

Inter-band DC

The TOC structure below is the one used in TR 37.718-11-11. The issues are similar to those discussed in the above sub-section.

It is noted that band combinations for HPUE are studied in separate basket WIs, maximum output power info is not necessary for PC3 band combinations.

Improvement for the TR quality

With above analysis, the current TP does not provide sufficient information from technical perspective in terms of a TR. Recently, there are a lot of efforts in the group to improve the block approval process and some candidate templates are also provided for the TPs. We think it’s the right time to re-consider the TR structure to include more useful information based on the technical analysis for band combinations.

An example structure was provided before for reference [7], but we think more discussion is needed. The final TR structure should be a balanced tradeoff between the efficiency of preparing the TPs and quality of the technical report.

Proposal 1: It is proposed to improve the TR structure for basket WI to include more technical information and simplify the content which are not essential for specifying the requirements and duplicated as in the spec from Rel-19.

Proposal 2: It is proposed to capture at least the references, if not the content, for the architectures, WF, discussion paper with proposals which are utilized for final averaging for deriving the MSD requirements.

Proposal 3: It is proposed to have a systematic solution for the diversified delta Tib and Rib values for different kinds of band combinations. Simplified rules-based delta Tib and Rib values are preferred rather than case by case tedious practice.

Spec impact for release independent issue in WID

Spec impact for release independent is a long existing issue for basket WIs. One issue confused rapporteurs and MCC often is whether release independent specification is considered as perf part or core part, which has impact to both WID as well as the SR. The clarification from MCC is clear, that is:

If the REL-independent requirement is a Core part requirement, then the REL-independent spec is impacted by the Core part.

If the REL-independent requirement is a Perf. part requirement, then the REL-independent spec is impacted by the Perf. part.

From the release independent specification, we do see both core requirements and perf requirements are considered in the specification, e.g. the requirements highlighted in yellow are perf related in the following table. However, for the referred requirements for band(s)/band combinations, those are core requirements in TS 3x.101 indeed.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RRM requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.1-1 | Rel-16 WI NR_HST introduced band independent RRM requirements: see Table C.1-1 |
| UE demodulation requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.2-1 | Rel-16 WI NR_HST introduced band independent UE demodulation requirements: see Table C.2-1 |
| RF requirements for 4Rx UEs | Rel-15 | Table B.4.10-1, Table B.4.10-2 |  |
| [Transparent Tx diversity] | [Rel-15] | [Table B.4.11-1] | [Rel-17 WI NR_RF_TxD introduced transparent Tx diversity requirements: see Table B.4.11-1] |
| UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference | Rel-15 | Table B.3.3-1 | Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration. |
| RRM enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 2) | Table C.3-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent RRM enhancement: see Table C.3-1 |
| HST-SFN CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 3) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-SFN CA demodulation enhancement: see Table C.4-1 |
| HST-DPS CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-15 (NOTE 4) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-DPS CA demodulation enhancement: see Table C.4-1 |

Observation 5: For most spectrum related WIs, the release independent requirements are core requirements rather than perf requirements.

Given the observation and the inconsistent issues identified and discussed in RAN plenary meeting, we have the following proposals to improve the handling of release independent issue for spectrum/basket WIs for future releases.

Proposal 4: It is proposed to start discussion on the draft WID for release 19 with consistent format which can cover specific issues for the corresponding WIs but with unified style in order to avoid inconsistent handling for the following SR.

Proposal 5: In general, if the configurations in a spectrum/basket WI have been included in the TS 3x.307 already, no need to include the release independent spec in the affected spec list in the WID. For this case, some clarification would be needed in the WID to mention the introduced band(s) or CA/DC combinations in the WI can be release independent from which release.

Proposal 6: If it is not clear whether release independent CR would be needed when the spectrum/basket WI is established, TS 3x.307 can still be listed as one of the affected spec in the WID.

Proposal 7: The TS 3x.307 should not be listed as perf spec without careful checking whether the affected requirements are core part of perf part.

Conclusion

This contribution provides consideration and proposals on improving the TR quality for basket WI and issue of spec impact for release independency in the WID for Rel-19.

Observation 1: For MSD analysis, the reference UE architecture, assumptions for RF components are important for the final requirements.

Observation 2: The analysis procedure is not recorded in the technical report though they are the most valuable part for the MSD requirements in terms of technical view.

Observation 3: Retrospect the MSD requirements sometimes in the group due to identified issues with development of the specific band combination is difficult since the analysis procedure is missing in the TR.

Observation 4: The TRs for existing basket WIs do not provide sufficient information with technical analysis for the suggested values if MSD is identified for the band combination.

Proposal 1: It is proposed to improve the TR structure for basket WI to include more technical information and simplify the content which are not essential for specifying the requirements and duplicated as in the spec from Rel-19.

Proposal 2: It is proposed to capture at least the references, if not the content, for the architectures, possible WF, discussion paper with proposals which are utilized for final averaging for deriving the MSD requirements.

Proposal 3: It is proposed to have a systematic solution for the diversified delta Tib and Rib values for different kinds of band combinations. Simplified rules-based delta Tib and Rib values are preferred rather than case by case tedious practice.

Observation 5: For most spectrum related WIs, the release independent requirements are core requirements rather than perf requirements.

Proposal 4: It is proposed to start discussion on the content of draft WID for release 19 with consistent format which can cover specific issues for the corresponding WIs but with unified style in order to avoid inconsistent handling for the following SR.

Proposal 5: In general, if the configurations in a spectrum/basket WI have been included in the TS 3x.307 already, no need to include the release independent spec in the affected spec list in the WID. For this case, some clarification would be needed in the WID to mention the introduced band(s) or CA/DC combinations in the WI can be release independent from which release.

Proposal 6: If it is not clear whether release independent CR would be needed when the spectrum/basket WI is established, TS 3x.307 can still be listed as one of the affected Spec in the WID.

Proposal 7: The TS 3x.307 should not be listed as perf spec without careful checking whether the affected requirements are core part of perf part.

- References

[1] R4-2400257, Further improvements to the block approval process in R19, Skyworks Solutions Inc., Nokia

[2] R4-2403721, WF on Rel-19 Band Combination Work, Nokia, et al.

[3] TR 38.881, Lower MSD for inter-band CA EN-DC DC combinations

[4] TR 38.718-02-01 for Inter-band Carrier Aggregation/Dual Connectivity for2 bands DL with x bands UL (x=1,2)

[5] TR 37.718-11-11 for Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL)

[6] TR 38.846, Study on simplification of band combination specification for NR and LTE

[7] R4-2302379, Restructure TR for basket WI with MSD analysis, Huawei, HiSilicon