3GPP TSG-RAN4 Meeting # 110-bis	R4-2405511

# Changsha, China, 15 - 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS38.133 Bandiwidth aggregation for positioning core requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To correct the core requirements for bandwidth aggregation for positioning mesurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Core requirement when UE is configured to perform PRS-RSRPP together with RSTD and UE Rx-Tx time difference measurement is clarified. - Brackets around IEs are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Core requirement when UE is configured to perform PRS-RSRPP together with RSTD or UE Rx-Tx time difference measurement by aggregatig PRS resources from multiple PFLs is not defined. - Applicability of IEs remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.6, 5.6.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

#### 5.6.2.6	Measurements Period Requirements with Bandwidth Aggregation

The requirements in this clause apply provided that UE receives requests from LMF to perform PRS measurement on aggregated positioning frequency layers (PFLs) via nr-DL-PRS-JointMeasurementRequested [34].

After receiving both NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,total= TRSTD, aggr+ TRSTD,non-aggr + Tmargin

Where:

-	TRSTD, aggr is the total measurement period for aggregated measurements, and

-	TRSTD,non-aggr is the total measurement period for non-aggregated measurements, and

-	When both TRSTD, aggr  and TRSTD,non-aggr are non-zero, Tmargin=maxTeffect,j, where the maximum Teffect,j is across all the PFL combination(s) and non-aggregated PFL(s) configured for positioning measurements; otherwise Tmargin is equal to zero.

TRSTD,non-aggr is zero if every resource set on every PFL is linked for aggregation to at least one other resource set on another PFL. Otherwise, TRSTD,non-aggr is as defined in clause 5.6.2.5 except that

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

TRSTD,aggr is zero if no resourse sets on any PFL are linked for aggregation with other resource sets on other PFLs. Otherwise, TRSTD,aggr is defined as

TRSTD,aggr= m=1MTRSTD,aggr,m+ M-1*maxTeffect,aggr,m

where:

-	m is the index of PFL combination,

-	M is total number of PFL combinations,

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

-	TRSTD,aggr,m is the measurement period for PRS RSTD measurement in PFL combination m as specified below.

TRSTD,aggr,m=Kcarrier,aggr*NRx,TEG,aggr,m*NRxBeam,aggr,m*NPRS,aggr,mslotNaggr,m'*LavailablePRS,aggr,mNaggr,m*Nsample-1*Teffect,aggr,m+Tlast,aggr,m

where:

-	Kcarrier,aggr is a scaling factor for PRS measurements in RRC_INACTIVE,

-	Kcarrier,aggr=1 if the UE supports parallelPRS-MeasRRC-Inactive-r17,

-	otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier,aggr=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier,aggr=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,aggr,m is a scaling factor for PRS measurements with multiple Rx TEGs

-	NRx,TEG,aggr,m =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,aggr,m is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NRxBeam,aggr,m is a scaling factor for PRS measurements with multiple Rx beams, and is defined as

-	NRxBeam,aggr,m = 1 if PFL combination m is in FR1,

-	NRxBeam,aggr,m is defined as follows if PFL combination m is in FR2

-	NRxBeam,aggr,m equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing PFL combination m, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation,

-	NRxBeam,aggr,m equals to 8 otherwise

-	NPRS,aggr,mslot is the maximum number of DL PRS resources in PFL combination m configured in a slot, and only the PRS resources in resource set(s) linked to other resource set in PFL combination m are counted

-	Naggr,m' is the UE capability on maximum number of DL PRS resources that can be processed in a slot for PFL combination m as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18 for FR1 and maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18 for FR2 specified in TS 37.355 [34].

-	LavailablePRS,aggr,m is the time duration of available PRS resources in PFL combination m to be measured during TPRS,aggr,m, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,aggr,m, only unmuted PRS resources in resource set(s) linked to other resource set in PFL combination m and that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Naggr,m is the UE capability on duration of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18 specified in TS 37.355 [34].

-	Nsample is number of PRS measurement samples,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	Nsample= 4 otherwise.

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

Teffect,aggr,m=Taggr,mTavailable,PRS,aggr,m*Tavailable,PRS,aggr,m

-	Taggr,m is the UE capability on time for processing of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsT specified in TS 37.355 [34].

-	Tavailable,PRS,aggr,m=LCMTPRS,aggr,m,TDRX, the least common multiple between TPRS,aggr,m and the DRX cycle length TDRX, where TPRS,aggr,m is the periodicity of DL PRS resource with muting on PFL combination m.

-	If more than one PRS periodicities are configured in PFL combination m, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets that are linked to other resource set in PFL combination m, is used to derive TPRS,aggr,m, and for each applicable PRS resource set,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, and

-	TperPRS is the periodicity of PRS resource set given by the higher-layer parameter DL-PRS-Periodicity, and

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where 	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Tlast,aggr,m is the measurement duration for the last PRS sample in PFL combination m, including the sampling time and processing time, Tlast,aggr,m=Taggr,m+Tavailable,PRS,aggr,m.

If PRS resources in one or more of the aggregated PFLs in PFL combination m are dropped because of collision with other signals, TRSTD,aggr,m can be longer than specified above.

The time TRSTD,Total starts from the first DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

Note:	No separate requirement on aggregated measurement based on TRSTD,non-aggr or on non-aggregated measurement based on TRSTD,aggr is applied.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

When PRS-RSRPP is configured for DL-TDOA, RSTD and PRS-RSRPP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6.1.

Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL or any PFL combination during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.6.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the RSTD measurement period then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.

END OF CHANGE 1

START OF CHANGE 2

#### 5.6.4.6	Measurement Period Requirements with Bandwidth Aggregation

The requirements in this clause apply provided that UE receives requests from LMF to perform PRS measurement on aggregated positioning frequency layers (PFLs) via nr-DL-PRS-JointMeasurementRequested [34].

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.4.3) UE Rx-Tx time difference measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TUERxTx,total= TUERxTx, aggr+ TUERxTx,non-aggr + Tmargin

Where:

-	TUERxTx, aggr is the total measurement period for aggregated measurements, and

-	TUERxTx,non-aggr is the total measurement period for non-aggregates measurements, and

-	Tmargin is maxTeffect,j, if both TRSTD, aggr  and TRSTD,non-aggr are non-zero, where Teffect,j is the Teffect from both PFL combination(s) and non-aggregated PFL(s) which are configured for positioning measurement; otherwise Tmargin equals to zero.

TUERxTx,non-aggr is zero if every resourse set on every PFL is linked for aggregation with at least one other resource set on another PFL. Otherwise, TUERxTx,non-aggr is as defined in clause 5.6.4.5 except that

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

TUERxTx,aggr is zero if no resourse sets on any PFL are linked for aggregation with other resource sets on other PFLs. Otherwise, TUERxTx,aggr is defined as

TUERxTx,aggr= m=1MTUERxTx,aggr,m+ M-1*maxTeffect,aggr,m

where:

-	m is the index of PFL combination,

-	M is total number of PFL combinations,

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

-	TUERxTx,aggr,m is the measurement period for UE Rx-Tx time difference measurement in PFL combination m as specified below.

TUERxTx,aggr,m=Kcarrier,aggr*NRx,TEG,aggr,m*NRxBeam,aggr,m*NPRS,aggr,mslotNaggr,m'*LavailablePRS,aggr,mNaggr,m*Nsample-1*Teffect,aggr,m+Tlast,aggr,m

where:

-	Kcarrier,aggr is a scaling factor for PRS measurements in RRC_INACTIVE, and is defined as TBD,

-	Kcarrier,aggr=1 if the UE supports parallelPRS-MeasRRC-Inactive-r17,

-	otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier,aggr=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier,aggr=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,aggr,m is a scaling factor for PRS measurements with multiple Rx TEGs, and is defined as TBD,

-	NRx,TEG,aggr,m =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,aggr,m is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NRxBeam,aggr,m is a scaling factor for PRS measurements with multiple Rx beams, and is defined as

-	NRxBeam,aggr,m = 1 if PFL combination m is in FR1,

-	NRxBeam,aggr,m is defined as follows if PFL combination m is in FR2

-	NRxBeam,aggr,m equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing PFL combination m, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation,

-	NRxBeam,aggr,m equals to 8 otherwise

-	NPRS,aggr,mslot is the maximum number of DL PRS resources in PFL combination m configured in a slot, and only the PRS resources in resource set(s) linked to other resource set in PFL combination m are counted

-	Naggr,m' is the UE capability on maximum number of DL PRS resources that can be processed in a slot for PFL combination m as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18 for FR1 and maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18 for FR2 specified in TS 37.355 [34].

-	LavailablePRS,aggr,m is the time duration of available PRS resources in PFL combination m to be measured during TPRS,aggr,m, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,aggr,m, only unmuted PRS resources in resource set(s) linked to other resource set in PFL combination m and that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Naggr,m is the UE capability on duration of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18 specified in TS 37.355 [34].

-	Nsample is number of PRS measurement samples,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	Nsample= 4 otherwise.

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

Teffect,aggr,m=Taggr,mTavailable,PRS,aggr,m*Tavailable,PRS,aggr,m

-	Taggr,m is the UE capability on time for processing of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsT specified in TS 37.355 [34].

-	Tavailable,PRS,aggr,m=LCMTPRS,aggr,m,TDRX, the least common multiple between TPRS,aggr,m and the DRX cycle length TDRX, where TPRS,aggr,m is the periodicity of DL PRS resource with muting on PFL combination m.

-	If more than one PRS periodicities are configured in PFL combination m, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets that are linked to other resource set in PFL combination m, is used to derive TPRS,aggr,m, and for each applicable PRS resource set,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, and

-	TperPRS is the periodicity of PRS resource set given by the higher-layer parameter DL-PRS-Periodicity, and

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where 	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Tlast,aggr,m is the measurement duration for the last PRS sample in PFL combination m, including the sampling time and processing time, Tlast,aggr,m=Taggr,m+Tavailable,PRS,aggr,m.

If PRS resources in one or more of the aggregated PFLs in PFL combination m are dropped because of collision with other signals, TRSTD,aggr,m can be longer than specified above.

The time TUERxTx,total starts from the first DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message are delivered from LMF to the UE via LPP [34].

Note:	No separate requirement on aggregated measurement based on TUERxTx,non-aggr or on non-aggregated measurement based on TUERxTx,aggr is applied.

If the RRC state transion occurs from RRC_INACTIVE to RRC_CONNECTED state during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the new serving cell.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the UE Rx-Tx time difference measurement period then the UE Rx-Tx time difference measurement period can be longer.

If Kcarrier_PRS changes for any PFL or any PFL combination during the measurement period, the measurement period could be longer.

If during UE Rx-Tx time difference measurement period PRS resources overlap with other DL signals/channels then the UE Rx-Tx time difference measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

When PRS-RSRPP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRPP measurements are performed over the same measurement period.

The requirements in clause 5.6.4 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

The UE shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1.25.

END OF CHANGE 2