3GPP TSG-RAN WG4 Meeting #110bis	R4-2405547

# Changsha, China, April 15th – 20th 2024

Source:	Ericsson

Title:	Discussion on NR eMIMO for UL demodulation requirements

Agenda item:	6.19.4.2

Document for:	Discussion

# Introduction

In previous RAN4#110 meeting, following agreements were achieved for eMIMO for UL demodulation requirements [1].

Issue 3-1-1: SCS & CBW

Agreement:

- 15KHz SCS, 5MHz; 30KHz SCS, 10MHz;

Issue 3-1-2: MCS

Agreement:

- MCS16

In this contribution, applicability rule is furtherly discussed.

# Discussion

Applicability rule for Rel-18 DMRS ports

One of the motivations of introducing new applicability rule is to avoid duplicated tests if enhanced DM-RS port configuration can pass the requirements. It could be better to capture new applicability rule under the section for normal PUSCH requirements applicability rule. The content could indicate the new requirement chapter and legacy requirement chapter, and state that the legacy requirement with same configurations besides DM-RS ports could be skipped. See following example for 38.141-1.

Proposal 1 	Consider following applicability rule for PUSCH with enhanced DM-RS port demodulation requirements.

##### 8.1.2.1.9	Applicability of PUSCH repetition type A requirements

Unless otherwise stated, PUSCH repetition type A requirements shall apply only for a BS declaring support of low spectral efficiency MCS index table 3 and PUSCH repetition type A (see D.121 and D.122 in table 4.6-1).

##### 8.1.2.1.X	Applicability of PUSCH DM-RS ports

Unless otherwise stated, a BS declare to support PUSCH enhanced-dmrs-Type_r18 (see D.xxx in table 4.6-1) and pass the requirement defined in 8.2.X, can also consider the tests defined in 8.2.1 with same configurations besides DM-RS ports as passed.

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1 	Consider following applicability rule for PUSCH with enhanced DM-RS port demodulation requirements.

##### 8.1.2.1.9	Applicability of PUSCH repetition type A requirements

Unless otherwise stated, PUSCH repetition type A requirements shall apply only for a BS declaring support of low spectral efficiency MCS index table 3 and PUSCH repetition type A (see D.121 and D.122 in table 4.6-1).

##### 8.1.2.1.X	Applicability of PUSCH DM-RS ports

Unless otherwise stated, a BS declare to support PUSCH enhanced-dmrs-Type_r18 (see D.xxx in table 4.6-1) and pass the requirement defined in 8.2.X, can also consider the tests defined in 8.2.1 with same configurations besides DM-RS ports as passed.

# References

[1] 	R4-2402868, “WF on [110][323] NR_MIMO_evo_DL_UL_demod”,  Samsung