3GPP TSG-RAN WG4 Meeting #110bis	R4-2405548

# Changsha, China, April 15th – 20th 2024

Source:	Ericsson

Title:	Simulation results for eMIMO UL demodulation requirements

Agenda item:	6.19.4.2

Document for:	Information

# Introduction

According to the WF in RAN4#110 [1], following simulation assumptions could be considered.

Table 1-1 Simulation parameters

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS durations | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {8}, {8, 9} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

In this contribution, simulation results are provided.

# Simulation results

Table 2-1 Simulation results for eMIMO PUSCH 15kHz SCS, 5 MHz CBW

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Mapping type | Additional DM-RS position | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal | impairment |
| 1 | 2 | Normal | TDLC300-100 Low | 70 % | 16 | A | pos1 | 8.7 | 10.1 |
|  |  |  |  |  |  | B | pos1 | 8.7 | 10.2 |
| 2 | 2 | Normal | TDLC300-100 Low | 70 % | 16 | A | pos1 | 16.4 | 18.6 |
|  |  |  |  |  |  | B | pos1 | 16.4 | 18.5 |

Table 2-2 Simulation results for eMIMO PUSCH 30kHz SCS, 10 MHz CBW

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Mapping type | Additional DM-RS position | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal | impairment |
| 1 | 2 | Normal | TDLC300-100 Low | 70 % | 16 | A | pos1 | 8.9 | 11.2 |
|  |  |  |  |  |  | B | pos1 | 8.9 | 11.3 |
| 2 | 2 | Normal | TDLC300-100 Low | 70 % | 16 | A | pos1 | 16.8 | 19.0 |
|  |  |  |  |  |  | B | pos1 | 16.8 | 18.9 |

# Conclusions

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

# References

[1]	R4-2402868, Way Forward for NR_MIMO_evo_DL_UL_Demod, Nokia, Nokia Shanghai Bell