3GPP TSG-RAN WG4 Meeting#110bis                              R4-2405629

Changsha, China, 15th – 19th April, 2024

Agenda item:	9.3.2

Source: 	ZTE Corporation

Title: 	draft LS reply for NR in 4400 to 4800 MHz

Document for:	Approval

Introduction

In the latest ITU-R meeting, WP5D has sent one LS to RAN/RAN4 to request the Parameters of terrestrial component of IMT in the following frequency ranges for sharing and compatibility studies. During the RAN#103 meeting, the corresponding SID for WP5D reply LS [3] is approved to facilitate the technical discussions in RAN4. In this contribution, we would like to share some views on IMT parameters for 4400 to 4800MHz.

–	4 400-4 800 MHz;

–	7 125-8 400 MHz; and

–	14.8-15.35 GHz.

Discussion

The 1st frequency range 4400-4800MHz which is still falling within the band n79, its IMT parameters and the corresponding RF requirements could refer to the previous ITU-R WP5D Reply LS RP-210037. In addition, in the previous RAN4#110 and RAN#103 meeting, we reached the consensus on the IMT parameters for 4400 to 4800MHz with following notes captured in the SID.

NOTE:	IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz are reused from the already available RAN4 evaluations.

Proposal 1: for 4400-4800MHz frequency range, the parameters of terrestrial network of IMT could refer to the previous ITU-R WP5D Reply LS RP-210037.

Conclusions

In this contribution, we want to share some initial views on IMT parameters for 4400 to 4800MHz and proposals/observations are made as following:

Proposal 1: for IMT parameters of 4400-4800MHz frequency range,refer to the previous ITU-R WP5D Reply LS RP-210037.

References

- R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz.

- RP-210037, Reply LS to WP5D, Approved.

- RP-240787,New SI proposal: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz, Approved.

Annex

ANNEX 1

IMT-2020 technology-related and deployment-related parameters for bands between 470 and 4990 MHz

TABLE 1

IMT-2020 technology related parameters in 470 – 4990 MHz

|  |  | IMT |  |  |
| --- | --- | --- | --- | --- |
| No. | Parameter | Base station (non-AAS) | Base station (AAS) | Mobile station |
| 1 | Duplex Method | FDD / TDD See [1], § 5.2. |  | FDD / TDD See [2], § 5.2. |
| 2 | Channel bandwidth (MHz) | See [1], § 5.3.5. |  | See [2], § 5.3.5. |
| 3 | Signal bandwidth (MHz) | See [1], § 5.3.2. Signal bandwidth = NRB x SCS x 12. |  | See [2], § 5.3.2. Signal bandwidth = NRB x SCS x 12. |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | See [1], § 6.3.3. |  | See [2], § 6.2.1 (UE max output power) and §6.3.1 (UE min output power). |
| 4.2 | Spectral mask (dB) | See [1], § 6.6.4. | See [1], § 9.7.4. | See [2], § 6.5.2.2. |
| 4.3 | ACLR | See [1], § 6.6.3. |  | See [2], § 6.5.2.4. |
| 4.4 | Spurious emissions | See [1], § 6.6.4. |  | See [2], § 6.5.3. |
| 4.5 | Maximum output power | See [1], § 6.2. | See [1], § 9.3. | See [2], § 6.2.1. |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | 5 dB (Wide Area BS) 10 dB (Medium Range BS) 13 dB (Local Area BS) For BS class definitions, see [1], § 4.4 |  | 9 dB |
| 5.2 | Sensitivity (dBm) | See [1], § 7.2.2. | See [1], § 10.3.2. | See [2], § 7.3. |
| 5.3 | Blocking response | See [1], § 7.5 and § 7.4.2. | See [1], § 10.6 and § 10.5.2. | See [2], § 7.6 and § 7.7. |
| 5.4 | ACS | See [1], § 7.4.1. | See [1], § 10.5.1. | See [2], § 7.5. |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |  |

References used in the Table:

[1]	3GPP TS 38.104 v.16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[2]	3GPP TS 38.101-1 v.16.3.0. “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

SINR operating range and mapping function

The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Tℎrougℎput SNIR, bps/Hz =0                                  for SNIR                                         âˆâˆ™SSNIR                     for SNIRMINâ‰¤SNIR<SNIRMAX âˆâˆ™SSNIRMAX               for SNIR â‰¥ SNIRMAX

Where:

S(SNIR)	Shannon bound, S(SNIR) =log2(1+SNIR) [bps/Hz]
		Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX 	Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2 represent a baseline case, which assumes:

- 1:1 antenna configurations

- AWGN channel model

- Link Adaptation (see table 2 for details of the highest and lowest rate codes)

- No HARQ

Table 2: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

_____________

ANNEX 2

Antenna characteristics for IMT-2020 AAS base stations 
for bands between 1710 and 4990 MHz

TABLE 3

Beamforming antenna characteristics for IMT in 1710 – 4990 MHz

|  |  | Rural | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |  | N/A |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 | 7.1 | 6.4 | 6.4 | N/A |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V | 90º for H 54º for V | 90º for H 65º for V | 90º for H 65º for V | N/A |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V | N/A |  |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º | Linear ±45º | N/A |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements | 8 × 8 elements | 8 × 8 elements | 8 × 8 elements | N/A |  |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | N/A |  |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 | ## 2 | N/A |  |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 | 25 | 25 | 16 | N/A |  |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 | 120 | 120 | N/A |  |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-120 | 90-120 | N/A |  |
| 1.12 | Mechanical downtilt (degrees) | 3 | 6 | 10 | N/A | N/A |  |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.8.

Note 3:	The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element).

Note 4:	8 × 8 means there are 8 vertical and 8 horizontal radiating elements. In the sub-array case, one implementation is 2 vertical radiating elements combined in a 2x1 sub-array.