3GPP TSG-RAN WG4 Meeting #110-bis	R4-2405660

# Changsha, China, April 15 – 19, 2024

Source:			Nokia

Title:			Enhanced channel raster UE capability

Agenda item:	4.2.1.1, 4.2.1.3

Document for:	Approval

# Introduction

Enhanced channel raster in UE feature list [1] was agreed in the following table at RAN4#110.

It has been agreed to define the mandatory bands for UE in TS 38.101-1 and TS 38.101-5 for TN and NTN, respectively. So far Band n1, n2, n3, n5, n25, n28, n66, n71 and n85 for TN and Band n254, n255 and n256 and for NTN are requested by operators to mandate from Rel-18 [2-6].

Two major issues are still open regarding Enhanced channel raster UE capability.

- How to define the mandatory bands in TS 38.101-1 and TS 38.101-5.

- Whether RedCap UE is mandated to support Enhanced channel raster and from which 3GPP release it is mandated.

# Discussion

Regarding the first open issue, one proposal [7,8] is to add the new column in the NR ARFCN for enhanced channel raster Table 5.4.2.3-5 of TS 38.101-1 to clarify in which bands the feature will be mandatory.

It will be sufficient to add “Yes” in new column for the requested bands if the feature is mandated for all different types of UEs such as V2X, RedCap, ATG, UAV and UE power classes. If the mandatory requirement is different among different UE types, the column must indicate which UE types are mandated for each band.

Observation 1: Mandatory bands can be specified in a new column of the NR ARFCN table for the enhanced channel raster. UE types (such as RedCap) shall be clarified if the mandatory requirement is not the same among different UE types.

The biggest concern discussed in RAN4#100 [5,8,9] was whether the feature is required for RedCap UEs already from Rel-17 in all supported bands. The maximum bandwidth that RedCap UE can support is 20 MHz. Therefore, if the system bandwidth (signaled in SIB1) is wider than 20 MHz, UE must put its channel filter (at most 20 MHz) somewhere inside the system bandwidth. It has not been clear if UE can put its center at Enhanced channel raster or only at the legacy 100 kHz channel raster. It is also unclear if the network must signal the location of the UE channel filter via UE dedicated signaling of the UE specific channel bandwidth. It was not discussed in Rel-17 RedCap WI whether the UE specific channel bandwidth is signaled from the network to explicitly indicate the position of the filter bandwidth.

Observation 2: It was not discussed in Rel-17 RedCap WI whether the UE specific channel bandwidth is signaled from the network to explicitly indicate the position of the filter bandwidth when the system bandwidth (SIB1) is wider than 20 MHz.

Our understanding is that the even/odd number of PRB issue was not well recognized during Rel-17 work for RedCap. Thus, it was probably left this issue for implementation. A UE may select the position of its 20 MHz channel filter on its own, or the network can signal a UE specific channel bandwidth of 20 MHz or less. For both cases, it must be clarified whether UE can accept or decline such configuration when the center for UE specific channel bandwidth cannot be placed on the legacy 100 kHz channel raster.

One critical example is RedCap UE in 25 MHz system bandwidth for 15 kHz SCS. The maximum transmission bandwidth configuration is 133 PRBs, while RedCap UE may use 20 MHz channel filter which corresponds to 106 PRBs. Due to the even/odd PRB issue, the center of 20 MHz cannot be on the legacy 100 kHz channel raster if the 25 MHz system bandwidth is on the legacy 100 kHz channel raster. It is requested for UE vendors to clarify if their Rel-17 RedCap UE implementations can support such configuration that requires UE channel filters off the 100 kHz raster, for the case that there is a UE specific CHBW signaled off the 100 kHz raster as well as for the case that no UE specific CHBW is signaled?.

Proposal 1: It is requested for UE vendors to clarify if their Rel-17 RedCap UE implementations can support the configuration that requires UE channel filter off the 100 kHz raster, for the case that there is a UE specific CHBW signaled off the 100 kHz raster as well as for the case that no UE specific CHBW is signaled.

If Rel-17 RedCap UEs cannot support the channel filter placement off the legacy 100 kHz channel raster (i.e., 10 kHz Enhanced channel raster), then operators will face issues to use the RedCap UEs in their deployments. For example, for 25 MHz system bandwidth at 100 kHz raster for 15 kHz SCS, 20 MHz RedCap UE cannot be used due to the even/odd PRB issue. A possible workaround is to restrict RedCap UE to 15 MHz  as the maximum transmission bandwidth configuration for 15 MHz is 79 PRBs which is an odd number and can avoid the even/odd PRB issue. Another possible workaround is to use the multiple numerologies and put the second numerology on the 100 kHz raster as we discussed earlier in [10, 11].

It is proposed to clarify in the RAN4 spec if Rel-17 RedCap UEs have any restriction regarding the channel filter placement within the system bandwidth.

Proposal 2: If Rel-17 RedCap UE has any restriction regarding the channel filter placement within the system bandwidth, it shall be clarified in TS 38.101-1.

# Summary

We have discussed Enhanced channel raster UE capability in Rel-18 RAN4 UE feature list.

Observation 1: Mandatory bands can be specified in a new column of the NR ARFCN table for the enhanced channel raster. UE types (such as RedCap) shall be clarified if the mandatory requirement is not the same among different UE types.

Observation 2: It was not discussed in Rel-17 RedCap WI whether the UE specific channel bandwidth is signaled from the network to explicitly indicate the position of the filter bandwidth when the system bandwidth (SIB1) is wider than 20 MHz.

Proposal 1: It is requested for UE vendors to clarify if their Rel-17 RedCap UE implementations can support the configuration that requires UE channel filter off the 100 kHz raster, for the case that there is a UE specific CHBW signaled off the 100 kHz raster as well as for the case that no UE specific CHBW is signaled.

Proposal 2: If Rel-17 RedCap UE has any restriction regarding the channel filter placement within the system bandwidth, it shall be clarified in TS 38.101-1.

# Reference

[1] R4-2403842 Rel-18 RAN4 UE feature list for NR, CMCC

[2] R4-2400647 Frequency bands with mandatory support of the new channel raster, Orange UK

[3] R4-2400723 Mandatory enhanced raster for NR bands n2, n5, and n66, AT&T

[4] R4-2402472 Request for mandatory enhanced raster for n25, n66, n71 and n85, T-Mobile USA

[5] R4-2400979 Discussion on mandatory of enhanced channel raster, CMCC

[6] R4-2400152 Enhanced channel raster for NTN FR1 bands, Apple, Ligado Networks, Inmarsat, Viasat, Globalstar, Thales, Hughes/Echostar

[7] R4-2400218 Clarification for the mandatory support of enhanced channel raster, Apple

[8] R4-2401062 Topic summary for [110][103] R18_UERF_maintenance

[9] R4-2401840 Definition of the enhanced raster and support for RedCap from Rel-17, Ericsson

[10] R4-2312525 Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell

[11] R4-2315511 Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell