3GPP TSG-RAN WG4 Meeting #110bis	R4-2405694

# Changsha, China,  15 April – 19 April, 2024

Source: 	NEC

Title: 	Discussion on requirement set applicability table for NCR

Agenda Item:		6.18.1	RF core requirements maintenance 	[NR_netcon_repeater-Core]

Document for:	discussion

Introduction

This contribution discusses the issues of the requirement set applicability tables in TS 38.106 and provides proposals to solve the issues.

Discussion

In the current repeater specification [1], requirement set applicability tables are defined as below.

4.5	Applicability of requirements

In tables4.5-1, 4.5-1a and 4.5-1b the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.5-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | Repeater type 1-C | Repeater type 2-O |
| Repeater output power | 6.2 |  |
| Frequency stability | 6.3 |  |
| Out of band gain | 6.4 |  |
| Unwanted emissions | 6.5 |  |
| Error Vector Magnitude | 6.6 | NA |
| Input intermodulation | 6.7 |  |
| Output intermodulation | 6.8 |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 |  |
| Transmit ON/OFF power | 6.10 |  |
| Repeater output power |  | 7.2 |
| OTA frequency stability |  | 7.3 |
| OTA out of band gain |  | 7.4 |
| OTA unwanted emissions |  | 7.5 |
| OTA Error Vector Magnitude | NA | 7.6 |
| OTA input intermodulation |  | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  | 7.8 |
| OTA transmit ON/OFF power |  | 7.9 |

Table 4.5-1a: Requirement set applicability for NCR-Fwd

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | NCR-MT type 1-C | NCR-MT type 1-H | NCR-MT type 2-O |
| (omitted) |  |  |  |

Table 4.5-1b: Requirement set applicability for NCR-MT

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | NCR-MT type 1-C | NCR-MT type 1-H | NCR-MT type 2-O |
| Conducted transmitter power | 6.2.3.2 | 6.2.3.2 |  |
| Conducted output power dynamics | 6.11 | 6.11 |  |
| Conducted transmit signal quality | 6.12 | 6.12 |  |
| Conducted output RF spectrum emissions | 6.5 | 6.5 |  |
| Conducted transmit intermodulation | 6.13 | 6.13 |  |
| Diversity characteristics | 6.15 | 6.15 | NA |
| Conducted reference sensitivity | 6.16 | 6.16 |  |
| Conducted maximum input level | 6.17 | 6.17 |  |
| Conducted adjacent channel selectivity | 6.18 | 6.18 |  |
| Conducted blocking characteristics | 6.19 | 6.19 |  |
| Conducted spurious response | 6.20 | 6.20 |  |
| Conducted intermodulation characteristics | 6.21 | 6.21 |  |
| Conducted spurious emissions | 6.5.5 6.22 | 6.5.5 6.22 |  |
| Radiated transmitter power |  |  | 7.2 |
| Radiated output power dynamics |  |  | 7.10 |
| Radiated transmit signal quality |  |  | 7.11 |
| Radiated output RF spectrum emissions |  |  | 7.5 |
| Diversity characteristics | NA | NA | 7.12 |
| Radiated reference sensitivity |  |  | 7.13 |
| Radiated maximum input level |  |  | 7.14 |
| Radiated adjacent channel selectivity |  |  | 7.15 |
| Radiated blocking characteristics |  |  | 7.16 |
| Radiated spurious emissions |  |  | 7.17 |

For RF repeater and NCR-Fwd, it is straightforward that conducted requirements are specified in clause 6.2 to 6.10, radiated requirements are specified in clause 7.2 to 7.9 where requirement names start with “OTA”.

For NCR-MT, the relation between the requirements for NCR-MT and corresponding clauses are not very simple. Most conducted requirements are specified in 6.11 to 6.22 and most radiated requirements are specified in 7.10 to 7.17 using consecutive clause numbers after clauses for RF repeater and NCR-Fwd. However, there are some exceptions, where

- Conducted transmitter power is in 6.2.3.2

- Conducted output RF spectrum emissions is in 6.5

- Conducted spurious emissions is in 6.5.5 and 6.22

- Radiated transmitter power is in 7.2

- Radiated output RF spectrum emissions is in 7.5

- Prefix “Conducted” and “Radiated” are used for NCR-MT requirement set

- Performance requirements are missing

These make the spec structure complex.

Looking into above clauses, we see some issues.

- Applicability table for NCR-MT says “conducted transmitter power” is in 6.2.3.2. Requirement name in 6.2 is “repeater output power” which is not aligned it. General clause in 6.2.1 is only for RF repeater and NCR-Fwd, not for NCR-MT. General clause for NCR-MT in 6.2.3.2.1 is curiously located under the clause for minimum requirement for NCR-MT. It is proposed 6.2 is only for RF repeater and NCR-Fwd. Repeater output power requirements for NCR-MT shall be placed outside of 6.2.

- Applicability table for NCR-MT says “Conducted output RF spectrum emissions” is in 6.5. Clause title of 6.5 is “unwanted emissions”. Requirement name shall be aligned. Unwanted emissions in 6.5 include ACLR, OBUE, transmitter spurious emissions, and receiver spurious emissions. ACLR, OBUE, and transmitter spurious emissions minimum requirements for NCR-MT are included in 6.5, but receiver spurious emissions minimum requirements for NCR-MT are not included. Furthermore, description in the general clause in 6.5.1, general clauses and basic limits under each requirement may not apply to NCR-MT. It is proposed 6.5 is only for RF repeater and NCR-Fwd. Unwanted emissions including receiver spurious emissions for NCR-MT shall specified outside of 6.5.

- Applicability table for NCR-MT says “conducted spurious emissions” is in 6.5.5 and 6.22. Both 6.5.5 and 6.22 are actually for receiver spurious emissions. NCR-MT requirements are not mentioned in 6.5.5. For receiver spurious emissions are specified under unwanted emissions for RF repeater and NCR-Fwd, it would be better to have the same way for NCR-MT, that is, receiver spurious emissions for NCR-MT are under unwanted emissions for NCR-MT.

- Applicability table for NCR-MT says “radiated transmitter power” is in 7.2. General clause in 7.2.1 applies to only RF repeater and NCR-Fwd. It does not apply to NCR-MT. It is proposed 7.2 is only for RF repeater and NCR-Fwd. OTA repeater output power requirements for NCR-MT shall be placed outside of 7.2.

- Applicability table for NCR-MT says “radiated output RF spectrum emissions” is in 7.5. We see similar issues as shown in 2. Title for 7.5 is OTA unwanted emissions. Requirement name shall be aligned with the title. It is proposed 7.5 is only for RF repeater and NCR-Fwd. Unwanted emissions for NCR-MT shall be specified outside of 7.5.

- Applicability table for NCR-MT uses prefix “conducted” or “radiated” for each requirement. It seems they imply conducted or radiated requirements for NCR-MT. We think it may lead to confusion because RF repeater and NCR-Fwd couducted or radiated requirements are named differently without any explanation. We propose to remove “conducted” or replace “radiated” with “OTA” and add “for NCR-MT”. It would be more straightforward. For example, we propose “conducted output power dynamics” is replaced with “output power dynamics for NCR-MT”. “Radiated output power dynamics” is replaced with “OTA output power dynamics for NCR-MT”

- Applicability table for NCR-MT does not have performance requirements and radiated performance requirements. They should be added in the table

Based on the above discussion, following proposal is formulated.

Proposal 1:

To modify the requirement set applicability table for NCR-MT in TS 38.106 as below.

Table 4.5-1b: Requirement set applicability for NCR-MT

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | NCR-MT type 1-C | NCR-MT type 1-H | NCR-MT type 2-O |
| Repeater output power for NCR-MT | 6.23 | 6.23 |  |
| Output power dynamics for NCR-MT | 6.11 | 6.11 |  |
| Transmit signal quality for NCR-MT | 6.12 | 6.12 |  |
| Unwanted emissions for NCR-MT | 6.24 | 6.24 |  |
| Transmit intermodulation for NCR-MT | 6.13 | 6.13 |  |
| Diversity characteristics for NCR-MT | 6.15 | 6.15 | NA |
| Reference sensitivity for NCR-MT | 6.16 | 6.16 |  |
| Maximum input level for NCR-MT | 6.17 | 6.17 |  |
| Adjacent channel selectivity for NCR-MT | 6.18 | 6.18 |  |
| Blocking characteristics for NCR-MT | 6.19 | 6.19 |  |
| Spurious response for NCR-MT | 6.20 | 6.20 |  |
| Receiver intermodulation characteristics for NCR-MT | 6.21 | 6.21 |  |
| Performance requirements for NCR-MT | 8 | 8 |  |
| OTA repeater output power for NCR-MT |  | 7.18 | 7.18 |
| OTA output power dynamics for NCR-MT |  |  | 7.10 |
| OTA transmit signal quality for NCR-MT |  |  | 7.11 |
| OTA unwanted emissions for NCR-MT |  |  | 7.19 |
| OTA diversity characteristics for NCR-MT | NA | NA | 7.12 |
| OTA reference sensitivity for NCR-MT |  |  | 7.13 |
| OTA maximum input level for NCR-MT |  |  | 7.14 |
| OTA adjacent channel selectivity for NCR-MT |  |  | 7.15 |
| OTA blocking characteristics for NCR-MT |  |  | 7.16 |
| OTA receiver spurious emissions for NCR-MT |  |  | 7.17 |
| Radiated performance requirements for NCR-MT |  |  | 9 |

In addition, there are some issues in applicability table for RF repeater and NCR-Fwd.

- RAN4 has agreed that EVM for RF repeater or NCR-Fwd is different from that for BS. It is called repeater EVM. Applicability tables for RF repeater and NCR-Fwd shall be modified accordingly.

- We see repeater output power in two places, one for 6.2 and another one for 7.2. Requirements in 7.2 shall be OTA repeater output power.

Based on the above discussion, following proposal is formulated.

Proposal 2:

To modify the requirement set applicability tables for RF repeater and NCR-Fwd in TS 38.106 as below.

Table 4.5-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | Repeater type 1-C | Repeater type 2-O |
| Repeater output power | 6.2 |  |
| Frequency stability | 6.3 |  |
| Out of band gain | 6.4 |  |
| Unwanted emissions | 6.5 |  |
| Repeater Error Vector Magnitude | 6.6 | NA |
| Input intermodulation | 6.7 |  |
| Output intermodulation | 6.8 |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 |  |
| Transmit ON/OFF power | 6.10 |  |
| OTA Repeater output power |  | 7.2 |
| OTA frequency stability |  | 7.3 |
| OTA out of band gain |  | 7.4 |
| OTA unwanted emissions |  | 7.5 |
| OTA Repeater Error Vector Magnitude | NA | 7.6 |
| OTA input intermodulation |  | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  | 7.8 |
| OTA transmit ON/OFF power |  | 7.9 |

Table 4.5-1a: Requirement set applicability for NCR-Fwd

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | NCR-Fwd type 1-C | NCR-Fwd type 1-H | NCR-Fwd type 2-O |
| Repeater output power | 6.2 | 6.2 |  |
| Frequency stability | 6.3 | 6.3 |  |
| Out of band gain | 6.4 | 6.4 |  |
| Unwanted emissions | 6.5 | 6.5 |  |
| Repeater Error Vector Magnitude | 6.6 | 6.6 | NA |
| Input intermodulation | 6.7 | 6.7 |  |
| Output intermodulation | 6.8 | 6.8 |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 | 6.9 |  |
| Transmit ON/OFF power | 6.10 | 6.10 |  |
| OTA Repeater output power |  | 7.2 | 7.2 |
| OTA frequency stability |  |  | 7.3 |
| OTA out of band gain |  |  | 7.4 |
| OTA unwanted emissions |  |  | 7.5 |
| OTA Repeater Error Vector Magnitude | NA | NA | 7.6 |
| OTA input intermodulation |  |  | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  |  | 7.8 |
| OTA transmit ON/OFF power |  |  | 7.9 |

It is also proposed to modify the spec structure of TS 38.106 to be aligned with the description of the requirement set applicability tables. Spec structure of TS 38.115-1 and 38.115-2 shall be also modified to be aligned with the applicability tables.

Proposal 3:

Spec structure of TS 38.106, 38.115-1, and 38.115-2 shall be modified to be aligned with the proposed applicability tables.

Conclusions

We discussed the issue of the requirement set applicability tables in TS 38.106 and provided the following proposals.

Proposal 1:

To modify the requirement set applicability table for NCR-MT in TS 38.106 as below.

Table 4.5-1b: Requirement set applicability for NCR-MT

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | NCR-MT type 1-C | NCR-MT type 1-H | NCR-MT type 2-O |
| Repeater output power for NCR-MT | 6.23 | 6.23 |  |
| Output power dynamics for NCR-MT | 6.11 | 6.11 |  |
| Transmit signal quality for NCR-MT | 6.12 | 6.12 |  |
| Unwanted emissions for NCR-MT | 6.24 | 6.24 |  |
| Transmit intermodulation for NCR-MT | 6.13 | 6.13 |  |
| Diversity characteristics for NCR-MT | 6.15 | 6.15 | NA |
| Reference sensitivity for NCR-MT | 6.16 | 6.16 |  |
| Maximum input level for NCR-MT | 6.17 | 6.17 |  |
| Adjacent channel selectivity for NCR-MT | 6.18 | 6.18 |  |
| Blocking characteristics for NCR-MT | 6.19 | 6.19 |  |
| Spurious response for NCR-MT | 6.20 | 6.20 |  |
| Receiver intermodulation characteristics for NCR-MT | 6.21 | 6.21 |  |
| Performance requirements for NCR-MT | 8 | 8 |  |
| OTA repeater output power for NCR-MT |  | 7.18 | 7.18 |
| OTA output power dynamics for NCR-MT |  |  | 7.10 |
| OTA transmit signal quality for NCR-MT |  |  | 7.11 |
| OTA unwanted emissions for NCR-MT |  |  | 7.19 |
| OTA diversity characteristics for NCR-MT | NA | NA | 7.12 |
| OTA reference sensitivity for NCR-MT |  |  | 7.13 |
| OTA maximum input level for NCR-MT |  |  | 7.14 |
| OTA adjacent channel selectivity for NCR-MT |  |  | 7.15 |
| OTA blocking characteristics for NCR-MT |  |  | 7.16 |
| OTA receiver spurious emissions for NCR-MT |  |  | 7.17 |
| Radiated performance requirements for NCR-MT |  |  | 9 |

Proposal 2:

To modify the requirement set applicability tables for RF repeater and NCR-Fwd in TS 38.106 as below.

Table 4.5-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | Repeater type 1-C | Repeater type 2-O |
| Repeater output power | 6.2 |  |
| Frequency stability | 6.3 |  |
| Out of band gain | 6.4 |  |
| Unwanted emissions | 6.5 |  |
| Repeater Error Vector Magnitude | 6.6 | NA |
| Input intermodulation | 6.7 |  |
| Output intermodulation | 6.8 |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 |  |
| Transmit ON/OFF power | 6.10 |  |
| OTA Repeater output power |  | 7.2 |
| OTA frequency stability |  | 7.3 |
| OTA out of band gain |  | 7.4 |
| OTA unwanted emissions |  | 7.5 |
| OTA Repeater Error Vector Magnitude | NA | 7.6 |
| OTA input intermodulation |  | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  | 7.8 |
| OTA transmit ON/OFF power |  | 7.9 |

Table 4.5-1a: Requirement set applicability for NCR-Fwd

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | NCR-Fwd type 1-C | NCR-Fwd type 1-H | NCR-Fwd type 2-O |
| Repeater output power | 6.2 | 6.2 |  |
| Frequency stability | 6.3 | 6.3 |  |
| Out of band gain | 6.4 | 6.4 |  |
| Unwanted emissions | 6.5 | 6.5 |  |
| Repeater Error Vector Magnitude | 6.6 | 6.6 | NA |
| Input intermodulation | 6.7 | 6.7 |  |
| Output intermodulation | 6.8 | 6.8 |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 | 6.9 |  |
| Transmit ON/OFF power | 6.10 | 6.10 |  |
| OTA Repeater output power |  |  | 7.2 |
| OTA frequency stability |  |  | 7.3 |
| OTA out of band gain |  |  | 7.4 |
| OTA unwanted emissions |  |  | 7.5 |
| OTA Repeater Error Vector Magnitude | NA | NA | 7.6 |
| OTA input intermodulation |  |  | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  |  | 7.8 |
| OTA transmit ON/OFF power |  |  | 7.9 |

Proposal 3:

Spec structure of TS 38.106, 38.115-1, and 38.115-2 shall be modified to be aligned with the proposed applicability tables.

References

[1] TS 38.106 v18.4.0