3GPP TSG-WG4 Meeting #110bis	R4-2405705

# Changsha, China, 15 April - 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.108: Correction of applicability table for SAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-4-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | OTA sensitivity requirement is not specified for SAN type 2-O. However, the clause for it is identified in the requirement set applicability table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the clause of OTA sensitivity requirement for SAN type 2-O to “NA”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement set applicability table is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

4.6	Applicability of minimum requirements

In table 4.6-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O | SAN type 2-O |
| SAN output power | 6.2 |  |  |
| Output power dynamics | 6.3 |  |  |
| Transmit ON/OFF power | NA |  |  |
| Frequency error | 6.5.1 |  |  |
| Modulation quality | 6.5.2 |  |  |
| Time alignment error | NA |  |  |
| Occupied bandwidth | 6.6.2 |  |  |
| ACLR | 6.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  |
| Transmitter intermodulation | NA | NA | NA |
| Reference sensitivity level | 7.2 |  |  |
| Dynamic range | 7.3 |  |  |
| ACS | 7.4.1 |  |  |
| In-band blocking | NA |  |  |
| Out-of-band blocking | 7.5 |  |  |
| Receiver spurious emissions | NA |  |  |
| Receiver intermodulation | NA |  |  |
| In-channel selectivity | 7.8 |  |  |
| Performance requirements | 8 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 |
| OTA SAN output power |  | 9.3 | 9.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |
| OTA transmit ON/OFF power |  | NA | NA |
| OTA frequency error |  | 9.6.1 | 9.6.1 |
| OTA modulation quality |  | 9.6.2 | 9.6.2 |
| OTA time alignment error |  | NA | NA |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |
| OTA transmitter intermodulation |  | NA | NA |
| OTA sensitivity | 10.2 | 10.2 | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |
| OTA dynamic range |  | 10.4 | NA |
| OTA ACS |  | 10.5.1 | 10.5.1 |
| OTA in-band blocking |  | NA | NA |
| OTA out-of-band blocking | NA | 10.6 | 10.6 |
| OTA receiver spurious emission |  | NA | NA |
| OTA receiver intermodulation |  | NA | NA |
| OTA in-channel selectivity |  | 10.9 | 10.9 |
| Radiated performance requirements |  | 11 | 11 |

NOTE:	Co-location requirements are not applicable to SAN.

--------------End of change-------------