3GPP TSG-RAN WG4 Meeting #110bis	R4-2405716

Changsha, China, 15th – 19th April, 2024

Source: 	Qualcomm Incorporated

Title: 	UE Demod Discussion and Simulation Results for Network Energy Savings

Agenda item:	6.24.4

Document for:	Approval

- Introduction

In this paper, we share our simulation results for the case of SSB-less SCell with time misalignment  with 3MHz BW, based on the simulation assumptions in the WF from the last RAN4 meeting.

# Discussion on PDSCH Requirements

In the previous RAN4 meeting (R4#110, February 2024) it was agreed for this WI to:

evaluate the performance of SSB-less SCells CA performance, if the observed performance constitutes a practical operating point of the feature, or if SSB-less SCell compliant UE shall be tested with normal CA requirements.

To that end, RAN4 agreed to a set of simulation assumptions, and we include in this paper the expected performance loss caused by the TO between PCell and SCell according to the simulation results.

Table 2-1

| BW/SCS | Propagation Condition | Antenna Configuration | Test Number | TO (CP) | TO (us) | Performance loss |
| --- | --- | --- | --- | --- | --- | --- |
| 40MHz/30kHz | TDLA30-10 | 2x2 | 1 | 0 | 0 | n.a. (Reference) |
|  |  |  | 2 | 0.4 | 0.92 | 0.2 |
|  |  |  | 3 | 0.9 | 2.07 | 0.1 |

In the table above, only simulation results with positive TO were collected. The UE performance is not expected to change if the time offset changes sign.

According to the results, there is only negligible performance impact to the absence of SSB in the SCell

Observation 1: RAN4 to consider the simulation results provided in Table 2-1 in this document.

As we have observed and argumented during the previous RAN4 meeting, it is not necessary to introduce UE Demod requirements for PDSCH processing with SSB-less SCell

Proposal 1: RAN4 not to introduce UE Demod PDSCH performance requirements with SSBless SCell.

- Conclusions

Observation 1: RAN4 to consider the simulation results provided in the document.

Proposal 1: RAN4 not to introduce UE Demod PDSCH performance requirements with SSBless SCell.