3GPP TSG-RAN WG4 Meeting #110bis	R4-2405717

Changsha, China, 15th – 19th April, 2024

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for PBCH and PDCCH Requirements for Less than 5MHz BW

Agenda item:	6.8.7.1

Document for:	Information

- Introduction

In this paper, we share our simulation results for the PBCH and PDCCH test cases, based on the simulation assumptions agreed in the WF from the last RAN4 meeting (R4-2402863).

# PBCH Simulation Results

Simulation results for PBCH were updated with respect to our contribution in R4#110. Please find Qualcomm’s reviewed results in Table 2-1.

Table 2-1

| Test Number | Num RX | Num RB | SSB/PBCH Index | Propagation Condition | Alignment SNR [dB] | Impairment SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 12 | Unknown | TDLC300-100 | -4.4 | -2.4 |
| 2 | 4 | 12 | Unknown | TDLC300-100 | -2.3 | -0.3 |

# PDCCH Simulation Results

PDCCH alignment and impairment results are included in Table 3-1.

Table 3-1

| TX | BW MHz | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Pm-dsg (%) | Alignment SNR [dB] | Impairment SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 15 | 3 | 4 | TDLA30-10 | 1x2 Low | 1 | 5.2 | 7.2 |
| 2 | 3 | 15 | 3 | 4 | TDLC300-100 | 2x2 Low | 1 | 1.9 | 3.9 |
| 1 | 3 | 15 | 3 | 4 | TDLA30-10 | 1x4 Low | 1 | -0.8 | 1.2 |
| 2 | 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | 1 | -2.9 | -0.9 |

- Conclusions

Proposal 1: RAN4 to consider the PBCH impairment results provided in the document for the definition of the requirements.

Proposal 2: RAN4 to consider the PDCCH impairment results provided in this document for the definition of the requirements.