3GPP TSG-RAN4 Meeting #110-bis	R4-2405726

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

Source: 	Qualcomm Incorporated

Title: 	Initial view for R19 RRM enhancements WID

Agenda item:	9.6.2

Document for:	Discussion

# Introduction

In this paper, we are presenting the initial view for R19 RRM enhancement WI.

# Discussion

In WID two different scenario is specified, first topic is for UE supporting multiple-RX simultaneous reception on single carrier and the other topic is for UE not in multiple-RX simultaneous reception mode.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

First, we want to clarify the definition and implication of “For UE supporting multiple-Rx simultaneous reception”. From our understanding, this means UE supporting ‘simultaneous reception of #N SSBs from different directions inside a SMTC window. But it does not mean UE can process #N SSBs in parallel processing.

Proposal: In first topic, “for UE supporting multiple-Rx simultaneous reception means UE supporting “simultaneous reception of #N SSBs from different directions inside a SMTC window. But it does not mean UE can process #N SSBs in parallel.”

Second, “For UE not in multiple-RX simultaneous reception mode” is not clear statement. It does not mean the solution of CSSF optimization is adopted based on multi-RX status in UE (e.g active multi-Rx or not). The condition can be about whether UE support or not support multiple-RX simultaneous reception instead of the status of UE. Therefore, second topic included for UE who does not support multiple-rx simultaneous reception. Moreover, CSSF is common parameter for measurement requirements which may not related to multi-RX simultaneous reception. It is more practical if RAN4 introduce the general solution by optimizing CSSF outside gap in CA/DC mode regardless of support of multiple-RX simultaneous reception.

Observation : In second topic, “UE not in multiple-RX simultaneous reception mode” is not clear statement. It does not mean the solution of CSSF optimization is adopted based on multi-RX status in UE (e.g active multi-Rx or not). For UE who does not support multiple-RX simultaneous reception is also included in second topic.

Observation :  CSSF is common parameter for measurement requirements which may not related to multi-RX simultaneous reception. It is more practical if RAN4 introduce the general solution by optimizing CSSF outside gap in CA/DC mode.

Proposal: “for UE not in multiple-RX simultaneous reception mode” includes “for UE not supporting multiple-RX simultaneous reception” for second topic.

- RAN4 can further study if the conditions for specifying UE type in second topic can be removed.

Proposal: RAN4 study the solution of CSSF optimization for UE not supporting multiple Rx simultaneous reception as baseline for second topic.

# Conclusions

Proposal 1: In first topic, “for UE supporting multiple-Rx simultaneous reception means UE supporting “simultaneous reception of #N SSBs from different directions inside a SMTC window. But it does not mean UE can process #N SSBs in parallel.”

Observation 1: In second topic, “UE not in multiple-RX simultaneous reception mode” is not clear statement. It does not mean the solution of CSSF optimization is adopted based on multi-RX status in UE (e.g active multi-Rx or not). For UE who does not support multiple-RX simultaneous reception is also included in second topic.

Observation 2:  CSSF is common parameter for measurement requirements which may not related to multi-RX simultaneous reception. It is more practical if RAN4 introduce the general solution by optimizing CSSF outside gap in CA/DC mode.

Proposal 2: “for UE not in multiple-RX simultaneous reception mode” includes “for UE not supporting multiple-RX simultaneous reception” for second topic.

- RAN4 can further study if the conditions for specifying UE type in second topic can be removed.

Proposal 3: RAN4 study the solution of CSSF optimization for UE not supporting multiple Rx simultaneous reception as baseline for second topic.

# References

[1] RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo, RAN #103