3GPP TSG RAN WG4 Meeting #110bis															R4-2405773

Changsha, China, 15 Apr. 2024 – 19 Apr. 2024

Agenda Item:	6.4.3

Source: 	Ericsson

Title:	Discussion on performance part of FR2 SCell activation enhancement

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the performance part of the FR2 SCell activation enhancements in R18.

# Discussion

RAN4 agreed on the following test cases in the last meeting.

| Feature | Index | Test case | CA/DC mode | Other configurations | Test applicability | Volunteer Company to lead CR |
| --- | --- | --- | --- | --- | --- | --- |
| FG31-1: L3 report based enhancement | 1 | FR2 unknown SCell activation with L3 report | - FR1 PCell+FR2 target SCell - FR2 PCell+FR2 inter-band target SCell - LTE PCell + FR1 PSCell + FR2 target SCell |  | Define test applicability and UE is required to perform one test for each FR. UE only needs to pass test in one CA/DC mode. | QC |
|  | 2 | FR2 PUCCH SCell activation delay with L3 report |  |  |  | Huawei |
|  | 5 | FR1 unknown SCell activation with L3 report | - FR1 PCell+FR1 inter-band target SCell - LTE PCell + FR1 PSCell + FR1 inter-band target SCell |  | Define test applicability and UE is required to perform one test for each FR. UE only needs to pass test in one CA/DC mode. | vivo |
|  | 7 | Multiple SCell activation delay with FR1 unknown SCell with L3 report |  |  |  | ZTE |
| FG31-2: Beam sweeping factors reduction | 9 | FR2 unknown SCell activation with FG31-2 and FG31-3 | - FR1 PCell+FR2 target SCell - FR2 PCell+FR2 inter-band target SCell - LTE PCell + FR1 PSCell + FR2 target SCell | DRX.8: DRX cycle = 320 ms and TAT = Infinity | Verify UE supporting FG 31-2 and/or FG 31-3 for FR2, depending the capabilities supported by the UE. FFS:UE only needs to pass test in one CA/DC mode. | Nokia |
| FG31-3: (1)Use SSB periodicity instead of SMTC periodicity” (2)“Performing L1-RSRP measurement in non-DRX mode even DRX is configured” | 17 | FR1 unknown SCell activation with FG31-3 | - FR1 PCell+FR1 inter-band target SCell - LTE PCell + FR1 PSCell + FR1 inter-band target SCell | DRX.8: DRX cycle = 320 ms and TAT = Infinity | Verify UE supporting FG 31-3 for FR1. UE only needs to pass test in one CA/DC mode. | MTK |

Further RAN4 also agreed on how to configure to have valid L3 report for FG31-1 TC. Agreement is captured below.

- Configure event-triggered reporting associated with target SCell MO such as A2-event along with new reporting type for L3 report-based unknown SCell activation, and maintain the SNR above threshold so that A2 event is not met.

- TE use DCI to schedule DG- PUSCH for L3 measurement report, and the M is as specified in clause 8.3.17/18 of TS38.133.

  - Further discuss the exact M values for the test.

- Configure the following conditions in TC to make UE report valid L3 result (apply to FR1+FR1 2CC CA, FR1+FR2 2CC CA, FR2+FR2 inter-band CA as examples):

  - Non-DRX is configured; and;

  - CSSF = 1, i.e., only single MO is configured on target SCC; and;

  - The SSB measured remains detectable according to the cell identification conditions specified in clause 9.2; and

  - the SCell has been configured for a time period longer than Tidentify_intra_with_index in clause 9.2.5.1, provided the SCell is newly configured in deactivated state and the side condition Ês/Iot ≥ -2dB is fulfilled.

One open issue from the above agreement is exact values for the test. M value is specified in 38.133, clause 8.3.17/18

UE is not required to report the L3 results after 3ms + THARQ+ M ms from receiving the SCell activation command where

-	M= (X1+X2)*TSSB + [TL1-RSRP,report] for UE supporting [beamsweepingfactor reducton capability for Scell activaiton] and [using SSB peridocity instead of SMTC peridodicity for scell activation] capability,

-	M = X1*TSMTC +X2*TSSB + [TL1-RSRP,report] for UE supporting [beamsweepingfactor reducton capability for Scell activaiton] without supporting [using SSB peridocity instead of SMTC peridodicity for scell activation],

-	M = 16*TSSB + [TL1-RSRP,report] for UE supporting [using SSB peridocity instead of SMTC peridodicity for scell activation] without supporting [beamsweepingfactor reducton capability for Scell activaiton],

-	Otherwise, M = 8*TSMTC +8*TSSB + [TL1-RSRP,report]

Since the test is to test when UE is expected to report and not when the UE is not expected to report, we think it is not required to specify M value in the test. Since the NW knows UE is not expected to report the measurement report if it is more than M ms, in real NW implementation, NW would trigger the report accordingly.

- RAN4 not to specify M value in the test for valid L3 report for FG31-1 TC.

Other open issue for the same test case is the time period for the test. One of the options discussed was mentioned below.

General test procedure for L3-reporting based SCell activation and SCell deactivation.

  - Cell power keeps remaining same to avoid event-triggered reporting

  - T1 shall be sufficiently large to perform deactivated SCell measurement for target SCell

  - T1 : start from RRC SCell addition to receive MAC-CE SCell activation.

  - T2 : start from receiving MAC-CE for SCell activation to L3 reporting

  - T3 : start from L3 reporting to CSI report

  - T4 : start from receiving SCell deactivation command

We have one suggestion to the above option. MO is configured on the SCell frequency before the start of the T1. T1 should be sufficiently large and power level should be such a way that A2 event is not triggered during T1. Before the end of T1, SCell is added, and it is in deactivated state. After RRC processing time, NW triggers the SCell activation command, and it is the starting point for T2. After the SCell activation command, NW keeps trigger DCI based dynamic grant and UE should report the L3 report within 7ms from the SCell activation command. L3 report is the starting point for T3, and UE shall send CSI report.

- RAN4 to agree on the following procedure for L3-reporting based SCell activation and SCell deactivation

  - Cell power keeps remaining same to avoid event-triggered reporting

  - Before start of T1, MO is configured. Before end of T1, and after sufficiently large time to perform measurement on the neighbour cell, SCell is added. T1 ends with SCell being added.

  - T2 : start from receiving MAC-CE for SCell activation to L3 reporting

  - T3 : start from L3 reporting to CSI report

  - T4 : start from receiving SCell deactivation command.

# Summary and Conclusion

In this contribution we have provided our views on the FR2 SCell activation delay reduction performance part and made the following proposals:

- RAN4 not to specify M value in the test for valid L3 report for FG31-1 TC.

- RAN4 to agree on the following procedure for L3-reporting based SCell activation and SCell deactivation

  - Cell power keeps remaining same to avoid event-triggered reporting

  - Before start of T1, MO is configured. Before end of T1, and after sufficiently large time to perform measurement on the neighbour cell, SCell is added. T1 ends with SCell being added.

  - T2: start from receiving MAC-CE for SCell activation to L3 reporting

  - T3: start from L3 reporting to CSI report

  - T4: start from receiving SCell deactivation command.

# References

- RP-220977 “WID on Even Further RRM enhancement for NR and MR-DC”, Apple Oppo

- R4-2403466