3GPP TSG RAN WG4 Meeting #110bis	R4-2405781

Changsha, China, 15 Apr. 2024 – 19 Apr. 2024

Agenda Item:	6.19.3.1

Source: 	Ericsson

Title:	Performance requirements for TDCP

Document for:	Discussion

# Introduction

In this contribution we provide our views on the RRM performance requirements for Rel-18 TDCP measurements.

# Discussion

In last meeting it was agreed [1] to define test cases for TDCP under certain conditions, and the conditions are FFS during last meeting.  Furthermore, RAN4 also agreed to define TDCP mapping table in RAN4 specification. We suggest capturing Table 1 in 3GPP 38.133.

Table 1: TDCP quantization values

| Quantization | TDCP range |
| --- | --- |
| 0 | 0.9945< TDCP <=1 |
| 1 | 0.9922< TDCP <=0.9945 |
| 2 | 0.9890< TDCP <=0.9922 |
| 3 | 0.9844< TDCP <=0.9890 |
| 4 | 0.9779 < TDCP <= .9844 |
| 5 | 0.9688 < TDCP <= 0.9779 |
| 6 | 0.9558 < TDCP <= 0.9688 |
| 7 | 0.9375 < TDCP <= 0.9558 |
| 8 | 0.9116 < TDCP <= 0.9375 |
| 9 | 0.875 < TDCP <= 0.9116 |
| 10 | 0.8234 < TDCP <= 0.875 |
| 11 | 0.75 < TDCP <= 0.8234 |
| 12 | 0.6464< TDCP <=0.75 |
| 13 | 0.5< TDCP <=0.6464 |
| 14 | 0.2929< TDCP <=0.5 |
| 15 | 0< TDCP <=0.2929 |

TDCP RRM test configuration:

Channel model for the tests: Since the simulations are performed using TDL-A channel, to avoid additional simulation work between April and May meeting, we are fine to reuse the same.

TRS configuration: Since a TRS separation of one slot is mandatory and other TRS spacings are optional, we suggest 1 slot so that all the UEs supporting TDCP can be tested.

Frequency: 3.5 GHz

SNR and doppler:

We think running tests for one lower Doppler and one higher Doppler makes sense. To make sure UE is tested with sufficient SNR coverage, we should run one test for higher SNR and one test for lower/medium SNR. Since the simulation values for lower Doppler at lower SNR have a large variance, defining a test for lower Doppler at lower SNR may not make any sense. Hence, we prefer defining a test for a lower doppler, i.e., 10 Hz at higher SNR. To make sure SNR coverage in lower region, we prefer defining a test for 300 Hz Doppler at SNR of 10 dB. Based on the simulation results we list the suitable values for the tests in the tables below.

Table 2: TDCP ideal values based on the 50% simulation

| Doppler | SNR [dB] | TDCP range based on 50% (range 1) |
| --- | --- | --- |
| 10 Hz | 20 | 0 to 4 |
| 300 Hz | 10 | 9 to 12 |

Table 3: TDCP ideal values based on the 10% simulation results

| Doppler | SNR [dB] | TDCP range based on 10% (range 2) |
| --- | --- | --- |
| 10 Hz | 20 | 0 to 7 |
| 300 Hz | 10 | 12 to 14 |

Test metric:

Generally, in RRM tests, UEs need to pass for the 90% of the times over repeated tests. Since the ideal values presented in table 2 are taken from the 50% of the CDF plot, we think 50% of the times UE should report within the range determined from the simulation results. For the remaining 40% of the times, UE should report the values selected from 10% of CDF plot results submitted by the companies. If we look at table 2 and 3, TDCP range in the lower end of the TDCP value needs to extend by 2 or 3 values for the remaining 40% of the tests.

Based on table 2 and 3 we suggest following table for the ideal values for the test.

Table 4: Range 1 and Range 2 for the TDCP reporting

| Doppler | SNR | TDCP range to be reported 50% of the tests (range 1) | TDCP range to be reported for remaining 40% of the tests (range 2) |
| --- | --- | --- | --- |
| 10 Hz | 20 | 0 to 4 | 0 to 7 |
| 300 Hz | 10 | 9 to 12 | 9 to 14 |

In one example, if we run repeated tests over 100 times, for the 50 tests UE should report in the range 1 and 40 tests UE can report in the range 1 or range 2, remaining 10 tests, UE can report any value to pass the test. Based on the above analysis we suggest the following as the test metric.

- For the test metric, over the repeated tests, UE should at least report values within range 1 for the 50% of the times and the values within range 1 or range 2 for the 40% of the times to pass the test (i.e., 90% of the tests UE should report within the range mentioned in table 4).

- For the test configuration agreed, adapt range 1 and range 2 from the table 4.

- RAN4 to agree on the following test configuration

- Channel model: TDL-A

- TRS spacing: 1 slot

- Freq: 3.5 GHz

- Doppler and SNR combinations: (10Hz, 20 dB), (300Hz, 10 dB)

# Summary and Conclusion

In this contribution we analyzed the methods to define TDCP accuracy requirements and made following proposals.

- For the test metric, over the repeated tests, UE should at least report values within range 1 for the 50% of the times and the values within range 1 or range 2 for the 40% of the times to pass the test (i.e., 90% of the tests UE should report within the range mentioned in table 4).

- For the test configuration agreed, adapt range 1 and range 2 from the table 4.

- RAN4 to agree on the following test configuration

- Channel model: TDL-A

- TRS spacing: 1 slot

- Freq: 3.5 GHz

- Doppler and SNR combinations: (10Hz, 20 dB), (300Hz, 10 dB)

# References

[1] 	R4-2403372

# Annex

| Doppler [Hz] | SNR [dB] | Companies results |  |  |  |  |  | Ideal value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 5 | 0,7028 | 0,7243 | 0,902291 | 0,995415 | 0,983412 | 0,7069 | 0.99975 |
|  | 10 | 0,8715 | 0,8757 | 0,970982 | 0,996408 | 0,992663 | 0,8881 |  |
|  | 15 | 0,9516 | 0,9587 | 0,991936 | 0,994709 | 0,996049 | 0,9611 |  |
|  | 20 | 0,9809 | 0,9862 | 0,997 | 0,995189 | 0,998341 | 0,9873 |  |
| 30 | 5 |  | 0,7166 | 0,903111 | 0,987968 | 0,981535 | 0,7106 |  |
|  | 10 |  | 0,8811 | 0,968989 | 0,990522 | 0,991117 | 0,8862 |  |
|  | 15 |  | 0,9552 | 0,989825 | 0,984471 | 0,994646 | 0,9609 |  |
|  | 20 |  | 0,9842 | 0,995 | 0,983903 | 0,997191 | 0,987 |  |
| 200 | 5 | 0,6182 | 0,5908 |  | 0,853547 | 0,937058 | 0,6747 | 0.90371 |
|  | 10 | 0,8002 | 0,7646 |  | 0,85233 | 0,949576 | 0,8557 |  |
|  | 15 | 0,895 | 0,8532 |  | 0,856702 | 0,952684 | 0,9339 |  |
|  | 20 | 0,9322 | 0,8935 | 0,902 | 0,855079 | 0,954907 | 0,9592 |  |
| 300 | 5 | 0,5976 | 0,551 |  | 0,768931 | 0,893131 | 0,644 | 0.78996 |
|  | 10 | 0,7648 | 0,7138 |  | 0,772939 | 0,90208 | 0,8031 |  |
|  | 15 | 0,8555 | 0,7892 |  | 0,778584 | 0,905922 | 0,9019 |  |
|  | 20 | 0,8913 | 0,8216 | 0,807 | 0,782067 | 0,90851 | 0,9264 |  |